TIPS & TRICKS

MODELING GIGABIT PRE-EMPHASIS WITH IBIS

By

Michael Mirmak and Arpad Muranyi, Intel Corporation, Santa Clara, Calif.

Donald Telian, Cadence Design Systems, Inc., San Jose, Calif.

Today’s high-speed system designs are making increasing use of serial differential buffers.  To combat interconnect losses and ISI effects, serial differential interface technologies such as, Serial ATA* and PCI Express* typically use pre-emphasis to selectively ‘boost’ their buffer output and increase margins.  System designers therefore need fast, accurate means of modeling pre-emphasis to analyze performance and prevent signal integrity problems before production.

While behavioral models using IBIS – the I/O Buffer Information Specification – are commonly used in PCB system EDA, many view IBIS models as either too difficult to use or “not up to the task” of accurate pre-emphasis modeling.  This article outlines how to use the IBIS [Driver Schedule] keyword to create simple, modular and accurate models of serial-differential buffer designs featuring the most common form of pre-emphasis. Read on to see how your normal IBIS modeling process now can be effectively applied to the challenges of  GHz I/O modeling.

DO:

- Use the IBIS [Driver Schedule] keyword to tie together the elements of your pre-emphasis driver.  [Driver Schedule] can activate or de-activate IBIS models using fixed values of delays from rising and/or falling edges in the buffer input stimulus pattern. Without [Driver Schedule], model users could only connect buffer models in a cumbersome wired-or fashion.

- Model your buffer as multiple [Model] sections.  In most of today’s two-tap buffer designs, a ‘boost’ driver adds current to the output of a ‘main’ driver after each transition of the input stimulus (the boost output is often some fraction of the main output).  Collect separate I-V and V-T table data for the main and boost sections of your design and generate an IBIS model for each one.  Create a third ‘top-level’ model and add a [Driver Schedule] keyword section to it, referencing the main and boost buffers.

- Use [Driver Schedule] to provide logical control over your driver output.  The [Driver Schedule] delay parameters control how buffer pullups and pulldowns turn on and off.  You can therefore use [Driver Schedule] to invert your output relative to the input stimulus pattern by filling in only the ‘turn off’ delays after input edges.  For example, this syntax describes a simple inverter with no actual delay added to the input stimulus.

[Driver Schedule]

| Model_name       Rise_on_dly    Rise_off_dly   Fall_on_dly    Fall_off_dly

inverter                  NA                  0.0ns              NA                  0.0ns

- Fill in the [Driver Schedule] delays based upon the bit duration of your interface.  To implement two-tap pre-emphasis, we assume the boost buffer turns on in response to the input stimulus pattern inverted and delayed by one bit’s duration.  By combining the logical control capability of [Driver Schedule] with bit duration information, you can achieve complex pre-emphasis switching effects.  For example, here is the syntax for a two-tap buffer with an eye width of 400 ps.

[Driver Schedule]

| Model_name       Rise_on_dly    Rise_off_dly   Fall_on_dly    Fall_off_dly

main                       0.0ns               NA                  0.0ns              NA

boost                      NA                  0.400ns           NA                 0.400ns

This buffer’s main section switches immediately after an input rising edge.  EDA tools will interpret the boost section delay format as inversion, while the 400 ps value provides the one bit delay component.  The same technique could be used to model additional taps, as needed.

- Follow the specification rules in adding [Driver Schedule] to your IBIS files.  [Driver Schedule] is used in a ‘top-level’ buffer model containing C_comp (buffer capacitance) and clamp information for use by the EDA tool.  Other parts of the top-level model are ignored in favor of the tables in the scheduled buffer.

- Use the [Diff Pin] keyword to link the inverting and non-inverting parts of your buffer.  Many EDA tools that support IBIS will permit you to use a single input pattern to stimulate serial-differential buffer designs if [Diff Pin] is present.

DON’T:

- Use the same delay values for all buffer speeds or designs.  For example, a Serial ATA Generation I buffer has a nominal eye width of 666.66 ps.  The delay to use for the ‘boost’ in a two-tap design is therefore 666.66 ps.  Serial ATA Generation II has a nominal eye width of 333.33 ps.  To simulate a buffer running at Generation II data rates requires editing the IBIS model [Driver Schedule] delay values.  Also, make sure your main and boost IBIS [Model] V-T tables are shorter than the minimum target eye width, to ensure that your buffer settles completely after each transition in simulation.

- Neglect buffer capacitance effects.  Models using [Driver Schedule], per the IBIS specification, are only required to have valid C_comp (buffer capacitance) data in the top-level model.  How your buffer design behaves at high frequencies may be dependent on the method your EDA software uses to compensate V-T table data for C_comp effects.  If miscorrelation problems crop up or you have doubts, check with your EDA tool vendor to find out how C_comp is treated.

- Fail to properly separate your main and boost stages when making their IBIS equivalents.  It may be necessary to manually disable portions of the silicon buffer’s netlist if control bits do not allow this to be done externally.

- Forget to correlate your IBIS model output to your original simulation model or lab data.  The IBIS 4.0 keyword [Test Data] enables you to give out correlation waveforms for your complete buffer design as V-T tables, with appropriate loading conditions, for the convenience of your customers and partners.

- Think that IBIS cannot model complex structures such as pre-emphasis until you’ve given these steps a try.

*  other brands and names may be claimed as the property of others.

Further information on [Driver Schedule] and pre-emphasis can be found in the presentation “Modeling Pre/De-Emphasis Buffers with [Driver Schedule]” by Arpad Muranyi (http://www.eda.org/ibis/summits/jan05/muranyi.pdf).  This and many other IBIS modeling techniques are described in detail in the IBIS 4.0 Cookbook, scheduled for release by the end of Q2 2005.  To obtain other free presentations, syntax parsing tools or working copies of the IBIS specification, visit the IBIS Open Forum website (http://www.eigroup.org/ibis/).

Authors:

Michael Mirmak (michael.mirmak@intel.com) and Arpad Muranyi (arpad.muranyi@intel.com) are senior signal integrity engineers at Intel Corporation (Santa Clara, Calif.).   Donald Telian (donaldt@cadence.com) is a technologist at Cadence Design Systems (San Jose, Calif.).  Mr. Muranyi and Mr. Telian are founders of the IBIS Open Forum, the standards organization which manages IBIS.  Mr. Mirmak is the IBIS Open Forum’s current chair.

Picture: show block diagram of IBIS [Driver Schedule] usage, waveforms, and buffer structure

Figure:  Typical main/boost pre-emphasis structure and waveforms.