******************************************************************************

******************************************************************************

BIRD ID#:       xx.x

ISSUE TITLE:    Algorithmic Modeling API (AMI) support in IBIS

REQUESTER:     Cadence, SiSoft, Mentor

DATE SUBMITTED: TBD

DATE REVISED:   08/21/2007 - Internal

DATE ACCEPTED BY IBIS OPEN FORUM: N/A

******************************************************************************

******************************************************************************

STATEMENT OF THE ISSUE:

- The Interoperability Problem

Advanced SERDES devices operating above 5Gbps often contain architectures that are difficult, if not impossible, to simulate using current simulation techniques. Such devices often include Feed Forward Equalization, Decision Feedback Equalization, Clock and Data Recovery modules as well as Intellectual Property (IP) for noise cancellation using new and proprietary techniques.  These complex modules are very difficult to simulate using traditional device/circuit level simulation techniques. In addition, the serial links that are driven by these devices often need to be simulated with extremely long bit streams (10s of millions of bits to accurately predict Bit Error Rates) making high simulation performance an important consideration.

Since current device/circuit level modeling techniques are inadequate, SERDES IP vendors have been forced to develop, distribute, and maintain proprietary software for the modeling and simulation of such devices. While this can address the initial device modeling issues, proprietary approaches from the SERDES suppliers result in a lack of interoperability between models, leaving the systems designers unable to simulate the serial links. This approach also adds a support burden for SERDES IP vendors.  A new device modeling approach is needed to allow SERDES IP vendors to create algorithmic models that plug into commercially available PCB system interconnect simulators, while at the same time protecting the critical IP contained in the models.

- Key Requirements for Solution

The proposed modeling solution must:

- Allow SERDES models from a single semiconductor manufacturer to operate in multiple EDA tools

- Allow SERDES models from different semiconductor manufacturers to work together within a given EDA platform and simulation

- Expose key user-accessible device parameters [e.g. tap settings] to the EDA platform for control and observation by the user

- Support automatic optimization of device settings based on simulation

- Protect semiconductor vendor IP to the satisfaction of the IP vendors

******************************************************************************

STATEMENT OF THE RESOLVED SPECIFICATIONS:

This proposal breaks SERDES device modeling into two parts – electrical and algorithmic.  The combination of the transmitter’s analog back-end, the serial channel and the receiver’s analog front-end are assumed to be linear and time invariant.  There is no limitation that the equalization has to be linear. The “analog” portion of the channel is characterized by means of an impulse response leveraging the pre-existing IBIS standard for device models.

The transmitter equalization, receiver equalization and clock recovery circuits are assumed to be electrically isolated <DEFERRED> from the analog portion of the channel. The behavior of these circuits is modeled algorithmically through the use of executable code provided by the SERDES vendor.  This proposal outlines the functions of the executable models, the standards for passing data to and from these executable models and how the executable models are called from the EDA platform.

Note:

1. Throughout the document, contents inside the symbols “<” and “>” signifies data to be supplied or substituted by the model maker. Words/values inside “<” and “>” are not reserved and can be replaced.

2. Throughout the document, terms “long”, “double” etc are used to depict the data types in the C programming language.

Changes in Section 3a - K E Y W O R D   H I E R A R C H Y

*****   Original Text   *****

|  |-- [Model]                              Model_type, Polarity, Enable,

|  |   -------                              Vinl, Vinh, C_comp, C_comp_pullup,

|  |     |                                  C_comp_pulldown,

.

.

.

|  |     |

|  |     |-- [External Model]               Language, Corner, Parameters,

|  |         ----------------               Ports, D_to_A, A_to_D

|  |           |-- [End External Model]

*****    New Text (Changes  noted with |*) *****

|  |-- [Model]                              Model_type, Polarity, Enable,

|  |   -------                              Vinl, Vinh, C_comp, C_comp_pullup,

|  |     |                                  C_comp_pulldown,

.

.

.

|  |     |

|  |     |-- [External Model]               Language, Corner, Parameters,

|* |     |   ----------------               Ports, D_to_A, A_to_D

|* |     |     |-- [End External Model]

|* |     |

|* |     |-- [Algorithmic Model]		 Executable

|* |     |   -------------------

|* |     |     |-- Executable <type> <name> <Parameter_File>

|* |     |     |-- [End Algorithmic Model]

Additions to Section 6 – M O D E L   S T A T E M E N T

|=============================================================================

|

| KEYWORD DEFINITIONS:

|

|=============================================================================

|    Keywords:  [Algorithmic Model]  [End Algorithmic Model]

|    Required:  No

| Description:  Used to reference an external compiled model. This compiled model encapsulates signal processing functions. In the case of a receiver it may additionaly include clock and data recovery functions. The compiled model can receive and modify waveforms with  the analog front end channel. This data exchange is implemented through a set of software functions. The signature of these functions is elaborated in the function signature section. The function interface should comply with ANSI ‘C’ language

|  Sub-Params:  Executable

| Usage Rules:

[Algorithmic Model]

Begins Algorithmic Model Section

Executable: (required if [Algorithmic Model] is present)

Executable <Platform/OS/Signature keyword> <File Name> <Parameter_File>

<Platform/OS/Signature keyword>

We need a reserved list of keywords indicating the platform, OS, and how the DLL entry points are to be called.

This has to be one of the following reserved list of keywords

Windows_Intel_32

Windows_VisualStudio_32

Windows_gcc_32

Linux_gcc_32

Solaris_cc_32

Solaris_gcc_32

<File Name>

Name of the DLL file. The EDA platform will be responsible to

resolve the path of the Dll file.

<Parameter_File>

Name of the parameter file with an extension of .ami. This will be an external file with reserved and user defined parameters for use by the EDA tool and for passing to the model. The parameter file and the parameters are discussed in the API DESIGNER’S GUIDE section of the BIRD.

Example: Executable Linux_gcc_32 serdes_rx.dll serdes_rx_params.ami

[End Algorithmic Model]

Ends Algorithmic Model Section

| Other Notes:

|-------------------------------------------------------------------------------

|

|Example 1

|This is an example of an RX model with init to initialize AMI_GetWave and all the |work is done inside the AMI_GetWave.

[Algorithmic Model]

Executable Windows_VisualStudio_32 example_rx.dll example_rx_params.ami

[End Algorithmic Model]

|Example 2

|This is an example of an TX model

[Algorithmic Model]

Executable Windows_VisualStudio_32 tx_getwave.dll tx_getwave_params.ami

Executable Solaris_cc_32 tx_getwave.so tx_getwave_params.ami

[End Algorithmic Model

|

|===============================================================================

******************************************************************************

API Designer’s Guide

1. AMI Function Calls and Description

-------------------------------------

TABLE OF CONTENTS

-----------------

1.1 OVERVIEW

1.2 APPLICATION SCENARIOS

1.2.1 Linear, Time-invariant Model

1.2.2 Nonlinear, Time-variant Model

1.3 FUNCTION SIGNATURES

1.3.1 AMI_Init

1.3.1.1 Declaration

1.3.1.2 Arguments

1.3.1.1 impulse_matrix

1.3.1.2 row_size

1.3.1.3 aggressors

1.3.1.4 sample_interval

1.3.1.5 bit_time

1.3.1.6 AMI_dll_parameters (_in and _out)

1.3.1.7 AMI_dll_memory_handle

1.3.1.8 msg

1.3.1.3 Return Value

1.3.2 AMI_GetWave

1.3.2.1 Declaration

1.3.2.2 Arguments

1.3.2.10 wave

1.3.2.11 wave_size

1.3.2.12 clock_times

1.3.2.13 AMI_dll_memory

1.3.2.3 Return Value

1.3.3 AMI_Close

1.3.3.1 Declaration

1.3.3.2 Arguments

1.3.3.3 Return Value

1.3.3.13 AMI_dll_memory

1.4 MODEL PARAMETER FILE

1.4.1 Model Parameter File Structure

1.4.2 Definitions

1.4.3 Reserved Parameters

1.4.4 User Defined Parameters

1.4.5 Sample Parameter File

1.5 CODE SEGMENT EXAMPLES

--------------------------------------------------------------------------

1.1 OVERVIEW

The algorithmic model of a SERDES transmitter or receiver consists of three functions: ‘AMI_Init’, ‘AMI_GetWave’ and ‘AMI_Close’. The interfaces to these functions are designed to support three different phases of the simulation process: initialization, simulation during a segment of time, and termination of the simulation.

These functions (‘AMI_Init’, ‘AMI_GetWave’ and ‘AMI_Close’) should all be supplied in a single dll, and their names and signatures must be as described in this specification. If they are not supplied in the dll named by the Executable sub-parameter, then they will be ignored. This is acceptable so long as

- The entire functionality of the model is supplied in the dll.

- All termination actions required by the model are included in the dll.

Case 1: dll has AMI_Init, AMI_Getwave and AMI_Close.

Case 2: dll has only AMI_Init.

Please note that the function ‘AMI_Init’ is always required.

<AMBRISH/MIKE TO CLEAN UP>

The interfaces to these functions are defined from three different perspectives. In addition to specifying the signature of the functions to provide a software coding perspective, anticipated application scenarios provide a functional and dynamic execution perspective, and a specification of the software infrastructure provides a software architecture perspective. Each of these perspectives is required to obtain interoperable software models.

1.2 APPLICATION SCENARIOS

1.2.1 Linear, Time-invariant Model

1. From the system netlist, the EDA platform determines that a given [Model]

is described by an IBIS file.

2. From the IBIS file, the EDA platform determines that the [Model] is

described at least in part by an algorithmic model, and that the AMI_Init

function of that model returns an impulse response for that [Model].

3. The EDA platform loads the DLL or shared object file containing the

algorithmic model, and obtains the addresses of the AMI_Init, AMI_GetWave,

and AMI_Close functions.

4. The EDA platform assembles the arguments for AMI_Init. These arguments

include the impulse response of the channel driving the [Model], a handle

for the dynamic memory used by the [Model], the parameters for configuring

the [Model], and optionally the impulse responses of any crosstalk interferors.

5. The EDA platform calls AMI_Init with the arguments previously prepared.

6. AMI_Init parses the configuration parameters, allocates dynamic memory,

places the address of the start of the dynamic memory in the memory handle, computes the impulse response for the [Model] and passes it back to the EDA platform.

7. The EDA platform completes the rest of the simulation/analysis using the  impulse response from AMI_Init as a complete representation of the behavior of the given [Model].

8. Before exiting, the EDA platform calls AMI_Close, giving it the address in the memory handle for the [Model].

9. AMI_Close de-allocates the dynamic memory for the block and performs whatever other clean-up actions are required.

10. The EDA platform terminates execution.

1.2.2 Nonlinear, Time-variant Model

1. From the system netlist, the EDA platform determines that a given block is described by an IBIS file.

2. From the IBIS file, the EDA platform determines that the block is described at least in part by an algorithmic model.

3. The EDA platform loads the DLL or shared object file containing the algorithmic model, and obtains the addresses of the AMI_Init, AMI_GetWave, and AMI_Close functions.

4. The EDA platform assembles the arguments for AMI_Init. These arguments include the impulse response of the channel driving the block, a handle for the dynamic memory used by the block, the parameters for configuring the block, and optionally the impulse responses of any crosstalk interferers.

5. The EDA platform calls AMI_Init with the arguments previously prepared.

6. AMI_Init parses the configuration parameters, allocates dynamic memory and places the address of the start of the dynamic memory in the memory handle. AMI_Init may also compute the impulse response of the block and pass the modified impulse response to the EDA tool.

7. A long time simulation may be broken up into multiple time segments. For each time segment, the EDA platform computes the input waveform to the [Model] for that time segment. For example, if a million bits are to be run, there can be 1000 segments of 1000 bits each, i.e. one time segment comprises of 1000 bits.

8. For each time segment, the EDA platform calls the AMI_GetWave function, giving it the input waveform and the address in the dynamic memory handle for the block.

9. The AMI_GetWave function computes the output waveform for the block.

In the case of a transmitter, this is the Input voltage to the receiver. In the case of the receiver, this is the voltage waveform at the decision point of the receiver.

10. The EDA platform uses the output of the receiver AMI_GetWave function to complete the simulation/analysis. For transmitter, it simply passes the output to the receiver AMI_GetWave.

11. Before exiting, the EDA platform calls AMI_Close, giving it the address in the memory handle for the block.

12. AMI_Close de-allocates the dynamic memory for the block and performs whatever other clean-up actions are required.

13. The EDA platform terminates execution.

1.3 FUNCTION SIGNATURES

1.3.1 AMI_Init

1.3.1.1 Declaration

long AMI_Init (double *impulse_matrix,

long row_size,

long aggressors,

double sample_interval,

double bit_time,

char *AMI_dll_parameters_in,

char **AMI_dll_parameters_out,

void **AMI_dll_memory_handle,

char **msg)

1.3.1.2 Arguments

1.3.1.2.1 impulse_matrix: Impulse matrix is the channel impulse response     matrix. The impulse values are in volts and are uniformly distributed.  The sample spacing is given by the parameter ‘sample_interval’.

The ‘impulse_matrix’ is stored in single dimensional array of floating point numbers. The matrix elements can be retrieved /identified using

impulse_matrix[idx] = element (row, col)

Idx = col * row_size + row

Row – row index , ranges from 0 to row_size-1

Col – column index, ranges from 0 to aggressors

The first column of the impulse matrix is the impulse response for the primary channel. The rest are the impulse responses from aggressor drivers to the victim receiver.

The AMI_Init function may return a modified impulse response by modifying the first column of impulse_matrix. If the impulse response is modified, the new impulse response is expected to represent the concatenation of the model block with the blocks represented by the input impulse response.

The aggressor columns of the matrix should not be modified.

1.3.1.2.2 row_size: the number of rows in the impulse matrix

1.3.1.2.3 aggressors: the number of aggressors in the impulse matrix.

1.3.1.2.4 sample_interval: This is the sampling interval of the impulse matrix. Sample_interval will usually be a fraction of the highest data rate (lowest bit_time) of the device.

E.g.,

Sample_interval = (lowest_bit_time/64)

1.3.1.2.5 bit_time: the bit time of the current data.   E.g 100ps, 200ps etc. The dll may use this information along with the impulse matrix to initialize the filter coefficients.

1.3.1.2.6 AMI_dll_parameters (_in and _out)

Memory for AMI_dll_parameters_in is allocated and deleted by the EDA platform. The memory pointed to by AMI_dll_parameters_out is allocated and de-allocate by the model.

This is a pointer to a string. All the input parameters from the IBIS file Algorithmic model section are passed using a string that has been formatted as a parameter tree.

Examples of the tree parameter passing is:

(dll

(tx

(taps 4)

(spacing sync)

)

)

And

(root

(branch1

(leaf1 value1)

(leaf2 value2)

(branch2

(leaf3 value3)

(leaf4 value4)

)

(leaf5 value5 value6 value7)

)

)

The EDA tool will convert the parameter information from the IBIS file to tree format for dll consumption.

The syntax for this string is:

1. Neither names nor individual values can contain white space characters.

2. Parameter name/value pairs are always enclosed in parentheses, with the value separated from the name by white space.

2. A parameter value in a name/value pair can be either a single value or a list of values separated by whitespace.

3. Parameter name/value pairs can be grouped together into parameter groups by starting with an open parenthesis followed by the group name followed by the concatenation of one or more name/value pairs followed by a close parenthesis.

4. Parameter name/values pairs and parameter groups can be freely intermixed inside a parameter group.

5. The top level parameter string must be a parameter group.

6. White space is ignored, except as a delimiter between the parameter name and value.

7. 6. Parameter values can be expressed either as a decimal number or in the standard C notation for floating point numbers (e.g., 2.0e-9).

The pseudo code for the parser is:

<tree>:

<branch>

<branch>:

( <branch name> <leaf list> )

<leaf list>:

<branch>

<leaf>

<leaf list> <branch>

<leaf list> <leaf>

<leaf>:

( <parameter name> whitespace <value list> )

<value list>:

<value>

<value list> whitespace <value>

<value>:

<decimal number>

<decimal number>e<exponent>

<decimal number>E<exponent>

1.3.1.2.7 AMI_dll_memory_handle: Used to point to local storage for the dll and will be passed back during the AMI_GetWave calls. e.g. a code snippet may look like the following

my_space = allocate_space( sizeof_space );

status = store_all_kinds_of_things( my_space );

*sedes_dll_memory_handle = my_space;

The memory pointed to by AMI_dll_handle is allocated and de-allocated by the model.

1.3.1.2.8 msg: (optional) Provides descriptive, textual message from dll to EDA platform. It will provide a character string message that can be used by EDA platform to update log file or display in user interface.

1.3.1.3 Return Value

1 for success

0 for failure

1.3.2 AMI_GetWave

1.3.2.1 Declaration

long AMI_GetWave (double *wave,

long wave_size,

double *clock_times,

char **AMI_dll_parameters_out,

void *AMI_dll_memory);

1.3.2.2 Arguments

1.3.2.2.1 wave: A vector of time domain waveform, sampled uniformly at an interval specified by the ‘sample_interval’ specified during the init call. The wave is both input and output. The EDA platform provides the wave. The dll is expected to modify the waveform in place.

Depending on the EDA platform and the analysis/simulation method chosen, the input waveform could include many components. For example, the input waveform could include:
- The waveform for the primary channel only.
- The waveform for the primary channel plus crosstalk and amplitude noise.
- The output of a time domain circuit simulator such as SPICE.

The output waveform is expected to be the waveform at the decision point of the receiver (that is, the point in the receiver where the choice is made as to whether the data bit is a “1” or a “0”). It is understood that for some receiver architectures, there is no one circuit node which is the decision point for the receiver. In such a case, the output waveform is expected to be the equivalent waveform that would exist at such a node were it to exist.

1.3.2.2.2 wave_size: Number of samples in the wave vector. Range from 1 to maximum available memory size.

1.3.2.2.3 clock_times: Vector to return clock times. The clock times are referenced to the start of the simulation (the first AMI_GetWave call). The time is always greater or equal to zero. The last clock is indicated by putting a value of -1 at the end of clocks for the current wave sample. The clock_time vector is allocated by the EDA platform and is guaranteed to be greater than the number of clocks expected during this wave call.

The clock times are the times at which clock signal at the output of the clock recovery loop crosses the logic threshold. It is to be assumed that the input data signal will be sampled at exactly one half clock period after a clock time.

1.3.2.2.4 AMI_dll_parameters_out: (optional) A handle to a ‘tree string’ as described in 1.3.1.2.6. This is used by the model maker to return dynamic information and parameters. The memory for this string is to be allocated and deleted by the model maker.

1.3.2.2.5 AMI_dll_memory: this is the memory which was allocated during the init call

1.3.2.2.6 Return Value

1 for success

0 for failure

1.3.3 AMI_Close

1.3.3.1 Declaration

long AMI_Close(void * AMI_dll_memory);

1.3.3.2 Arguments

1.3.3.2.1 AMI_dll_memory

Same as for AMI_GetWave. See section 1.3.2.2.4

1.3.3.3 Return Value

1 for success

0 for failure

1.4 MODEL PARAMETER FILE

1.4.1 Model Parameter File Structure

The model parameter file will be organized in the parameter tree format as discussed in section 1.3.1.2.6. The file will have 2 distinct sections, ‘Reserved Parameters’ section and ‘Model Specific’ section.

(sampleAMI

(Reserved_Parameters |Reserved Keyword

…

…

) | End Reserved_Parameters

(Model_specific |Reserved Keyword

…

…

) | End Model_specific

)  | End SampleAMI

1.4.2 Definitions

The following <Usage>, <Data Type> and <Data Format> definitions are used throughout the following sections.

<usage> (required for user defined parameters)

In	Parameter is required Input to executable

Out	Parameter is Output only from executable

Info	Information for user or EDA platform

InOut	Required Input to executable. Executable may     return different value.

<data type> (default is float)

float

integer

string

Boolean (True/False)

Tap (For use by TX and RX equalizers. If data type of

parameter is ‘Tap’, it must be followed by <Tap

Numbers>)

UI (Unit Interval, 1 UI is the inverse of the data rate frequency, ex. 1 UI of a

channel operating at 10 Gbps will be 100ps)

<data format> (default is range)

<value> Single value data

Range	<typ value> <min value> <max value>

List    <typ value> <value> <value> <value> ... <value>

Corner  <typ value> <slow value> <fast value>

Increment 	<typ> <min> <max> <delta>

After expansion, the allowed values of the

parameter are typ+N*delta where N is any

positive or negative integer value such that:

min <= typ + N*delta <= max

Steps 	<typ> <min> <max> <# steps>

Treat exactly like Increment with

<delta> == (<max>-<min>)/<# steps>

Table <column names> ... < column names >

EndTable

Gaussian <mean> <sigma>

Dual-Dirac <mean> <mean> <sigma>

Composite of two Gaussian

DjRj <minDj> <maxDj> <sigma>

Convolve Gaussian (sigma) with uniform Modulation PDF

Note: In the context of Algorithmic Model for type ‘Corner’,

<slow value> and <fast value> align implicitly to slow and fast

corners, and <slow value> does not have to be less than <fast

value>. For type ‘Range’ and ‘Increment’, <min value>, <max

value> does not imply slow and fast corners.

1.4.3 Reserved Parameters

The model parameter file will have a sub tree with the heading ‘Reserved_Parameters’. This subtree will contain all the reserved parameters for the model.

The following reserved parameters are used by the EDA tool and are required if the [Algorithmic Model] keyword is present.

All reserved parameters, except Model_Specific and

Description will be in the following format:

(parameter_name (usage <usage>)(data type <data_type>)   (Default<values>))

Init_Returns_Impulse Info Boolean <True/False>

Tells the EDA platform whether the AMI_Init function returns a modified impulse response.  When this value is set to True, the model returns the convolution of the input impulse response with the impulse response of the equalization.  In this example, the model does not return the modified response.

GetWave_Exists Info Boolean <True/False>

Tells the EDA platform whether the “AMI_GetWave” function is implemented in this model.  Note that if Init_Returns_Impulse is set to “False”, then Getwave_Exists MUST be set to “True”.

The following reserved parameters are optional.  If these parameters are not present, the values are assumed as “0”.

Max_Init_Aggressors Info Integer <4>

Tells the EDA platform how many aggressor Impulse Responses the AMI_Init function is capable of processing.  In this example, the model can handle up to 4 aggressor inputs.

Ignore_Bits Info Integer <500000>

Tells the EDA platform how long the model takes to complete initialization.  This parameter is meant for AMI_GetWave functions that model how equalization adapts to the input stream.  The value in this field tells the EDA platform how many bits of the AMI_Getwave output should be ignored.

User_Defined

Any parameter defined after this sub parameter will a user defined parameter. If there is ‘User_Defined’ present, there must be ‘End_User_Defined’ present as well.

The following reserved parameter provides textual description to the user defined parameters.

Description <parameter> <string>

<parameter>

Name of a non-reserved parameter defined in this model

<string>

ASCII description for this non-reserved parameter, used by the EDA platform to convey information to the end-user allowing proper setting of parameter values. The entire line (including  ‘Description’ and the parameter name) has to be limited to IBIS line length specification. Each user defined sub-parameter must start with new line.

Tx-only reserved parameters

---------------------------

These reserved parameters only apply to Tx models.  These parameters are optional; if the parameters are not specified, the values default to “0 and will be in the following format:

(<parameter_name> (Usage <usage>)(Type <data_type>) (Format <data format> (Default<values>))

Tx_Jitter Info Float Gaussian <mean> <sigma>

Tx_Jitter Info Float Dual-Dirac <mean> <mean> <sigma>

Tx_Jitter Info Float DjRj <minDj> <maxDj> <sigma>

Tx_Jitter Info Float Table <time> <probability>

<data> <data>

EndTable

Tells the EDA platform how much jitter exists at the input to the transmitter’s analog output buffer.  Several different data formats are allowed as listed.

Tx_DCD Info Float Range <typ> <min> <max>

Transmit Duty Cycle Distortion tells the EDA platform the

maximum percentage deviation of the duration of a transmitted

pulse from the nominal pulse width.

Rx_Clock_Ticks_Generated Info Boolean <True/False>

If True, it tells the EDA platform if the RX generates clock ticks

Rx-only reserved parameters

---------------------------

These reserved parameters only apply to Rx models.  These parameters are optional; if the parameters are not specified, the values default to “0” and will be in the following format:

(<parameter_name> (usage <usage>)(data type <data_type>) (data format <data format> (Default<values>))

Rx_Clock_PDF Info	Float Gaussian <mean> <sigma>

Rx_Clock_PDF Info Float Dual-Dirac <mean> <mean> <sigma>

Rx_Clock_PDF Info Float DjRj <minDj> <maxDj> <sigma>

Rx_Clock_PDF Info Float Table <time> <probability> <data> <data>

EndTable

Tells the EDA platform the Probability Density Function of the recovered clock.  Several different data formats are allowed as listed.

Rx_Receiver_Sensitivity Info Float <value>

Rx_Receiver_Sensitivity Info Float Range <typ value> <min value> <max value>

Rx_Receiver_Sensitivity Info Float List <typ value> <value> <value> <value> ... <value>

Rx_Receiver_Sensitivity Info Float Corner <typ value> <slow value> <fast value>

Rx_Receiver_Sensitivity Info Float Increment <typ> <min> <max> <delta>

Rx_Receiver_Sensitivity Info Float Steps 	<typ> <min> <max> <# steps>

Rx_Receiver_Sensitivity Info Float <0.10>

Tells the EDA platform the voltage needed at the receiver data decision point to ensure proper sampling of the equalized signal.  In this example, 100mV (above +100mV or below -100mV) is needed to ensure the signal is sampled correctly.

The general rules, allowed usage and a brief summary of the data types and data formats allowed for each reserved parameter is presented in the following tables.

1.4.4 Model Specific Parameters

The Following section describes the user defined parameters. The algorithmic model expects these parameters and their values to function appropriately. The model maker can specify any number of user defined parameters for their model. The user defined parameter section subtree will begin with the reserved parameters ‘Model_Specific’.

The user defined parameters will be in the following format:

(<parameter_name> (usage <usage>) (Type <data type>) (Format <data format>) (Default <values>))

And

(<tap_parameter_name> (Usage <usage>) (tap <tap number>) (Format <data format>) (Default <values> ))

Where:

<parameter_name>

Arbitrary Names (IP Vendors choose what names their models accept; The IBIS Parser does not verify the names. It would, however, check the usage, data type, data format and values.) The names, types and description of the parameters used for algorithmic modeling will not be specified in the IBIS standard.

Published List of Standard Parameter Names

The IBIS Librarian will publish a list of parameter names. Each entry will include a unique data type and description. Model creators wishing to create new parameters may submit requests to the IBIS Librarian for approval

<tap_parameter_name>

Name for an N tap equalizer, where N is the number of taps in the equalizer. All the names for the N tap equalizer will be the same. For ex., a four tap equalizer will have four parameters with the same <tap parameter name>

<tap number>

The numbers for a particular tap for the N tap equalizer with the name <tap_parameter_name>. Tap number 0 is the primary (cursor) tap, negative tap numbers are pre-cursor taps, positive tap numbers are post_cursor taps. The sum of the absolute values of taps with the same parameter name must be <= 1.

1.4.5 Sample Parameter File

(sampleAMI

(Reserved_Parameters |Reserved Keyword

| all the parameters under this section are "standard". ie they have unique interpretation

(Ignore_Bits (Usage Info) (Type Integer) (Default 21))

(Max_Init_Aggressors (Usage Info) (Type Integer)(Default 25))

(Init_Returns_Impulse (Usage Info) (Type Boolean)(Default True))

(GetWave_Exists (Usage Info) (Type Boolean) (Default True))

) | End Reserved

(Model_specific |Reserved Keyword

| txtaps is a structure

(txtaps

(tapid (Range -1 3) (type int) )

(txtapcoeff

(-1 (Range -0.1 0.1 0.1) (Type float) (Default 0))

(0 (Range 1 1 0) (Type float) (Default 1))

(1 (List -0.1 0.1) (Type float)(Default  0))

(2 (Range -0.01 0.02 0.005) (Type float)
  (Default 0))

(3 (Range -0.01 0.02 0.005) (Type float)
  (Default 0))

)  | tapcoeff

)  | txtaps

(tx_freq_offset (Range 0 150 1) (Type ppm) (Default 0))

) | End User_Defined

| here is an example of txtaps instance

| (txtapcoeff (-1 -0.05) (0 1) (1 0) (2 0) (3 0))

)  | End SampleAMI

1.5 CODE SEGMENT EXAMPLES

extern long AMI_GetWave (wave, wave_size, clock_times, AMI_dll_memory);

my_space = AMI_dll_memory;

clk_idx=0;

time = my_space->prev_time + my_space->sample_interval;

for(i=0; i<wave_size; i++)

{

wave = filterandmodify(wave, my_space);

if (clock_times && found_clock (my_space, time))

clock_times[clk_idx++] = getclocktime (my_space, time);

time += my_space->sample_intevel;

}

clock_times[clk_idx] = -1;   //terminate the clock vector

Return 1;

******************************************************************************

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

This section of the IBIS specification has been driven primarily by the following factors:

1. The interaction between a SERDES and the system surrounding it is quite

complex, thus requiring sophisticated and detailed modeling.

2. There is considerable variation in the architectures and circuit techniques

used in SERDES devices.

3. There is not a commonly accepted set of parameters that can be measured to

fully and reliably characterize the performance of a given SERDES device

independently from the system that surrounds it.

Because of these factors, IP vendors' experience has been that customers use

the models delivered by the IP vendor as a form of performance specification.

If the model predicts a level of performance in a given application, then the

IP is held to that level of performance or better when the system is tested.

For this reason, IP vendors are reluctant to supply any but most detailed and

accurate models they can produce. This is a fundamental shift in that in the

past, the models that were presumed to be utterly complete and reliable were

SPICE models, and IBIS models were understood to be a useful approximation

that could be shared without divulging sensitive proprietary information.

By setting the algorithmic model as the primary deliverable, this specification

maximizes the flexibility available to the model developers and also maximizes

the degree of protection for proprietary information. By standardizing the

interface to these algorithmic models, this specification also enables the

required degree of interoperability.

******************************************************************************

Any Other Background Information

SUPPLEMENTARY MATERIAL (To be resolved before final submission.)

REVISION HISTORY CHANGES:

********************************************************************************