IBISCHK4 Parser Specification and Behaviors

Revision 0.2

By Michael Mirmak, Intel Corp.

Revised October 7, 2005

| Revision | Date | Changes |
| --- | --- | --- |
| 0.1 | Oct. 2, 2005 | Initial revision |
| 0.2 | Oct. 7, 2005 | Corrected R tolerance to 50 ohms; added several new sections plus a table of contents |

IBISCHK4 Parser Specification and Behaviors	1

1.0	Introduction	3

2.0	General Limits	3

3.0	Warning and Error Summary	3

3.1	[Alternate Package Model]	3

3.2	[Component] and Related Checks	3

3.3	[Diff Pin]	4

3.4	[Driver Schedule]	4

3.5	[External Circuit] and Related Checks	5

3.6	[Model Selector]	6

3.7	[Model Spec]	6

3.8	[Model] and Related Checks	6

3.9	[Package Model]	6

3.10	[Package]	6

3.11	[Pin Mapping]	6

3.12	[Pin]	7

3.13	[Ramp]	7

3.14	[Receiver Threshold]	7

3.15	[Row]	7

3.16	[Series MOSFET]	8

3.17	[Series Pin Mapping]	8

3.18	[Series Switch]	8

3.19	[Submodel Spec]	8

3.20	[Submodel]	8

3.21	[Test Data]	8

3.22	[Test Load]	8

3.23	[TTxxx]	8

3.24	Basic Checks	8

3.25	EBD Files	8

3.26	Header	10

3.27	I-V Tables	10

3.28	Package Matrix	10

3.29	RLC Data	10

3.30	Series Elements	11

3.31	Waveform Checking	11

# Introduction

The following document summarizes the warnings, errors and basic assumptions of the IBISCHK4 IBIS parser program, based on an analysis of the code.  This document should enable model makers, future specification contributors and model users to better understand how the IBIS specification is actually interpreted through the  parser.

Note that the code used for the analysis is 4.1.1.

Non-IBIS related bugs reported by the parser – for example, an inability to write files to directories or allocate sufficient memory – have been omitted from this summary.

Four general types of parser messages are issued: Warnings, Errors, Line-based Warnings and Line-based Errors.  The summaries below preserve this distinction wherever possible.

# General Limits

The following general assumptions are made by the parser in analyzing IBIS files:

- When analyzing V-t tables for matching to I-V data, no interpolation of the I-V data is assumed.  Mismatches greater than 2% but less than 10% will generate a warning message.  Mismatches greater than 10% will generate an error message.

- The maximum value of C_comp permitted without warning is 1 nF.

- The maximum value of package pin capacitance permitted without warning is 1 nF.

- The maximum value of package pin inductance permitted without warning is 1 uH.

- The maximum value of package pin resistance permitted without warning is 50 ohms.

- The maximum value of [Ramp] dt permitted without warning is 1 ms.

# Warning and Error Summary

## [Alternate Package Model]

- ERROR (LINE) Alternate Package Model has no package models defined

- ERROR (LINE) Invalid Package Model name 'xxx' is too long.

- ERROR (LINE) Missing Package Model name

- ERROR (LINE) Package Model Name Previously Defined("xxx")

- WARNING (LINE) Alternate Package Model has only one package model defined

## [Component] and Related Checks

- ERROR Component <xxx> pin <xxx> model <xxx> type <xxx> using [External Model] without corresponding [Diff_Pin]

- ERROR Required keyword 'Component' not found

- ERROR (LINE) column header not found or out of order

- ERROR (LINE) Data on this line appears to be for the Component keyword, which does not allow data on subsequent lines

- ERROR (LINE) Data on this line appears to be for the Manufacturer keyword, which does not allow data on subsequent lines

- ERROR (LINE) Data on this line appears to be for the Package Model keyword, which does not allow data on subsequent lines

- ERROR (LINE) Duplicate [Alternate Package Models] keyword

- ERROR (LINE) Duplicate [Series_Switch_Groups] keyword found

- ERROR (LINE) Duplicate column header 'xxx'

- ERROR (LINE) Duplicate Diff_Pin keyword found

- ERROR (LINE) Duplicate Node Declaration keyword found

- ERROR (LINE) Duplicate Package keyword

- ERROR (LINE) Duplicate Pin_Mapping keyword found

- ERROR (LINE) Duplicate Pin keyword found

- ERROR (LINE) Duplicate Series_Pin_Mapping keyword found

- ERROR (LINE) Expecting End Alternate Package Models keyword

- ERROR (LINE) Orphan Component keyword

- ERROR (LINE) Orphan data line keyword

- ERROR (LINE) Unable to find column headers after xxx keyword

- ERROR (LINE) Unexpected End Alternate Package Models keyword

- ERROR (LINE) Unknown column header 'xxx' or out of order

- ERROR (LINE) Unknown column header after function_table_group

- ERROR (LINE) Unknown data after [Alternate Package Models] keyword

- ERROR (LINE) Unknown data after [End Alternate Package Models] keyword

- ERROR (LINE) Unknown data after Package keyword

- ERROR (LINE) Unknown Line after Component Description

- WARNING (LINE) Component 'xxx' contains a blank character

## [Diff Pin]

- ERROR (LINE) Bad value for xxx column

- ERROR (LINE) No Diff_Pin column entry

- ERROR (LINE) No inv_pin column entry

- ERROR (LINE) No tdelay_typ column entry

- ERROR (LINE) No vdiff column entry

- ERROR (LINE) Too many Diff Pin data columns

- WARNING Component 'xxx' Diff_Pin 'xxx' is not unique

- WARNING Component 'xxx' inv_pin 'xxx' is not unique

- WARNING (LINE) Suspicious value for xxx column

- WARNING (LINE) Typ value is not in between Min and Max

## [Driver Schedule]

- ERROR Model 'xxx': The [Driver_Schedule] has no models defined

- ERROR (LINE) At least two delays must be specified

- ERROR (LINE) Driver Schedule Already Defined for this model

- ERROR (LINE) Driver Schedule Missing model Name

- ERROR (LINE) Invalid Model Name in Driver Schedule ("xxx"): Reserved Word

- ERROR (LINE) Model Name ("xxx") previously used in this Driver Schedule

- ERROR (LINE) Model Name in Driver Schedule is the same as enclosing Model ("xxx")

- ERROR (LINE) Model Name Over xxx Characters Long

- WARNING Model 'xxx': The [Driver_Schedule] has only one model defined

## [External Circuit] and Related Checks

- ERROR [CIRCUIT CALL] Line <xxx> Redundant use of port <xxx>

- ERROR [Circuit Call] at (line xxx), no node/pin <xxx> in Component xxx

- ERROR [Circuit Call] at (line xxx), no port <xxx> in [External Circuit] xxx

- ERROR [Circuit Call] at (line xxx) first reference pin <xxx> is not declared CIRCUIT CALL in [Pin] section

- ERROR [Circuit Call] at (line xxx) second reference pin <xxx> is not declared CIRCUIT CALL in [Pin] section

- ERROR <xxx> is connect to a reserved port

- ERROR CCALL Line <xxx> A2D Line <xxx> Illegal connection

- ERROR CCALL Line <xxx> A2D Line <xxx> Illegal connection, 2 analog reference/driver ports <xxx>=<xxx> <xxx>=<xxx>

- ERROR Circuit <xxx> Floating User Defined Digital Port <xxx>

- ERROR Circuit Call Line xxx instantiates External Circuit at xxx with Floating Internal Node <xxx> type xxx

- ERROR CMPT <xxx>, [CIRCUIT CALL] at (line xxx) no reference pin (need Signal_pin, Diff_signal_pins, Series_pins)

- ERROR CMPT <xxx>, [CIRCUIT CALL] at (line xxx) references same pin twice <xxx>

- ERROR CMPT <xxx>, [Circuit Call] at (line xxx) refererences illegal pin <xxx>

- ERROR CMPT <xxx>, [Circuit Call] at (line xxx) refererences illegal pin <xxx>

- ERROR Component xxx Node <xxx>  is analog and digital xxx

- ERROR Component <xxx> Floating Node <xxx> (1 connection) xxx

- ERROR Component <xxx> Floating Pin <xxx> (no connections but used in CIRCIUT CALL)

- ERROR Component <xxx> Node <xxx> has a digital receiver but no digital driver xxx

- ERROR Component <xxx> Node <xxx> has a digital driver but no digital receiver xxx

- ERROR Component <xxx> Node <xxx> has no defined drivers xxx

- ERROR Component <xxx> Node <xxx> has a analog receiver but no analog driver xxx

- ERROR Component <xxx> Node <xxx> has a analog driver but no analog receiver xxx

- ERROR Component <xxx> Node <xxx> Used as Analog and Digital in CIRCUIT CALL(s)

- ERROR Component <xxx> Pin <xxx> Used as Digital in CIRCUIT CALL(s)(pins are analog)

- ERROR Component <xxx> Pin <xxx> Used as Digital in CIRCUIT CALL(s)(pins are analog) xxx

- ERROR Internal Node <xxx>, External Circuit at xxx is analog and digital

- ERROR (LINE) No [External Circuit] xxx for [Circuit Call]

- ERROR (LINE) No [External Circuit] for [Circuit Call] xxx in Component xxx

- ERROR (LINE) Extra Charaters on PIN line <xxx>

- ERROR (LINE) Extra Charaters on PIN line <xxx>

- ERROR (LINE) Illegal parameter <xxx>

- ERROR (LINE) Missing port or pad name in Port_map

- ERROR (LINE) Redundant use of port <xxx>

- ERROR (LINE) Too Many Signal Pins (only two allowed) <xxx>

- ERROR (LINE) Use of reserved port <xxx> in Port Map

- WARNING Component <xxx> Node <xxx> has a digital receiver but no obvious digital driver

- WARNING Component <xxx> Node <xxx> has a digital driver but no obvious digital receiver

- WARNING Component <xxx> Node <xxx> has a analog receiver but no obvious analog driver

- WARNING Component <xxx> Node <xxx> has a analog driver but no obvious analog receiver

- WARNING Component <xxx> Unused Node <xxx> (0 connection) xxx

## [Model Selector]

- ERROR Model Selector 'xxx' has no models defined

- ERROR (LINE) Invalid Model Name ("NA")

- ERROR (LINE) Invalid Model Selector Name ("xxx")

- ERROR (LINE) Model 'xxx' already used in this Model Selector

- ERROR (LINE) Model Name Over xxx Characters Long

- ERROR (LINE) Model Selector Keyword Missing model name

- ERROR (LINE) Model Selector Keyword Missing name

- ERROR (LINE) Model Selector Name Over xxx Characters Long

- ERROR (LINE) Model Selector Name Previously Defined ("xxx")

- ERROR (LINE) Orphan Data line

- ERROR (LINE) Orphan Model Selector keyword

- WARNING  Model Selector 'xxx' has only one model defined

## [Model Spec]

- ERROR (LINE) Model Specification Already Defined for this model

- ERROR (LINE) Unknown Line Among Model Spec Data

- WARNING (LINE) xxx should not be specified for model type xxx

## [Model] and Related Checks

## [Package Model]

## [Package]

## [Pin Mapping]

- ERROR (LINE) xxx is an invalid Pin_Mapping column entry

- ERROR (LINE) Duplicate Pin Mapping Pin

- ERROR (LINE) Extra data columns on this line

- ERROR (LINE) No Pin number

- ERROR (LINE) No pulldown_ref data

- ERROR (LINE) No pullup_ref data

- WARNING Component xxx  Pulldown Reference xxx is also used as a Pullup Reference

- WARNING Pulldown Reference xxx is also used as a Pullup Reference

## [Pin]

- ERROR Component 'xxx' Pin 'xxx' not referenced in Pin Mappings

- ERROR Pin 'xxx' found in Package_Model 'xxx' Pin_Numbers list not found in Component 'xxx' Pinlist

- ERROR (LINE) Duplicate Pin 'xxx'

- ERROR (LINE) Model name not found

- ERROR (LINE) Pin not found

- ERROR (LINE) Signal name not found

## [Ramp]

- ERROR (LINE) Ramp Already Defined

- ERROR (LINE) Unknown Line Among Ramp Data

## [Receiver Threshold]

## [Row]

- ERROR (LINE) [Row] xxx must contain a self-coupled index and entry

- ERROR (LINE) Duplicate Row column index xxx

- ERROR (LINE) Excessive data <xxx>, only Index and Value should be specified for a Sparse_matrix

- ERROR (LINE) Expected xxx columns of Row data for [Row] xxx in Matrix ending previous to this line, found xxx column xxx

- ERROR (LINE) Expected at least one column of Row data for [Row] xxx in Matrix ending previous to this line

- ERROR (LINE) Expected Pin <xxx> at matrix row index xxx, found at matrix row index xxx

- ERROR (LINE) More row data than allowed for this row

- ERROR (LINE) No Column Index

- ERROR (LINE) No row data

- ERROR (LINE) Only xxx columns of data allowed for this Row

- ERROR (LINE) PIN NUMBER string too long

- ERROR (LINE) Repeated ROW <xxx>

- ERROR (LINE) This Row Column Index defines an entry in the lower matrix triangle

- ERROR (LINE) Unable to find Pin <xxx>in Package Model Pin_Number list

- ERROR (LINE) Unknown PIN NUMBER

- WARNING (LINE) Suspicious row data

## [Series MOSFET]

## [Series Pin Mapping]

## [Series Switch]

## [Submodel Spec]

- ERROR Submodel xxx  Empty Submodel Spec

- ERROR (LINE) Sub Model Specification Already Defined for this sub model

- ERROR (LINE) Unknown Line Among Sub Model Spec Data

## [Submodel]

- ERROR SubModel xxx  Both Pullup and Pulldown cannot be specified when Off_delay is specified

- ERROR (LINE) [Submodel Spec] should be specified before other keywords for a model

- ERROR (LINE) [GND Pulse Table] Already Defined

- ERROR (LINE) [POWER Pulse Table] Already Defined

- ERROR (LINE) Invalid Sub Model Name ("xxx")

- ERROR (LINE) Orphan Sub Model keyword

- ERROR (LINE) Sub Model Keyword Missing name

- ERROR (LINE) Sub Model Name Over xxx Characters Long

- ERROR (LINE) Sub Model Name Previously Defined ("xxx")

- ERROR (LINE) Unknown Line Among Sub Model Data

## [Test Data]

## [Test Load]

## [TTxxx]

- ERROR (LINE) TTgnd Max value should be > 0

- ERROR (LINE) TTgnd Min value should be > 0

- ERROR (LINE) TTgnd Typical value should be > 0

- ERROR (LINE) TTpower Max value should be > 0

- ERROR (LINE) TTpower Min value should be > 0

- ERROR (LINE) TTpower Typical value should be > 0

- WARNING TTgnd typ value is not in between Min and Max

- WARNING TTpower typ value is not in between Min and Max

## Basic Checks

## EBD Files

- ERROR Component 'xxx' not found in file 'xxx'

- ERROR Ebdfile 'xxx' recursively references the component xxx in the calling Ebd file 'xxx'

- ERROR EBD Model 'xxx' has no paths defined

- ERROR Length of Pin 'xxx' referenced for Component 'xxx' is too long

- ERROR No Pin Numbers names found

- ERROR Number of Pins <xxx> not the same as count of Pin Numbers names found <xxx>

- ERROR Path xxx or a branch does not terminate after NC

- ERROR Path Descriptor 'xxx' has no segments

- ERROR Pin 'xxx' not found on Component 'xxx' in file 'xxx'

- ERROR Pin 'xxx' used in Path 'xxx' not defined

- ERROR Pin 'xxx' was not referenced on any Electrical Path

- ERROR Pin 'xxx' was referenced xxx times on Electrical Paths

- ERROR Pin 'xxx' was referenced on Electrical Paths but is implicitly connected to a power or a gnd plane

- ERROR Reference Designator 'xxx'refers to the Component 'xxx' itself being defined

- ERROR Reference Designator 'xxx'used in Path'xxx'not defined

- ERROR Required End_Board_Description keyword not found

- ERROR Required keyword Pin_List not found

- ERROR (LINE) Bad Number_of_Pins value

- ERROR (LINE) Blank Number_of_Pins value

- ERROR (LINE) Component name 'xxx' too long. Will be truncated

- ERROR (LINE) Detected previous unterminated Forks

- ERROR (LINE) Detected an orphan 'Node' subparam

- ERROR (LINE) Detected previous unterminated Forks

- ERROR (LINE) Detected the start of an orphan 'Endfork'

- ERROR (LINE) Detected the start of an orphan 'Fork'

- ERROR (LINE) Detected the start of an orphan 'NC'

- ERROR (LINE) Detected the start of an orphan R/L/C section

- ERROR (LINE) Duplicate Path_Description 'xxx'

- ERROR (LINE) Duplicate Reference Designator 'xxx'

- ERROR (LINE) Duplicate 'xxx' keyword found

- ERROR (LINE) Duplicate [Number of Pins] keyword

- ERROR (LINE) Duplicate [Pin List] keyword found

- ERROR (LINE) Duplicate Number_of_Pins keyword

- ERROR (LINE) Duplicate Pin <xxx>

- ERROR (LINE) Empty Fork/Endfork found

- ERROR (LINE) Endfork found without enclosing Fork

- ERROR (LINE) Extraneous data on 'xxx' line

- ERROR (LINE) Filename 'xxx' too long. Will be truncated

- ERROR (LINE) Invalid Node specified. Missing '.'

- ERROR (LINE) Invalid filename 'xxx'. It has no extension

- ERROR (LINE) More Pin Numbers names than Number of pins

- ERROR (LINE) No Component name found

- ERROR (LINE) No filename found

- ERROR (LINE) No Pin_name found

- ERROR (LINE) No Pin Name found after Pin subparam

- ERROR (LINE) No ref_designatior. pin found after Node subparam

- ERROR (LINE) No Reference Designator name found

- ERROR (LINE) No signal_name found

- ERROR (LINE) Number_of_Pins = 0

- ERROR (LINE) Orphan Electrical Board Description keyword

- ERROR (LINE) Reference Designator 'xxx' refers to the Component 'xxx' itself being defined

- ERROR (LINE) Reference Designator name 'xxx' too long. Will be truncated

- ERROR (LINE) signal_name column header not found

- ERROR (LINE) signal_name too long. will be truncated

- ERROR (LINE) Unable to find 'signal_name' after [Pin List] keyword

- ERROR (LINE) Unexpected '/' found

- ERROR (LINE) Unexpected '=' found

- ERROR (LINE) Unexpected data after signal_name

- ERROR (LINE) Unknown data after component name

- ERROR (LINE) Unknown file extension 'xxx'.  Should be '.ibs' or '.ebd'

- ERROR (LINE) Unknown token found 'xxx'

- ERROR (LINE) Unrecognized data after 'signal_name'

- WARNING Pin 'xxx' was referenced on Electrical Paths but is implicitly connected to a power or a gnd plane

- WARNING (LINE) Found a Pin Name 'xxx' containing '/'. Possible syntax ERRor

- WARNING (LINE) Found a Pin Name 'xxx' containing '='. Possible syntax ERRor

- WARNING (LINE) Found a Pin Name 'xxx'. Possible syntax ERRor

- WARNING (LINE) Found a Node 'xxx'. Possible syntax error

- WARNING (LINE) Found a Node 'xxx' containing '/'. Possible syntax error

- WARNING (LINE) Found a Node 'xxx' containing '='. Possible syntax error

- WARNING (LINE) Suspicious Number_of_Pins value

## Header

- ERROR (LINE) Duplicate Copyright keyword

- ERROR (LINE) Duplicate Date keyword

- ERROR (LINE) Duplicate File_name keyword

- ERROR (LINE) Duplicate File_Rev keyword

- ERROR (LINE) Duplicate IBIS_Ver keyword

- ERROR (LINE) Duplicate Notes keyword

- ERROR (LINE) Duplicate Source keyword

- ERROR (LINE) File_name 'xxx' must contain one period

- ERROR (LINE) IBIS_Version must be first keyword

- ERROR (LINE) Illegal IBIS_Version 'xxx'

- ERROR (LINE) Orphan File Header keyword

## I-V Tables

## Package Matrix

## RLC Data

- ERROR (LINE) L and C must be specified when Len != 0

- ERROR (LINE) C should not be less than zero

- ERROR (LINE) Duplicate C sub param in R/L/C section

- ERROR (LINE) Duplicate L sub param in R/L/C section

- ERROR (LINE) Duplicate Len sub param in R/L/C section

- ERROR (LINE) Duplicate R sub param in R/L/C section

- ERROR (LINE) Empty R/L/C section found

- ERROR (LINE) L should not be less than zero

- ERROR (LINE) Len should be specified before C

- ERROR (LINE) Len should be specified before L

- ERROR (LINE) Len should be specified before R

- ERROR (LINE) Len should not be less than zero

- ERROR (LINE) Premature end of R/L/C section. Trailing '/' missing

- ERROR (LINE) R should not be less than zero

## Series Elements

- ERROR Model xxx [Series_MOSFET] is not allowed in conjunction with the [Off] keyword

- ERROR Model xxx xxx No series data found

- ERROR Model xxx xxx Rl_series cannot be specified if L_series is not specified

- ERROR Model xxx xxx Rc_series cannot be specified if C_series is not specified

- ERROR Model xxx xxx Lc_series cannot be specified if C_series is not specified

- ERROR (LINE) xxx Series Info Already Defined for this model

## Waveform Checking

- WARNING  Model 'xxx' : xxx VI curves cannot drive through Vmeas= xxx V given load Rref= xxx Ohms to Vref= xxx V

- WARNING  Model 'xxx' : xxx AC xxx Endpoints (xxx V, xxx V) not within xxx V (xxx %) of (xxx V, xxx V) on VI curves for xxx Ohms to xxx V

- WARNING  Model 'xxx' : xxx AC xxx Endpoints (xxx V, xxx V) *ARE* WITHIN   xxx V (xxx %) of (xxx V, xxx V) on VI curves for xxx Ohms to xxx V

- WARNING  Model 'xxx' : Currents <> 0.0mA in 'Pullup' for Open_drain/Open_sink device

- WARNING  Model 'xxx' : Currents <> 0.0mA in 'Pulldown' for Open_source device

- WARNING  Model xxx data is non-monotonic

- ERROR Require at least 2 Data Points for xxx Waveform Beginning at Line xxx

- ERROR (LINE) xxx specified after beginning of timing data

- ERROR (LINE) Duplicate definition for xxx

- ERROR (LINE) Invalid xxx line

- ERROR (LINE) More than xxx data points provided for xxx table

- ERROR (LINE) More than 100 xxxs are defined

- ERROR (LINE) Sub Parameters are not allowed in Golden Waveforms

- ERROR (LINE) Time value did not increase

- ERROR (LINE) Unknown Line Among xxx Data