******************************************************************************

******************************************************************************

BIRD ID#:       xx.x

ISSUE TITLE:    Algorithmic Modeling API (AMI) support in IBIS

REQUESTERS (in alphabetical order): Ian Dodd, Agilent, C. Kumar, Hemant Shah and Ambrish Varma, Cadence, John Angulo and Arpad Muranyi, Mentor Graphics, Walter Katz, Mike Steinberger and Todd Westerhoff, SiSoft

REVIEWERS: 	Bob Ross, Teraspeed, Michael Mirmak, Intel

DATE SUBMITTED: TBD

DATE REVISED:   10/02/2007 - Internal

DATE ACCEPTED BY IBIS OPEN FORUM: N/A

******************************************************************************

******************************************************************************

STATEMENT OF THE ISSUE:

- The Interoperability Problem

Advanced Serializer-Deserializer(SERDES) devices operating above 5 Gb/s often contain architectures that are difficult to simulate in a time efficient and accurate manner using current techniques. Such devices often include Feed Forward Equalization, Decision Feedback Equalization, Clock and Data Recovery modules as well as Intellectual Property (IP) for noise cancellation using new and proprietary techniques.  These complex modules are very difficult to simulate using traditional device/circuit level simulation techniques. In addition, the serial links that are driven by these devices often need to be simulated with extremely long bit streams (tens of millions of bits to accurately predict Bit Error Rates) making high simulation performance an important consideration.

Since current device/circuit level modeling techniques are inadequate, SERDES IP vendors have been forced to develop, distribute, and maintain proprietary software for the modeling and simulation of such devices. While this can address the initial device modeling issues, proprietary approaches from the SERDES suppliers result in a lack of interoperability between models, leaving the systems designers unable to simulate the serial links. This approach also adds a support burden for SERDES IP vendors.  A new device modeling approach is needed to allow SERDES IP vendors to create algorithmic models that plug into commercially available PCB system interconnect simulators, while at the same time protecting the critical IP contained in the models.

- Key Requirements for Solution

The proposed modeling solution detailed below:

- allows SERDES models from a single semiconductor manufacturer to operate in multiple EDA tools

- allow SERDES models from different semiconductor manufacturers to work together within a given EDA platform and simulation

- exposes key user-accessible device parameters [e.g. tap settings] to the EDA platform for control and observation by the user

- supports automatic optimization of device settings based on simulation

- protects semiconductor vendor IP to the satisfaction of the IP vendors

******************************************************************************

STATEMENT OF THE RESOLVED SPECIFICATIONS:

This proposal uses much of IBIS directly for pinout, package, specification, and other content and proposes linking to external shared library for an additional method to model and report advanced SERDES devices.

A new Section 6c describes this extension.

A new Section 10 ‘NOTES ON ALGORITHMIC MODELING INTERFACE AND PROGRAMMING GUIDE’ is also introduced to serve as a guide to programming the shared library model.

The Section 6c addition is below:

|=============================================================================

|=============================================================================

|

|                                 Section 6c

|

|  	     A L G O R I T H M I C  M O D E L  A P I  S U P P O R T

|

|=============================================================================

|=============================================================================

| INTRODUCTION:

|

| Executable shared library files to model advanced Serializer-Deserializer(SERDES) | devices are supported by IBIS.  This chapter describes how executable models       | written for these devices can be referenced and used by IBIS files.

|

| The shared obkjects use the following keywords within the IBIS framework:

| [Algorithmic Model]

| [End Algorithmic Model]

|

|

| The placement of these keywords within the hierarchy of IBIS is shown in the

| following diagram:

|

|

|   |-- [Component]

|   |   | ...

|   |   | ...

|   | ...

|   |-- [Model]

|   |   | ...

|   |   |-- [Algorithmic Model]

|   |   |-- [End Algorithmic Model]

|   |   | ...

|   | ...

|   | ...

|

| Figure 1: Partial keyword hierarchy

|

| GENERAL ASSUMPTIONS:

<<Ignoring the comment character hence forth>>

This proposal breaks SERDES device modeling into two parts – electrical and algorithmic.  The combination of the transmitter’s analog back-end, the serial channel and the receiver’s analog front-end are assumed to be linear and time invariant.  There is no limitation that the equalization has to be linear and time invariant. The “analog” portion of the channel is characterized by means of an impulse response leveraging the pre-existing IBIS standard for device models.

The transmitter equalization, receiver equalization and clock recovery circuits are assumed to have a high-impedance (electrically isolated) connection to the analog portion of the channel. This makes it possible to model these circuits based on a characterization of the analog channel. The behavior of these circuits is modeled algorithmically through the use of executable code provided by the SERDES vendor.  This proposal defines the functions of the executable models, the methods for passing data to and from these executable models and how the executable models are called from the EDA platform.

| DEFINITIONS:

The following 'Usage, Type Format and Default definitions are used throughout the following sections.

Note: Usage, Type Format and Default and their allowed values are reserved names in the parameter definition file (.ami) discussed in the ‘KEYWORD DEFINITION’ section.

Usage: (required for model specific parameters)

In	Parameter is required Input to executable

Out	Parameter is Output only from executable

Info	Information for user or EDA platform

InOut	Required Input to executable. Executable may     return different value.

Type: (default is Float)

Float

Integer

String

Boolean (True/False)

Tap (For use by TX and RX equalizers. If data type of

parameter is ‘Tap’, it must be followed by <Tap

Numbers>)

UI (Unit Interval, 1 UI is the inverse of the data rate

frequency, ex. 1 UI of a channel operating at 10 Gb/s is

100 ps)

Format: (default is range)

Value	<value> Single value data

Range	<typ value> <min value> <max value>

List    <typ value> <value> <value> <value> ... <value>

Corner  <typ value> <slow value> <fast value>

Increment 	<typ> <min> <max> <delta>

After expansion, the allowed values of the

parameter are typ+N*delta where N is any

positive or negative integer value such that:

min <= typ + N*delta <= max

Steps 	<typ> <min> <max> <# steps>

Treat exactly like Increment with

<delta> == (<max>-<min>)/<# steps>

Table

The parameter name "Table" names a branch of the parameter tree rather than a single leaf. One of the leaves of this branch can be named "Labels" and, if provided, is to be assigned a string value containing a list of column names. For example:
...
   (Rx_Clock_PDF
      (Usage Info)
      (Type Float)
      (Format Table
         (Labels Row_No Time_UI Density)
         (-50 -0.1 1e-35)
         (-49 -0.98 2e-35)
         ...
         (0 0 1e-2)
         ...
         (49 0.98 2e-35)
         (50 0.1 1e-35)
      ) | End Table
   ) | End Rx_Clock_PDF

Gaussian <mean> <sigma>

Dual-Dirac <mean> <mean> <sigma>

Composite of two Gaussian

DjRj <minDj> <maxDj> <sigma>

Convolve Gaussian (sigma) with uniform Modulation PDF

Default <value>

Depending on the Type, <value> will provide a default value for the parameter. For ex, if the Type is Boolean, <value> could be True or False, if the Type is Integer, the <value> can be an integer value.

Description <string>:

ASCII string following Description describes a reserved parameter, model specific parameter, or the Algorithmic model itself. It is used by the EDA platform to convey information to the end-user. The entire line has to be limited to IBIS line length specification.

String literals begin and end with a double quote (") and no double quotes are allowed inside the string literals.

The location of Description will determine what the parameter or model is being described.

Note that in the context of Algorithmic Model for type ‘Corner’, <slow value> and <fast value> align implicitly to slow and fast corners, and <slow value> does not have to be less than <fast value>. For type ‘Range’ and ‘Increment’, <min value>, <max value> does not imply slow and fast corners.

Notes:

1. Throughout the section, text strings inside the symbols “<” and “>” should be considered to be supplied or substituted by the model maker. Text strings inside “<” and “>” are not reserved and can be replaced.

2. Throughout the document, terms “long”, “double” etc. are used to indicate the data types in the ANSI ‘C’ programming language.

|=============================================================================

|

| KEYWORD DEFINITIONS:

|

|=============================================================================

|    Keywords:  [Algorithmic Model], [End Algorithmic Model]

|    Required:  No

| Description:  Used to reference an external compiled model. This compiled model encapsulates signal processing functions. In the case of a receiver it may additionally include clock and data recovery functions. The compiled model can receive and modify waveforms with the analog channel, where the analog channel consists of the transmitter output stage, the transmission channel itself and the receiver input stage. This data exchange is implemented through a set of software functions. The signature of these functions is elaborated in section 10 of this document. The function interface must comply with ANSI ‘C’ language.

|  Sub-Params:  Executable

| Usage Rules:  The [Algorithmic Model] keyword must be positioned within a [Model] |		    section and it may appear only once for each [Model] keyword in a   |		    .ibs file. It is not permitted under the [Submodel] keyword.

| 		    The [Algorithmic Model] keyword is allowed in a [Model] if the      |	  	    [Model] is used in [Pins] that are not referenced in the [Diff Pin] |		    section. (To allow for single ended models??)

|		    Differential signals are handled in a manner identical to the       |		    [External Model] keyword.

[Algorithmic Model]

|		Begins Algorithmic Model Section

Subparameter Definitions:

Executable:

Three entries follow the Executable subparameter on each line:

Platform_Compiler_Bits	File_Name	Parameter_File

The Platform_Compiler_Bits entry provides the name of the operating system, compiler and its version and the number of bits the shared object library is compiled for. It is a string without white spaces, consisting of three fields separated by an underscore ’_’. The first field consists of the name of the operating system followed optionally by its version. The second field consists of the name of the compiler followed by optionally by its version. The third field is an integer indicating the platform architecture. If the version for either the operating system or the compiler contains an underscore, it must converted to a hyphen ‘-‘. This is so that an underscore is only present as a separation character in the entry.

The architecture entry can be either “32” or “64”. Examples of  Platform_Compiler_Bits:

Linux_gcc3.2.3_32

Solaris5.10_gcc4.1.1_64

Solaris_cc5.7_32

Windows_VisualStudio7.1.3088_32

HP-UX_accA.03.52_32

The EDA tool will check for the compiler information and verify if the shared object library is compatible with the operating system and platform._

Multiple occurrences, without duplication, of Executable are permitted to allow for providing shared object libraries for as many combinations of operating system platforms and compilers for the same algorithmic model.

The File_Name provides the name of the shared library file. The shared object library should be in the same directory as the IBIS (.ibs) file.

The Parameter_File entry provides the name of the parameter file with an extension of .ami. This must be an external file and should reside in the same directory as the .ibs file and the shared object library file. It will consist of reserved and model specific (user defined) parameters for use by the EDA tool and for passing parameter values to the model.

The model parameter file must be organized in the parameter tree format as discussed in section 3.1.2.6 of “NOTES ON ALGORITHMIC MODELING INTERFACE AND PROGRAMMING GUIDE”, Section 10 of this document. The file must have 2 distinct sections, or sub-trees, ‘Reserved_Parameters’ section and ‘Model_Specific’ section with sections beginning and ending with parentheses. The complete tree format is described in the section 3.1.2.6 of the Section 10 of this document.

The ‘Reserved_Parameter’ section is required while the ‘Model_Specific’ section is optional. The sub-trees can be in any order in the parameter file. The ‘|’ character is the comment character. Any text after the ‘|’ character will be ignored by the parser.

The Model Parameter File must be organized in the following way

(my_AMIname				|Name given to the Parameter file

(Reserved_Parameters 		|Required heading to start the |required Reserve_Parameters section

…  (reserved parameter text)		|

…

) 				|End of Reserved_Parameters section

(Model_Specific 			|Required heading to start the |optional Model_Specific section

…  (Model Specific parameter text) |

…

…

) 					|End of Model_Specific section

(Description <string>)		|description of the model (optional)

)  					|End my_AMIname parameter file

Reserved Parameters: Init_Returns_Impulse, GetWave_Exists, Max_Init_Aggressors and Ignore_Bits

The model parameter file must have a sub-tree with the heading ‘Reserved_Parameters’. This sub-tree shall contain all the reserved parameters for the model.

The following reserved parameters are used by the EDA tool and are required if the [Algorithmic Model] keyword is present. The entries following the reserved parameters points to its usage, type and default value.

All reserved parameters must be in the following format:

(parameter_name (Usage <usage>)(Type <data_type>) (Default <values>)  (Description <string>))

Init_Returns_Impulse:

Init_Returns_Impulse is of usage Info and type Boolean. It tells the EDA platform whether the AMI_Init function returns a modified impulse response.  When this value is set to True, the model returns the convolution of the input impulse response with the impulse response of the equalization.

GetWave_Exists:

GetWave_Exists is of usage Info and type Boolean. It tells the EDA platform whether the “AMI_GetWave” function is implemented in this model.  Note that if Init_Returns_Impulse is set to “False”, then Getwave_Exists MUST be set to “True”.

The following reserved parameters are optional.  If these parameters are

not present, the values are assumed as “0”.

Max_Init_Aggressors:

Max_Init_Aggressors is of usage Info and type Integer. It tells the EDA platform how many aggressor Impulse Responses the AMI_Init function is capable of processing.

Ignore_Bits:

Ignore_Bits is of usage Info and type Integer. It tells the EDA platform how long the time variant model takes to complete initialization.  This parameter is meant for AMI_GetWave functions that model how equalization adapts to the input stream.  The value in this field tells the EDA platform how many bits of the AMI_Getwave output should be ignored.

The following reserved parameter provides textual description to the user defined parameters.

Tx-only reserved parameters: Tx_Jitter and Tx_DCD

These reserved parameters only apply to Tx models.  These parameters are optional; if the parameters are not specified, the values default to “no jitter specified in the model (“0” jitter). If specified, they must be in the following format:

(<parameter_name> (Usage <usage>)(Type <data_type>) (Format <data format>) (Default <values>) (Description <string>))

Tx_Jitter:

Tx_Jitter can of Usage Info and Out and can be of Type Float or UI. It can be of Data Format Gaussian, Dual-Dirac, DjRj or Table. It tells the EDA platform how much jitter exists at the input to the transmitter’s analog output buffer.  Several different data formats are allowed as listed. Examples of Tx_Jitter declarations are:

(Tx_Jitter (Usage Info)(Type Float)(Format Gaussian <mean> <sigma>))

(Tx_Jitter (Usage Info)(Type Float)(Format Dual-Dirac <mean> <mean> <sigma>))

(Tx_Jitter (Usage Info)(Type Float)(Format DjRj <minDj> <maxDj> <sigma>))

(Tx_Jitter (Usage Info)(Type Float)(Format Table

(Labels Row_No Time Probability)

(-5  -5e-12  1e-10)

(-4  -4e-12  3e-7)

(-3  -3e-12  1e-4)

(-2  -2e-12  1e-2)

(-1  -1e-12  .29)

(0    0      .4)

(1    1e-12  .29)

(2    2e-12  1e-2)

(3    3e-12  1e-4)

(4    4e-12  3e-7)

(5    5e-12  1e-10) ))

Tx_DCD:

Tx_DCD (Transmit Duty Cycle Distortion) can be of Usage Info and Out. It can be of Type  Float and UI and can have Data Format of Value, Range and Corner. It tells the EDA platform the maximum percentage deviation of the duration of a transmitted pulse from the nominal pulse width. Example of TX_DCD declaration is:

(Tx_DCD (Usage Info)(Type Float)(Format Range <typ> <min> <max>))

Rx-only reserved parameters: Rx_Clock_PDF and Rx_Receiver_Sensitivity

These reserved parameters only apply to Rx models.  These parameters are optional; if the parameters are not specified, the values default to “0”. If specified, they must be in the following format:

(<parameter_name> (usage <usage>)(data type <data_type>) (data format <data format> (Default <values>) (Description <string>))

Rx_Clock_PDF:

Rx_Clock_PDF can be of Usage Info and Out and of Type Float and UI and of Data Format Gaussian, Dual-Dirac, DjRj or Table. Rx_Clock_PDF tells the EDA platform the Probability Density Function of the recovered clock.  Several different data formats are allowed as listed. Examples of Rx_Clock_PDF declarations are:

(Rx_Clock_PDF (Usage Info)(Type Float)(Format Gaussian <mean> <sigma>))

(Rx_Clock_PDF (Usage Info)(Type Float)( Format Dual-Dirac <mean> <mean> <sigma>))

(Rx_Clock_PDF (Usage Info)(Type Float)( Format DjRj <minDj> <maxDj> <sigma>))

(Rx_Clock_PDF (Usage Info)(Type Float)( Format Table

(Labels Row_No Time Probability)

(-5  -5e-12  1e-10)

(-4  -4e-12  3e-7)

(-3  -3e-12  1e-4)

(-2  -2e-12  1e-2)

(-1  -1e-12  .29)

(0    0      .4)

(1    1e-12  .29)

(2    2e-12  1e-2)

(3    3e-12  1e-4)

(4    4e-12  3e-7)

(5    5e-12  1e-10) ))

Rx_Receiver_Sensitivity:

Rx_Receiver_Sensitivity can be of Usage Info and Out and of Type Float and of Data Format Value, Range and Corner. Rx_Receiver_Sensitivity tells the EDA platform the voltage needed at the receiver data decision point to ensure proper sampling of the equalized signal.  In this example, 100 mV (above +100 mV or below -100 mV) is needed to ensure the signal is sampled correctly. Examples of Rx_Clock_PDF declarations are:

(Rx_Receiver_Sensitivity (Usage Info)(Type Float)(Format Value <value>))

(Rx_Receiver_Sensitivity (Usage Info)(Type Float)(Format Range <typ> <min> <max>))

(Rx_Receiver_Sensitivity (Usage Info)(Type Float)(Format Corner <slow> <fast>))

The general rules, allowed usage and a brief summary of the data types and data formats allowed for each reserved parameter is presented in the following tables.

______________________________________________

|  General     Rules     	|   Allowed Usage   	|

========================================================================

| Reserved Parameter      	| Required   Default     	| Info In Out InOut 	|

------------------------------------------------------------------------

| Init Returns Impulse    		|    Yes        NA       	|  X                		|

| GetWave Exists         		|    Yes        NA       	|  X                		|

| Ignore Bits             		|    No         0        	|  X       X        		|

| Max Init Aggressors     		|    No         0        	|  X                		|

| Tx Jitter               		|    No     No Jitter    	|  X       X        		|

| Tx DCD                  		|    No         0        	|  X       X        		|

| Rx Receiver Sensitivity 	|    No         0        	|  X       X        		|

| Rx Clock PDF            		|    No   Clock Centered |  X       X        		|

------------------------------------------------------------------------

_____________________________________________

|                 Data Type                 		|

======================================================

| Reserved Parameter      	| Float |  UI  | Integer | String | Boolean 	|

-----------------------------------------------------------------------

| Init Returns Impulse    	|  	                                       X    	   	|

| GetWave Exists          	|           		                           X		|

| Ignore Bits             		|                   	X            	           		|

| Max Init Aggressors     	|                   	X                       		|

| Tx Jitter               		|   X      X                                			|

| Tx DCD                  		|   X      X                                			|

| Rx Receiver Sensitivity 	|   X                                       			|

| Rx Clock PDF            	|   X      X                                			|

-----------------------------------------------------------------------

<INSERT DATA FORMAT TABLE HERE>

Model Specific Parameters:

The Following section describes the user defined parameters. The algorithmic model expects these parameters and their values to function appropriately. The model maker can specify any number of user defined parameters for their model. The user defined parameter section subtree must begin with the reserved parameters ‘Model_Specific’.

The user defined parameters must be in the following format:

(<parameter_name> (usage <usage>) (Type <data type>) (Format <data format>) (Default <values>) (Description <string>))

And

(<tap_parameter_name> (Usage <usage>) (Type Tap <tap number>) (Format <data format>) (Default <values> ))

Where:

<parameter_name>

Arbitrary Names (IP Vendors choose what names their models accept; The IBIS parser does not verify the names. It would, however, check the usage, data type, data format and values.) The names, types and description of the parameters used for algorithmic modeling shall not be specified in the IBIS standard.

<tap_parameter_name>

Name for an N tap equalizer, where N is the number of taps in the equalizer. All the names for the N tap equalizer must be the same. For ex., a four tap equalizer must have four parameters with the same <tap parameter name>

<tap number>

The numbers for a particular tap for the N tap equalizer with the name <tap_parameter_name>. Tap number 0 is the primary (cursor) tap, negative tap numbers are pre-cursor taps, positive tap numbers are post_cursor taps. The sum of the absolute values of taps with the same parameter name must be <= 1.

--------------------------------------------------------

| Example of Parameter File

--------------------------------------------------------

(mySampleAMI				| Name given to the Parameter file

(Description "Sample AMI File")

(Reserved_Parameters 		| Required heading

(Ignore_Bits (Usage Info) (Type Integer) (Default 21))

(Max_Init_Aggressors (Usage Info) (Type Integer)(Default 25))

(Init_Returns_Impulse (Usage Info) (Type Boolean)(Default True))

(GetWave_Exists (Usage Info) (Type Boolean) (Default True))

) 					| End Reserved_Parameters

(Model_Specific 			| Required heading

(txtaps

(tapid (Usage Inout)(Format Range -1 3) (Type Integer) )

(txtapcoeff

(-1 (Format Range -0.1 0.1 0.1) (Type float) (Default 0))

(0 (Format Range 1 1 0) (Type float) (Default 1))

(1 (Format List -0.1 0.1) (Type float)(Default  0))

(2 (Format Range -0.01 0.02 0.005) (Type float)
  	     (Default 0))

(3 (Format Range -0.01 0.02 0.005) (Type float)
           Default 0))

)  				| End txtapcoeff

)  				| txtaps

(tx_freq_offset (Format Range 0 150 1) (Type UI) (Default 0))

) 					| End Model_Specific

)  | End SampleAMI

[End Algorithmic Model]

Ends Algorithmic Model Section

|-------------------------------------------------------------------------------

| Example of RX model in [Algorithmic Model]

|-------------------------------------------------------------------------------

[Algorithmic Model]

Executable Windows_VisualStudio_32 example_rx.dll example_rx_params.ami

[End Algorithmic Model]

|-------------------------------------------------------------------------------

| Example of TX model in [Algorithmic Model]:

|-------------------------------------------------------------------------------

[Algorithmic Model]

Executable Windows_VisualStudio_32 tx_getwave.dll tx_getwave_params.ami

Executable Solaris_cc_32 libtx_getwave.so tx_getwave_params.ami

[End Algorithmic Model

|

|===============================================================================

The Section 10 addition is below:

|=============================================================================

|=============================================================================

|

|                                 Section 10

|

|     N O T E S    O N    A L G O R I T H M I C  M O D E L I N G  I N T E R F A C E | 					 A N D

| 			    P R O G R A M M I N G  G U I D E

|

|=============================================================================

|=============================================================================

|

| INTRODUCTION:

|

| This section is organized as an interface and programming guide for writing the   | executable code to be interfaced by the [Algorithmic Model] keyword described in  | Section 6c.  Section 10 is structured as a reference document for the software    | engineer.

|

| TABLE OF CONTENTS

1 OVERVIEW

2 APPLICATION SCENARIOS

2.1 Linear, Time-invariant equalization Model

2.2 Nonlinear, and / or Time-variant equalization Model

3 FUNCTION SIGNATURES

3.1 AMI_Init

3.1.1 Declaration

3.1.2 Arguments

3.1.1 impulse_matrix

3.1.2 row_size

3.1.3 aggressors

3.1.4 sample_interval

3.1.5 bit_time

3.1.6 AMI_parameters (_in and _out)

3.1.7 AMI_memory_handle

3.1.8 msg

3.1.3 Return Value

3.2 AMI_GetWave

3.2.1 Declaration

3.2.2 Arguments

3.2.10 wave

3.2.11 wave_size

3.2.12 clock_times

3.2.13 AMI_memory

3.2.3 Return Value

3.3 AMI_Close

3.3.1 Declaration

3.3.2 Arguments

3.3.3 Return Value

3.3.13 AMI_memory

4 CODE SEGMENT EXAMPLES

--------------------------------------------------------------------------

1 OVERVIEW

----------

The algorithmic model of a Serializer-Deserializer (SERDES) transmitter or receiver consists of three functions: ‘AMI_Init’, ‘AMI_GetWave’ and ‘AMI_Close’. The interfaces to these functions are designed to support three different phases of the simulation process: initialization, simulation of a segment of time, and termination of the simulation.

These functions (‘AMI_Init’, ‘AMI_GetWave’ and ‘AMI_Close’) should all be supplied in a single shared library, and their names and signatures must be as described in this section. If they are not supplied in the shared library named by the Executable sub-parameter, then they shall be ignored. This is acceptable so long as

- The entire functionality of the model is supplied in the shared library.

- All termination actions required by the model are included in the shared library.

The 3 functions can be included in the shared object library in one of the 2 following combinations

Case 1: Shared library has AMI_Init, AMI_Getwave and AMI_Close.

Case 2: shared library has AMI_Init and AMI_Close.

Case 3: Shared library has only AMI_Init.

Please note that the function ‘AMI_Init’ is always required.

The interfaces to these functions are defined from three different perspectives. In addition to specifying the signature of the functions to provide a software coding perspective, anticipated application scenarios provide a functional and dynamic execution perspective, and a specification of the software infrastructure provides a software architecture perspective. Each of these perspectives is required to obtain interoperable software models.

2 APPLICATION SCENARIOS

-----------------------

2.1 Linear, Time-invariant Equalization Model

1. From the system netlist, the EDA platform determines that a given [Model]

is described by an IBIS file.

2. From the IBIS file, the EDA platform determines that the [Model] is

described at least in part by an algorithmic model, and that the AMI_Init

function of that model returns an impulse response for that [Model].

3. The EDA platform loads the shared library containing the

algorithmic model, and obtains the addresses of the AMI_Init, AMI_GetWave,

and AMI_Close functions.

4. The EDA platform assembles the arguments for AMI_Init. These arguments

include the impulse response of the channel driving the [Model], a handle

for the dynamic memory used by the [Model], the parameters for configuring

the [Model], and optionally the impulse responses of any crosstalk interferers.

5. The EDA platform calls AMI_Init with the arguments previously prepared.

6. AMI_Init parses the configuration parameters, allocates dynamic memory,

places the address of the start of the dynamic memory in the memory handle, computes the impulse response for the [Model] and passes it back to the EDA platform.

7. The EDA platform completes the rest of the simulation/analysis using the  impulse response from AMI_Init as a complete representation of the behavior of the given [Model].

8. Before exiting, the EDA platform calls AMI_Close, giving it the address in the memory handle for the [Model].

9. AMI_Close de-allocates the dynamic memory for the block and performs whatever other clean-up actions are required.

10. The EDA platform terminates execution.

2.2 Nonlinear, and / or Time-variant Equalization Model

1. From the system netlist, the EDA platform determines that a given block is described by an IBIS file.

2. From the IBIS file, the EDA platform determines that the block is described at least in part by an algorithmic model.

3. The EDA platform loads the shared library or shared object file containing the algorithmic model, and obtains the addresses of the AMI_Init, AMI_GetWave, and AMI_Close functions.

4. The EDA platform assembles the arguments for AMI_Init. These arguments include the impulse response of the channel driving the block, a handle for the dynamic memory used by the block, the parameters for configuring the block, and optionally the impulse responses of any crosstalk interferers.

5. The EDA platform calls AMI_Init with the arguments previously prepared.

6. AMI_Init parses the configuration parameters, allocates dynamic memory and places the address of the start of the dynamic memory in the memory handle. AMI_Init may also compute the impulse response of the block and pass the modified impulse response to the EDA tool.

7. A long time simulation may be broken up into multiple time segments. For each time segment, the EDA platform computes the input waveform to the [Model] for that time segment. For example, if a million bits are to be run, there can be 1000 segments of 1000 bits each, i.e. one time segment comprises 1000 bits.

8. For each time segment, the EDA platform calls the AMI_GetWave function, giving it the input waveform and the address in the dynamic memory handle for the block.

9. The AMI_GetWave function computes the output waveform for the block.

In the case of a transmitter, this is the Input voltage to the receiver. In the case of the receiver, this is the voltage waveform at the decision point of the receiver.

10. The EDA platform uses the output of the receiver AMI_GetWave function to complete the simulation/analysis. For transmitter, it simply passes the output to the receiver AMI_GetWave.

11. Before exiting, the EDA platform calls AMI_Close, giving it the address in the memory handle for the block.

12. AMI_Close de-allocates the dynamic memory for the block and performs whatever other clean-up actions are required.

13. The EDA platform terminates execution.

3 FUNCTION SIGNATURES

---------------------

3.1 AMI_Init

3.1.1 Declaration

long AMI_Init (double *impulse_matrix,

long row_size,

long aggressors,

double sample_interval,

double bit_time,

char *AMI_parameters_in,

char **AMI_parameters_out,

void **AMI_memory_handle,

char **msg)

3.1.2 Arguments

3.1.2.1 impulse_matrix: Impulse matrix is the channel impulse response     matrix. The impulse values are in volts and are uniformly spaced in time.  The sample spacing is given by the parameter ‘sample_interval’.

The ‘impulse_matrix’ is stored in a single dimensional array of floating point numbers which is formed by concatenating the columns of the impulse response matrix, starting with the first column and ending with the last column. The matrix elements can be retrieved /identified using

impulse_matrix[idx] = element (row, col)

idx = col * number_of_rows + row

row – row index , ranges from 0 to row_size-1

col – column index, ranges from 0 to aggressors

The first column of the impulse matrix is the impulse response for the primary channel. The rest are the impulse responses from aggressor drivers to the victim receiver.

The AMI_Init function may return a modified impulse response by modifying the first column of impulse_matrix. If the impulse response is modified, the new impulse response is expected to represent the concatenation of the model block with the blocks represented by the input impulse response.

The aggressor columns of the matrix should not be modified.

3.1.2.2 row_size: the number of rows in the impulse matrix

3.1.2.3 aggressors: the number of aggressors in the impulse matrix.

3.1.2.4 sample_interval: This is the sampling interval of the impulse matrix. Sample_interval is usually a fraction of the highest data rate (lowest bit_time) of the device.

E.g.,

Sample_interval = (lowest_bit_time/64)

3.1.2.5 bit_time: the bit time or unit interval (UI) of the current data, e.g., 100 ps, 200 ps etc. The shared library may use this information along with the impulse matrix to initialize the filter coefficients.

3.1.2.6 AMI_parameters (_in and _out)

Memory for AMI_parameters_in is allocated and de-allocate by the EDA platform. The memory pointed to by AMI_parameters_out is allocated and de-allocate by the model.

This is a pointer to a string. All the input parameters from the IBIS AMI parameter file are passed using a string that has been formatted as a parameter tree.

Examples of the tree parameter passing is:

(dll

(tx

(taps 4)

(spacing sync)

)

)

And

(root

(branch1

(leaf1 value1)

(leaf2 value2)

(branch2

(leaf3 value3)

(leaf4 value4)

)

(leaf5 value5 value6 value7)

)

)

The syntax for this string is:

1. Neither names nor individual values can contain white space characters.

2. Parameter name/value pairs are always enclosed in parentheses, with the value separated from the name by white space.

3. A parameter value in a name/value pair can be either a single value or a list of values separated by whitespace.

4. Parameter name/value pairs can be grouped together into parameter groups by starting with an open parenthesis followed by the group name followed by the concatenation of one or more name/value pairs followed by a close parenthesis.

5. Parameter name/values pairs and parameter groups can be freely intermixed inside a parameter group.

6. The top level parameter string must be a parameter group.

7. White space is ignored, except as a delimiter between the parameter name and value.

8. Parameter values can be expressed either as a string literal, decimal number or in the standard ANCI ‘C’ notation for floating point numbers (e.g., 2.0e-9). String literal values are delimited using a double quote (") and no double quotes are allowed inside the string literals.

9. A parameter can be assigned an array of values by enclosing the parameter name and the array of values inside a single set of parentheses, with the parameter name and the individual values all separated by white space.

The modified BNF specification for the syntax is:

<tree>:

<branch>

<branch>:

( <branch name> <leaf list> )

<leaf list>:

<branch>

<leaf>

<leaf list> <branch>

<leaf list> <leaf>

<leaf>:

( <parameter name> whitespace <value list> )

<value list>:

<value>

<value list> whitespace <value>

<value>:

<string literals>

<decimal number>

<decimal number>e<exponent>

<decimal number>E<exponent>

3.1.2.7 AMI_memory_handle: Used to point to local storage for the algorithmic block being modeled and shall be passed back during the AMI_GetWave calls. e.g. a code snippet may look like the following

my_space = allocate_space( sizeof_space );

status = store_all_kinds_of_things( my_space );

*sedes_memory_handle = my_space;

The memory pointed to by AMI_handle is allocated and de-allocated by the model.

3.1.2.8 msg: (optional) Provides descriptive, textual message from the algorithmic model to the EDA platform. It must provide a character string message that can be used by EDA platform to update log file or display in user interface.

3.1.3 Return Value

1 for success

0 for failure

3.2 AMI_GetWave

3.2.1 Declaration

long AMI_GetWave (double *wave,

long wave_size,

double *clock_times,

char **AMI_parameters_out,

void *AMI_memory);

3.2.2 Arguments

3.2.2.1 wave: A vector of a time domain waveform, sampled uniformly at an interval specified by the ‘sample_interval’ specified during the init call. The wave is both input and output. The EDA platform provides the wave. The algorithmic model is expected to modify the waveform in place.

Depending on the EDA platform and the analysis/simulation method chosen, the input waveform could include many components. For example, the input waveform could include:
- The waveform for the primary channel only.
- The waveform for the primary channel plus crosstalk and amplitude noise.
- The output of a time domain circuit simulator such as SPICE.

It is assumed that the electrical interface to either the driver or the receiver is differential. Therefore, the sample values are assumed to be differential voltages centered nominally around zero volts. The algorithmic model's logic threshold may be non-zero, for example to model the differential offset of a receiver; however that offset will usually be small compared to the input or output differential voltage.

The output waveform is expected to be the waveform at the decision point of the receiver (that is, the point in the receiver where the choice is made as to whether the data bit is a “1” or a “0”). It is understood that for some receiver architectures, there is no one circuit node which is the decision point for the receiver. In such a case, the output waveform is expected to be the equivalent waveform that would exist at such a node were it to exist.

3.2.2.2 wave_size: Number of samples in the waveform vector.

3.2.2.3 clock_times: Vector to return clock times. The clock times are referenced to the start of the simulation (the first AMI_GetWave call). The time is always greater or equal to zero. The last clock is indicated by putting a value of -1 at the end of clocks for the current wave sample. The clock_time vector is allocated by the EDA platform and is guaranteed to be greater than the number of clocks expected during the AMI GetWave call.

The clock times are the times at which clock signal at the output of the clock recovery loop crosses the logic threshold. It is to be assumed that the input data signal is sampled at exactly one half clock period after a clock time.

3.2.2.4 AMI_parameters_out: (optional) A handle to a ‘tree string’ as described in 1.3.1.2.6. This is used by the algorithmic model to return dynamic information and parameters. The memory for this string is to be allocated and deleted by the algorithmic model.

3.2.2.5 AMI_memory: this is the memory which was allocated during the init call

3.2.2.6 Return Value

1 for success

0 for failure

3.3 AMI_Close

3.3.1 Declaration

long AMI_Close(void * AMI_memory);

3.3.2 Arguments

3.3.2.1 AMI_memory

Same as for AMI_GetWave. See section 3.2.2.4

3.3.3 Return Value

1 for success

0 for failure

4 CODE SEGMENT EXAMPLES

------------------------

extern long AMI_GetWave (wave, wave_size, clock_times, AMI_memory);

my_space = AMI_memory;

clk_idx=0;

time = my_space->prev_time + my_space->sample_interval;

for(i=0; i<wave_size; i++)

{

wave = filterandmodify(wave, my_space);

if (clock_times && found_clock (my_space, time))

clock_times[clk_idx++] = getclocktime (my_space, time);

time += my_space->sample_interval;

}

clock_times[clk_idx] = -1;   //terminate the clock vector

Return 1;

******************************************************************************

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

This section of the IBIS specification has been driven primarily by the following factors:

1. The interaction between a SERDES and the system surrounding it is quite

complex, thus requiring sophisticated and detailed modeling.

2. There is considerable variation in the architectures and circuit techniques

used in SERDES devices.

3. There is not a commonly accepted set of parameters that can be measured to

fully and reliably characterize the performance of a given SERDES device

independently from the system that surrounds it.

Because of these factors, IP vendors' experience has been that customers use

the models delivered by the IP vendor as a form of performance specification.

If the model predicts a level of performance in a given application, then the

IP is held to that level of performance or better when the system is tested.

For this reason, IP vendors are reluctant to supply any but most detailed and

accurate models they can produce. This is a fundamental shift in that in the

past, the models that were presumed to be utterly complete and reliable were

SPICE models, and IBIS models were understood to be a useful approximation

that could be shared without divulging sensitive proprietary information.

By setting the algorithmic model as the primary deliverable, this specification

maximizes the flexibility available to the model developers and also maximizes

the degree of protection for proprietary information. By standardizing the

interface to these algorithmic models, this specification also enables the

required degree of interoperability.

******************************************************************************

Any Other Background Information

SUPPLEMENTARY MATERIAL (To be resolved before final submission.)

REVISION HISTORY CHANGES:

********************************************************************************