|=============================================================================

|

|          I B I S     C o n n e c t o r     S p e c i f i c a t i o n

|

|Last Updated: 5-23-2000, KC

|

|=============================================================================

|=============================================================================

|

|                    T A B L E    O F    C O N T E N T S

|

|=============================================================================

|=============================================================================

|

| Section 1 .... GENERAL INTRODUCTION

| Section 2 .... STATEMENT OF INTENT

| Section 3 .... REVISION HISTORY

| Section 4 .... GENERAL SYNTAX RULES AND GUIDELINES

| Section 5 .... KEYWORD TREE DIAGRAM

| Section 6 .... KEYWORD DESCRIPTIONS

| Section 7 .... MATRIX DATA

|=============================================================================

|

|                                 Section 1

|

|                  G E N E R A L    I N T R O D U C T I O N

|

|=============================================================================

|=============================================================================

|

| This section gives a general overview of the remainder of this document.

|

| Sections 2 to 5 contain general information about the IBIS versions and

| the general rules and guidelines.

| The remaining sections describe the connector keywords and usage rules.

|

|=============================================================================

|=============================================================================

|

|                                 Section 2

|

|                  S T A T E M E N T    O F    I N T E N T

|

|=============================================================================

|=============================================================================

|

| In order to enable an industry standard method to electronically transport

| IBIS connector modeling data between connector vendors, simulation vendors,

| and end customers, this template is proposed.  The intention of this

| template is to specify a consistent format that can be parsed by software,

| allowing simulation vendors to derive models compatible with their own

| products.

|

| One goal of this template is to represent the current state of IBIS data,

| while allowing a growth path to more complex models / methods (when deemed

| appropriate).  This would be accomplished by a revision of the base

| template, and possibly the addition of new keywords or categories.

|

| Another goal of this template is to ensure that it is simple enough for

| connector vendors and customers to use and modify, while ensuring that

| it is rigid enough for simulation vendors to write reliable parsers.

|

| This template is meant to contain a complete description of the

| elements on an entire connector or family of connectors.

|

| Finally this template is intended to provided a basis for future versions

| which are expected to be backward compatibility with this initial version.
|

|=============================================================================

|=============================================================================

|

|                                 Section 3

|

|                         R E V I S I O N   H I S T O R Y

|

|=============================================================================

|=============================================================================

|5-23-2000, KC

| - Rolled to V0.943

| - Under “General Syntax” in section 4; added item 15 describing connector
|   modeling as using transmission-line models as opposed to lumped models.
|   Ian is to supply alternate text for this section.
| - changed the “required” description for [Begin_Cn_Auto_Map],
|   [Begin_Cn_Pin_Map], and [Begin_Cn_Phy_Map] keywords to clarify their usage.
| - Added to description of [Begin_Cn_Phy_Map] more information about it’s
|   usage as a mechanical pin connection checking aid.

| - Added to description of [Begin_Cn_Phy_Map] more detail of how the
|   physical map is created.

| 
|5-11-2000,KC

| - Rolled to V0.942

| - Improved the clarity of the section on "swath"

| - Changed the words "EDGE ONLY" to "EDGE ONLY" to better represent 
| the function.

| - Changed the minimum number of "EDGE ONLY" columns from 1 to 0

|

|4-10-2000, KC

| - Rolled to V0.941

| - Changed file extent from .ibiscnn to .icm per IBIS meeting at Design Con

| - Changed line length to 120 limit instead of mixed 80 and 120 limit

| - Removed keywords xxx and xxx and replaced with single keyword Web

| - Removed [Email] keyword and loosened restrictions on [Web_Site] keyword

| - Changed keyword [Web_Site] to [Support].

| - Changed General Syntax item 12 on 100,000 pin limit to be a warning.

| - Changed [Comment Char] syntax to match IBIS V3.2 specification exactly.

| - Modified the [Source] and [Notes] sections to indicate a max. of 24 lines

|   is recommended but remove the requirement that only 24 lines be used.

| - Modified the [Disclaimer] and [Copyright] sections to indicate a max. of 96

|   lines is recommended but remove the requirement that only 96 lines be used.

| - Changed [Manufacturer] to say "should" instead of "must" for use of

|   consistent name (last line of section)

| - The [Redistribution] keyword was changed:

|     Manufacturer_No_Fee was deleted

|     Full_Public_No_Fee replaced with YES

|     Do_Not_Distribute replaced with NO

|     Specific replaced with SPECIFIC

| - Changed Max_Slew_Time to Min_Slew_Time (it was a typo)

| - [Begin_Cn_Model_List] keyword notes item 6 changed from "may be" to

|   "shall be"

| - [Begin_Cn_Model] Change ModelName to less than 40 characters as opposed

|   to 20.

| - [Begin_Cn_Model] Change SGR description in item 5 to allow decimal

|   e.g. 2.5:1

| - Begin_Cn_Pin_Map and Begin_Cn_Auto_Map change to allow > 100,000 pins with

|   warnings only

| - [Cn_Rows_of_Pins] and [Cn_Columns_of_Pins] change to allow > 100,000 pins

|   with warnings only

| - [Begin_Cn_Auto_Map] changed to allow > 100,000 pins with warning only.

| - [Begin_Cn_Phy_Map] changed the associated keyword [Row] to [PhyRow]

|   in order to avoid confusion with the keyword [Row] used for matrices

| - incorporated several dozen typo fixes and readability corrections found

|   by Mike Labonte.

|

|=============================================================================

|=============================================================================

|

|                                 Section 4

|

|   G E N E R A L   S Y N T A X   R U L E S   A N D   G U I D E L I N E S

|

|=============================================================================

|=============================================================================

|

| 1)  The content of the files is case sensitive, except for reserved

|     words and keywords.  File names must be all lower case.

|

| 2)  The following words are reserved words and must not be used for

|     any other purposes in the document:

|        PWRGND- reserved model name, used with power or ground pins,

|        POWER - reserved model name, used with power supply pins,

|        GND   - reserved model name, used with ground pins,

|        RET   - reserved word indicating either power or ground used

|                as a high-frequency return path.

|        NC    - reserved model name, used with no-connect pins,

|        NA    - used where data not available.

|

| 3)  Each line in the file must be 120 characters or less excluding

|     end of line characters.

|

| 4)  Anything following the comment character is ignored and considered a

|     comment on that line.  The default "|" (pipe) character can be changed

|     by the keyword [Comment Char] to any other character. The [Comment Char]

|     keyword can be used throughout the file as desired.

|

| 5)  Keywords must be enclosed in square brackets, [], and must start in

|     column 1 of the line.  No space or tab is allowed immediately after the

|     opening bracket '[' or immediately before the closing bracket ']'.  If

|     used, only one space (' ') or underscore ('_') character separates the

|     parts of a multi-word keyword.

|

| 6)  Underscores and spaces are equivalent in keywords.  Spaces are not

|     allowed in sub-parameter names.

|

| 7)  Valid scaling factors are:

|        T = tera        k = kilo        n = nano

|        G = giga        m = milli       p = pico

|        M = mega        u = micro       f = femto

|     When no scaling factors are specified, the appropriate base units are

|     assumed.  (These are volts, amperes, ohms, farads, henries, and seconds.)

|     The parser looks at only one alphabetic character after a numerical

|     entry, therefore it is enough to use a single character suffix to scale

|     the parameters.  However, for clarity, it is allowed to use full

|     abbreviations for the units, (e.g., pF, nH, mA, mOhm).  In addition,

|     scientific notation is allowed (e.g., 1.2345e-12).

|

| 8)  WARNING: use of TAB characters may make imported files invalid.

|     The usage of TAB characters is legal but NOT RECOMMENDED.

|     They should be avoided as far as possible.

|     This is to eliminate possible complications which might

|     arise in situations when TAB characters are automatically converted to

|     multiple spaces by text editing, file transferring and similar software.

|     In such cases lines might become longer than allowed causing

|     a syntax error.

|

| 9)  Currents are considered positive when their direction is into the

|     component.

|

| 10) All temperatures are represented in degrees Celsius.

|

| 11) Important information is contained in the sample files provided

|     with the specification and these should be referred to as a

|     starting point for creating new models.

|

| 12) Use of connectors with more than 100000 pins is not recommended and

|     should generate a warning by the parser indicating that CAE tools

|     may not support this model.

|

| 13) The numeric accuracy used in the file should not be significantly

|     greater than the accuracy of the model.

|     It is recommended that for numeric data the mantissa portion be

|     less than or equal to 5 decimal digits.  In no case shall more than

|     15 decimal digits be used.

|     For example:

|        1.2345e-12            is considered acceptable

|        1.23456789e-12        is allowed but not recommended

|        1.234567890123456e-12 is not allowed

|     The 5 digit recommendation will aid in human readability of files

|     and reduce the file size.

|

| 14) Only ASCII characters, as defined in ANSI Standard X3.4-1986, shall be

|     used in an IBIS file.  The use of characters with codes greater than

|     Hexadecimal 07E is not allowed.  Also, ASCII control characters

|     (those numerically less than hexadecimal 20) are not allowed, except

|     for tabs or in a line termination sequence. As mentioned in item 10

|     above, the use of tab characters is discouraged.

|

| 15) All interconnect described in this document shall be considered 
|     distributed transmission lines NOT lumped interconnect.

|     If desired particular users of this model may convert the model into a

|     lumped equivalent model provide the bandwidth needs of the maximum
|     specified rise time are still meet.  This may require additional 
|     sections.
|

|=============================================================================

|=============================================================================

|

|                                 Section 5

|

|                      K E Y W O R D   T R E E   D I A G R A M

|

|=============================================================================

|=============================================================================

|

|  (ml) Indicates that keyword may occur multiple times at any location.

|

|  (m)  Indicates that keyword or sub-parameter may occur multiple times

|       within the context for which it applies.

|

|  /-- Start of File

|  |   -----------------

|  |     |-- [IBIS_Cn_Model_Ver]

|  |     |-- [Comment Char]                           (ml)

|  |     |-- [File Name]

|  |     |-- [File Rev]

|  |     |-- [Date]

|  |     |-- [Source]

|  |     |-- [Notes]

|  |     |-- [Disclaimer]

|  |     |-- [Copyright]

|  |

|  |-- [Begin_Cn_Model_Family]

|  |   -----------------------

|  |     |-- [Begin_Cn_Family_Description]

|  |     |     \-- [End_Cn_Family_Description]

|  |     |-- [Manufacturer]

|  |     |-- [Support]

|  |     |-- [Redistribution]

|  |     |-- [Begin_Cn_Model_List]

|  |     |     \-- [End_Cn_Model_List]

|  |     |

|  |     |-- [Begin_Cn_Model]                         (m)

|  |     |    ---------------

|  |     |     |-- [Begin_Cn_Model_Description]

|  |     |     |     |-- [End_Cn_Model_Description]

|  |     |     |-- [Cn_Number_of_Conductors]

|  |     |     |-- [Cn_Columns_of_Pins]

|  |     |     |-- [Cn_Rows_of_Pins]

|  |     |     |-- [Begin_Cn_Auto_Map]

|  |     |     |     |-- [End_Cn_Auto_Map]

|  |     |     |-- [Begin_Cn_Swath]

|  |     |     |   ----------------

|  |     |     |     |-- [Cn_Z]

|  |     |     |     |-- [End_Cn_Swath]

|  |     |     \-- [End_Cn_Model]

|  |     |

|  |     |-- [Begin_Cn_Pin_Map]                       (m)

|  |     |     \-- [End_Cn_Pin_Map]

|  |     |

|  |     |-- [Begin_Cn_Phy_Map]                       (m)

|  |     |     |-- [PhyRow]                           (m)

|  |     |     \-- [End_Cn_Phy_Map]

|  |     |

|  |     |-- [Begin_Cn_Section]                       (m)

|  |     |   ------------------

|  |     |     |-- [Resistance Matrix]      Banded_matrix, Sparse_matrix,

|  |     |     |                            Full_matrix, Diagonal_Matrix

|  |     |     |   -------------------

|  |     |     |     |-- [Bandwidth]

|  |     |     |     |-- [Row]                        (m)

|  |     |     |-- [Inductance Matrix]      Banded_matrix, Sparse_matrix,

|  |     |     |                            Full_matrix, Diagonal_Matrix

|  |     |     |   -------------------

|  |     |     |     |-- [Bandwidth]

|  |     |     |     |-- [Row]                        (m)

|  |     |     |-- [Capacitance Matrix]     Banded_matrix, Sparse_matrix,

|  |     |     |                            Full_matrix, Diagonal_Matrix

|  |     |     |   --------------------

|  |     |     |     |-- [Bandwidth]

|  |     |     |     |-- [Row]                        (m)

|  |     |     |

|  |     |     \-- [End_Cn_Section]

|  |     |

|  |     \-- [End_Cn_Model_Family]

|  |

|  \-- [End]

|

|=============================================================================

|=============================================================================

|

|                                 Section 6

|

|                      K E Y W O R D   D E S C R I P T I O N S

|

|=============================================================================

|=============================================================================

|

|=============================================================================

| Keyword:        [IBIS_Cn_Model_Ver]

| Required:       Yes

| Description:

|     This keyword allows electronic parsers to immediately determine

|     that this file contains an IBIS connector model.

|     The version number is used to inform the parser what keywords

|     are valid for this model and to allow backward support as new

|     keywords are added.

|

| Usage Context:

|     1) MUST BE THE FIRST KEYWORD in the file.

|     2) It is normally on the first line of the file, but can be preceded

|        by comment lines that must begin with a "|".

|

| Usage Rules:

|     [IBIS_Cn_Model_Ver] MUST BE THE FIRST KEYWORD in the file.

|     It is normally on the first line of the file, but can be preceded

|     by comment lines that must begin with a "|".

|-----------------------------------------------------------------------------

|[IBIS_Cn_Model_Ver]  0.943

|

|=============================================================================

| Keyword:        [File Name]

| Required:       Yes

| Description:

|     Specifies the name of the IBIS file, "filename.icm".  This allows

|     users to recreate the original file name even if it has been shortened

|     or modified during the process of transmitting this file between

|     computers.

| Usage Context:

|     1) MUST BE THE SECOND KEYWORD in the file.

|     2) It is normally on the second line of the file, but can be preceded

|        by comment lines that must begin with a "|".

| Usage Rules:

|     1) The file name must have the file extension .icm

|        Note: icm stands for "Ibis Connector Model"

|     2) The file name must be 40 characters or less in length.

|     3) The file name must be all lower case to avoid problems with certain

|        operating systems.

|     4) This name must match the actual name used for this file.

| Notes on file naming:

|     To make maintenance of libraries of connector models from different

|     companies easier the following recommendations are made:

|     1) It is recommended that the first 8 characters or less of the file name

|        indicate the name of the company creating the model.  Typically the

|        abbreviation indicating the connector company.

|     2) It is recommended that manufacturing company names or abbreviations

|        only be used when the model is created by said company or their

|        representative.  If a user creates a model then the name prefix should

|        represent their company not the manufacturer of the part to avoid

|        confusion of vendor created vs. user created models.

|     3) In the example below note the name of the company manufacturing the

|        connector is "Ibis Connector Manufacturing"  -> abbreviation = iconm

|-----------------------------------------------------------------------------

|[File Name]     iconm_hdi_202.icm

|

|=============================================================================

| Keyword:        [File Rev]

| Required:       Yes

| Description:

|     Version of IBIS connector model as defined by the Manufacturer

|     It is normally on the third line of the file, but can be preceded

|     by comment lines that must begin with a "|".

| Usage Context:

|     1) MUST BE THE THIRD KEYWORD in the file.

| Usage Rules:

|     1) Revision level is set at the discretion of the engineer

|        defining the file.  The following guidelines are recommended:

|        0.x     connector in development, early model

|        1.x     connector model based on design data only not correlated.

|        2.x     file correlated to actual connector measurements

|        3.x     mature product, no more changes likely

|-----------------------------------------------------------------------------

|[File Rev]     0.941                    | Used for .icm file versions

|

|=============================================================================

| Keyword:		[Comment Char]

| Required:		No

| Description:  Defines a new comment character to replace the default

|               "|" (pipe) character, if desired.

| Usage Rules:  The new comment character to be defined must be followed by

|               the underscore character and the letters "char".  For example:

|               "|_char" redundantly redefines the comment character to be

|               the pipe character.  The new comment character is in effect

|               only following the [Comment Char] keyword.  The following

|               characters MAY be used:

|

|                 ! " # $ % & ' ( ) * , : ; < > ? @ \ ^ ` { | } ~

|

| Other Notes:  The [Comment Char] keyword can be used throughout the file, as

|               desired.

|-----------------------------------------------------------------------------

|[Comment Char]  |_char

|

|=============================================================================

| Keyword:        [Date]

| Required:       Yes

| Description:

|     Date this file was last modified

|     This field is provided to insure the last changed date for this

|     file is not lost if the file is transmitted between computer

|     systems.

| Usage Context:

|     1) This keyword may appear anywhere in the file.

|     2) It is recommended that his keyword appear immediately following the

|        keyword [File Rev].

| Usage Rules:

|     1) The argument is limited to a maximum of 40 characters,

|        and the month should be spelled out for clarity.

|     2) This keyword may only appear once in the file.

|-----------------------------------------------------------------------------

|[Date]          Jan 1, 2000          | The latest file revision date

|

|=============================================================================

| Keyword:        [Source]

| Required:       Yes

| Description:

|     Name of person and or company that created the model

| Usage Context:

|     1) May not appear before keyword [File Rev].  It is recommended that this

|        keyword appear immediately following the keyword [Date].

| Usage Rules:

|     1) It is recommended that a maximum of 24 lines of text be used for

|        the [Source] description.  The end of this section is determined by

|        finding the next keyword in square brackets.

|     2) This keyword may only appear once in the file.

|-----------------------------------------------------------------------------

|[Source] Put originator and the source of information here. For example:

|From silicon level SPICE model at Intel.

|From lab measurement at IEI.

|Compiled from manufacturer's data book at Quad Design, etc.

|

|=============================================================================

| Keyword:        [Notes]

| Required:       No

| Description:

|     Use this section for any special notes related to the file.

| Usage Context:

|     1) May not appear before keyword [File Rev].

|     2) If used; it is recommended that this keyword appear immediately

|        follow the keyword [Source].

| Usage Rules:

|     1) It is recommended that a maximum of 24 lines of text be used for

|        the [Notes] description.  The end of this section is determined by

|        finding the next keyword in square brackets.

|     2) This keyword may appear more than once in the file.

|-----------------------------------------------------------------------------

|[Notes]         Use this section for any special notes related to the file.

|

|=============================================================================

| Keyword:        [Disclaimer]

| Required:       No

| Description:

|     Legal boiler plate here.

| Usage Context:

|     1) If used; it is recommended that this keyword appear shortly after

|        the keyword [Source].

| Usage Rules:

|     1) It is recommended that a maximum of 96 lines of text be used for

|        the [Disclaimer] section. The end of this section is determined

|        by finding the next keyword in square brackets.

|     2) This keyword may only appear once in the file.

|-----------------------------------------------------------------------------

|[Disclaimer]    This information is for modeling purposes only,

|                and is not guaranteed.

|

|=============================================================================

| Keyword:        [Copyright]

| Required:       No

| Description:

|     Who "owns" The model

| Usage Context:

|     1) If used; it is recommended that this keyword appear shortly after

|        the keyword [Source].

| Usage Rules:

|     1) It is recommended that a maximum of 96 lines of text be used for

|        the [Copyright] section. The end of this section is determined

|        by finding the next keyword in square brackets.

|     2) This keyword may only appear once in the file.

|-----------------------------------------------------------------------------

|[Copyright]     Copyright 2000, XYZ Corp., All Rights Reserved

|

|=============================================================================

| Keywords:       [Begin_Cn_Model_Family]  [End_Cn_Model_Family]

| Required:       yes

| Description:

|     The name of the connector family in this file

|     These commands wrap the entire model in this file.

| Usage Context:

|     1) It is recommended that [Begin_Cn_Model_Family] appear shortly after

|        the keyword [Source].

|     2) It is recommended that [End_Cn_Model_Family] be the last line in

|        the file.

| Usage Rules:

|     1) Only a single occurrence of each [Begin_Cn_Model_Family] and

|        [End_Cn_Model_Family] is allowed in the file.

|-----------------------------------------------------------------------------

|[Begin_Cn_Model_Family]

|[End_Cn_Model_Family]

|

|=============================================================================

| Keyword:        [Manufacturer]

| Required:       Yes

| Description:  Clarifies the component's manufacturer.

| Usage Context:

|     1) It is recommended that this keyword appear shortly after

|        the keyword [Begin_Cn_Model_Family].

| Usage Rules:

|     1) The length of the Manufacturer's Name must not exceed 40

|        characters (blank characters are allowed, e.g., Texas

|        Connector Corp.).

| NOTICE: Each manufacturer should use a consistent name in all .icm files.

|-----------------------------------------------------------------------------

|[Manufacturer]  XYZ Corp.

|

|

|=============================================================================

| Keyword:        [Support]

| Required:       No

| Description:

|     A web site that can be visited to get the latest version of the  model.

| Usage Context:

|     1) If used it is recommended that this keyword appear shortly after

|        the keyword [Manufacturer].

| Usage Rules:

|     1) The end of this section is determined by finding the next keyword

|        in square brackets.

|     2) Keyword may appear only once in the file

|     3) If the path is the web site root the .html or .htm extension is not

|        required.

|

| NOTICE:  It is recommended that a separate IBIS model web site (not page)

|     be maintained to prevent accidental changes of page name from breaking

|     this link.

|-----------------------------------------------------------------------------

|[Support] www.VendorName.com\with\a\very\very\very\very\very\very\very\very

|\very\very\very\very\long\path.html            |example with long path

|[Support] www.VendorNameIbisModels.com       |example ibis web site root

|

|

|=============================================================================

| Keyword:        [Redistribution]

| Required:       Yes

| Description:

|     Indicates to CAE tool companies and model users who may

|     use and redistributed this file.

| Usage Context:

|     1) It is recommended that this keyword appear shortly after

|        the keyword [Manufacturer].

| Usage Rules:

|     1) This keyword and associated attribute must be on the same line.

|     2) The attribute must be one of the following keywords:

|

|     YES

|        May be freely distributed as long as no fee is charged.

|        May be freely distributed by CAE tool vendors.

|        May be freely distributed by end users.

|        The model shall be redistributed without changes unless authorized

|        by the creators.

|

|     NO

|        May not be redistributed or retransmitted in

|        any form.  Typically used for connectors

|        created for fee by a service group.

|

|     SPECIFIC

|        See notes field for specific license information.

|        This field gives complete flexibility but prevents

|        automated redistribution.

|NOTICE:

|     The recommended Redistribution is: YES as it

|     allows CAE tool vendors to make models available directly to

|     end users and allows end users to receive models for use

|     immediately when receiving tools from CAE vendors.

|     YES facilitates rapid propagation of a vendors

|     model to end users.

|     The [Support] keyword goes hand-in hand with the

|     YES to make the latest version of the model

|     rapidly available to end users.

|==============================================================================

|[Redistribution]  YES

|

|==============================================================================

| Keyword:        [Begin_Cn_Family_Description]  [End_Cn_Family_Description]

| Required:       Yes

| Description:

|     Provides a concise yet easily human-readable description of

|     this connector family.

| Usage Context:

|     1) It is recommended that the keyword [Begin_Cn_Family_Description] be

|        the first keyword used in the file after the keyword 
|        [Begin_Cn_Model_Family].

|     2) It is required that the keyword [End_Cn_Family_Description] be 
|        the first keyword used after keyword [Begin_Cn_Family_Description].

| Usage Rules:

|     1) Text may continue for up to 4 lines.  Each line must be 120
|        characters or less.

|     2) The end of this section is determined by finding the keyword

|        [End_Cn_Family_Description]

|     3) Only a single occurrence of each [Begin_Cn_Family_Description] and

|        [End_Cn_Family_Description] is allowed in the file.

|     4) Do not put spaces or indent a the beginning of the line as this

|        text will be formatted in text display controls by CAE tools.

|------------------------------------------------------------------------------

| [Begin_Cn_Family_Description]

| High Density 0.1 center square pin connector designed

| for use on IEEE 99999 buses.

| [End_Cn_Family_Description]

|==============================================================================

| Keywords:       [Begin_Cn_Model_List]  [End_Cn_Model_List]

| Required:       yes

|

| Description:

|     For each connector described in this file an entry is provided which

|     names the connector specifies it's mating, maximum slew time

|     (for which the model is valid), and provides a link to a picture

|     of the connector.

|

| Usage Context:

|     1) It is required that [Begin_Cn_Model_List] appear shortly after

|        the keyword [Begin_Cn_Model_Family], but must appear before the

|        keyword [End_Cn_Model_Family].

|     2) It is required that the keyword [End_Cn_Model_List] be the first

|        keyword used after keyword [Begin_Cn_Model_List].

|

| Usage Rules:

|     1) Defines which models are present in the file.

|     2) Mating is required and describes the mating for this model, e.g.

|     3) Mated means both halves of the connector are mated together for

|        the model.

|     4) Valid mating types are:

|        Mated

|        UnMated_Side_A

|        UnMated_Side_B

| NOTES:

| 1: Using two unmated models in series DOES NOT provide a correct model

|    of a mated connector.

| 2: "UnMated_Side_A" MUST BE the back plane side if this applies to this

|    connector.

| 3: Unmated models should be provided for connectors that are

|    likely to be used in backplanes, motherboards, or buses.

| 4: At least one of these "Mating" choices is required

| 5: Min_Slew_Time is required and specifies the minimum slew time

|    (i.e. fastest edge) for any signal using this model.

|    This is a 20% to 80% slew time.

| 6: Image is optional and specifies an associated image file.

|    The image file shall be either a .jpg or .txt (ASCII) formatted picture.

| 7: It is suggested that CAE tools provide a viewer for the .jpg

|    format and that model builders provide at least one picture for

|    a connector.

| 8: The same image file may be repeated for all model types if desired.

| 9: Images if present should be maintained in the same directory with the

|    connector model file or stored in the same compressed file with the

|    connector to insure they files remained paired.

| 10: Suggested .jpg image attributes:

|       size = 160 horizontal by 120 vertical (pixels)

|       color = 24 bit

|       Image quality = 85%

|    Larger or smaller images may be used but are not recommended.

|    A standard image size will allow CAE tools to provide uniform look

|    and feel for images from all vendors.  The size was chosen to be

|    a small disk file size and be useful as either a thumb-nail in a

|    model selection window or scaled up to give a larger picture.

|    The 160x120 was chosen as a perfect 4:1 scale in both axis of

|    a 640x480 picture.

| 12: The connector model MUST be considered valid by CAE tools even if

|     an image file is specified but is missing.

|------------------------------------------------------------------------------

| [Begin_Cn_Model_List]

|        Name                       Mating      Min_Slew_Time 	Image

|-------------------------------------------------- --------  -----------------

|HDI_TEST_202                         Mated          100ps     |HDI_TEST_202_Mated.jpg

|HDI_TEST_202_UnMatedA                UnMated_Side_A 100ps     |HDI_TEST_202_UnMatedA.jpg

|HDI_TEST_202_UnMatedB                UnMated_Side_B 100ps

|HDI_TEST_202_Mated_ThruHole_to_Cable Mated          25ps      |HDI_TEST_202_Mated.jpg

|HDI_TEST_202_Mated_SMT_to_Cable      Mated          25ps      |HDI_TEST_202_Mated.jpg

|HDI_TEST_202_Mated_SMT_to_ThruHole   Mated          25ps      |HDI_TEST_202_Mated.jpg

|[End_Cn_Model_List]

|

|==============================================================================

| Keyword:        [Begin_Cn_Model]  [End_Cn_Model]

| Required:       Yes

| Description:

|     Used to define the start of single or multiple sections for a

|     single connector model as defined in the [Begin_Cn_Model_List].

|     These sections define the R,L,C information making up the actual

|	  simulation data for the connector.

| Usage Context:

|     1) It is required that [Begin_Cn_Model] appear after

|        the keyword [Begin_Cn_Model_List], but must appear before the

|        keyword [End_Cn_Model_Family].

|     2) It is required that the keyword [End_Cn_Model] be the first

|        keyword used after keyword [Begin_Cn_Model].

|     3) This keyword pair may appear more than once in the file.

|     4) Only the following list of keywords are allowed within the

|        context of the [Begin_Cn_Model] and [End_Cn_Model] keywords:

|        *****************************************************************

|			[Begin_Cn_Model_Description], [End_Cn_Model_Description]

|			[Cn_Number_of_Conductors]

|			[Cn_Columns_of_Pins]

|			[Cn_Rows_of_Pins]

|			[Begin_Cn_Swath], [End_Cn_Swath]

|			[Cn_Z]|                   Cn_Section, Cn_Stub

|

| Usage Rules:

|  1) The keyword [Begin_Cn_Model] must be followed by four required

|     parameters in the following order: ModelName, PinMapName,

|     PhyMapName, Type. An optional SGR (signal to ground ratio) parameter

|     may be included if the ModelType is SLM.

|

|     e.g. [Begin_Cn_Model] ModelName ModelPinMap ModelPhyMap ModelType SGR

|  2) The field ModelName is required and must be less than 40 characters.

|     The ModelName MUST appear after the [Begin_Cn_Model] and may optionally

|     appear as a parameter following the [End_Cn_Model] keyword.

|

|  3) The field ModelPinMap is required and must be less than 20 characters.

|     The ModelPinMap MUST appear after the ModelName.  This name must

|     correspond exactly to the name following a [Begin_Cn_Pin_Map]

|     defined in this file.

|

|  4) The field ModelPhyMap is required and must be less than 20 characters.

|     The ModelPhyMap MUST appear after the ModelPinMap. This name must

|     correspond exactly to the name following a [Begin_Cn_Phy_Map]

|     defined in this file.

|

|  5) The field ModelType is required and must be either SLM or MLM

|     SLM indicates this model is a "single-line model" i.e. non-coupled.

|     MLM indicates this model is a "multi-line model" i.e. coupled.

|     SPECIAL NOTICE:

|       If ModelType is SLM then ALL sections MUST be specified using

|       a matrix method that describes only the diagonal of a matrix.

|       For SLM models it is recommended that all sections use only

|       Diagonal_matrix descriptions.

|     If ModelType is SLM then simulators should use this data to

|     create a non-coupled fully distributed transmission line model.

|     It may be also be used to create a multi-section ladder RLC-LCR

|     model as is often used with SPICE simulators.

|     If a ladder network is substituted for

|     a fully distributed transmission line model the simulator vendor should

|     insure enough sections are provided to insure the bandwidth specified

|     by the Min_Slew_Time of this model is achieved.  The symmetric

|     RLC-LCR method should be used as opposed to the non-symmetric

|     RLC-RLC method if a ladder network is used.

|

|     The "SGR" Signal-Ground-Ratio value is optional.  The SGR value

|     is only allowed for use with type SLM.  For example: 3:1 would

|     indicate there is one (power or ground pin) for every 3 signal

|     pins.  No spaces are allowed between the numbers and ":".

|     Valid range of signal number is 1 to 100.  The number after the

|     colon must always be "1".  The number before the colon may include

|     a decimal if desired.  (e.g. 2.5:1)

|     The SGR value is the signal-to-ground ratio that was assumed

|     when the model was created.  The simulator may use this value to

|     provide a warning if the model is being used with a higher or lower SGR.

|     If the simulator provides such a warning then a mechanism should be

|     provided to allow a user defined amount of tolerance.  For example

|     some areas may have 4:1 and others 2:1 with the nominal value being 3:1.

|     It is recommended that the simulator make an average SGR display

|     available to the user if the Model type is SLM.

|

|     MLM indicates this model is a "multi line model" and is

|     a coupled or partially coupled model.

|     Any combination of Diagonal_matrix, Sparse_matrix, Banded_matrix, or

|     Full_matrix may be used in an MLM model.

|     The SGR value MUST NOT be present if the connector is an MLM model.

|

|  6) The model is composed of sections of RLC matrices either in series

|     or as "Tee" sections called stubs.  The following keywords are

|     used for this purpose:

|        Cn_Section  - defines portion of the connectors series sections

|        Cn_Stub     - defines start of a "Tee" section

|

|  7) Cn_Section keyword must be followed by one RLC matrix section name.

|     There must be a section name defined with [Begin_Cn_Section] that

|     matches this name.  Any number of Cn_Sections may be used.

|     Each Cn_Section is connected in series with the previous Cn_Section.

|     The first Cn_Section is the first section connected to the

|     connector pin.

|

|  8) Cn_Stub keyword must be followed by one to 10 RLC matrix section names.

|     There must be a section name defined with [Begin_Cn_Section] that

|     matches each section name.

|     A maximum of 10 RLC matrix sections may be used with a Cn_Stub.

|     The total length of the Cn_Stub line must meet the line length limits

|     defined above.  (Use short names if all 10 stubs are needed)

|     When more then one section matrices appears on the Cn_Stub line the

|     subsequent matrices are connected in series in that "T" section.

|     The last section matrix of a Cn_Stub is open circuit at the far end.

|

|  9) Cn_Stub section matrices MUST each be separated by at least one space

|

|  10) Simulators SHALL automatically replace all pins in an SLM connector

|     used for power or ground with the following values:

|        R = 0.0 ohms,

|        L = 0.0 nH

|        C = 0.0 pF

|     The connector model creator SHALL create an SLM models with the

|     knowledge that the simulators shall follow the above rules.

|

|     For example a 2 row by 10 column pin field connector would

|     have a matrix with 20 rows to define it.  Every diagonal (i.e. pin)

|     could have a corresponding R,L,C value that was created assuming

|     an SGR of say 1:1.  When used by the simulator those that are grounded

|     would be considered as ideal grounds and R,L,C values of 0.0 used.

|     In this way an SLM model could be created that would allow either the

|     top or bottom row to be grounded or every other pin of each row to

|     be grounded and the same SLM model used for all cases.

|

|****************************************************************************

| VALID (straight through series of matrices, no stubs) with example

|

|       SectionA     SectionB    SectionC     SectionD     SectionE

| Pin >----------< >----------< >---------< >----------< >------------< Pin

|

|[Begin_Cn_Model] TestName1 TestName1Pin TestName1Phy SLM 3:1

|Cn_Section SectionA

|Cn_Section SectionB

|Cn_Section SectionC

|Cn_Section SectionD

|Cn_Section SectionE

|[End_Cn_Model] TestName1

|

|****************************************************************************

| VALID (2 series sections with one stub)

|

|       SectionA       SectionB

| Pin >------------< >--------------< Pin

|                   |stub1

|

|[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap MLM

|Cn_Section SectionA

|Cn_Stub stub1

|Cn_Section SectionB

|[End_Cn_Model] MyModel

|

|****************************************************************************

| VALID ( 2 stubs, one stub has 3 series matrices, one stub matrix is re-used)

|        A              B              C

| Pin >----------< >-----------< >-----------< Pin

|                 |stub1        |stub1

|                 |stub2

|                 |stub3

|

|[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap MLM

|Cn_Section A

|Cn_Stub stub1 stub2 stub3

|Cn_Section B

|Cn_Stub stub1

|Cn_Section C

|[End_Cn_Model] MyModel

|

|****************************************************************************

| VALID

|         A        B        C        D

| Pin >------< >------< >------< >-------< Pin

|             |Stub1   |Stub2   |Stub1

|                      |Stub3

|                      |Stub3

|

|[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap MLM

|Cn_Section A

|Cn_Stub Stub1

|Cn_Section B

|Cn_Stub Stub2 Stub3 Stub4

|Cn_Section C

|Cn_Stub Stub1

|Cn_Section D

|[End_Cn_Model] MyModel

|

|****************************************************************************

| VALID (has a parallel stubs)

|

|       A      |stub1  Bee       C

| Pin >-------< >----------< >-------< Pin

|              |stub2

|              |stub3

|

|** BAD EXAMPLE **

|[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap MLM

|Cn_Section A

|Cn_Stub Stub1

|Cn_Stub Stub2 Stub3

|Cn_Section Bee

|Cn_Section C

|[End_Cn_Model]

|

|****************************************************************************

| *** NOT VALID **** (more than 10 series stubs)

|

|             |StubA          C

| Pin >------< >-------< >---------< Pin

|         A        B    |Stub1

|                       |Stub2

|                       |Stub3

|                       |Stub4

|                       |Stub5

|                       |Stub6

|                       |Stub7

|                       |Stub8

|                       |Stub9

|                       |Stub10

|                       |Stub11

|** BAD EXAMPLE **

|[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap MLM

|Cn_Section A

|Cn_Stub StubA

|Cn_Section B

|Cn_Stub Stub1 Stub2 Stub3 Stub4 Stub5 Stub6 Stub7 Stub8 Stub9 Stub10 Stub11

|Cn_Section C

|[End_Cn_Model] MyModel

|

|------------------------------------------------------------------------------

|

[Begin_Cn_Model] MyModel MyModelPinMap MyModelPhyMap SLM 3:1

Cn_Section Diagonal_matrix1

[End_Cn_Model] MyModel

|

| Notice the signal to ground ratio in the SLM example above was specified

| as 3 to 1

|

|==============================================================================

| Keyword:        [Begin_Cn_Model_Description]  [End_Cn_Model_Description]

| Required:       No

| Description:

|     Provides a concise yet easily human-readable description of

|     this connector model

|

| Usage Context:

|     1) This keyword pair MUST BE USED BETWEEN KEYWORDS

|        [Begin_Cn_Model] and [End_Cn_Model].

|     2) The keyword [End_Cn_Model_Description] must be the first keyword

|        following the keyword [Begin_Cn_Model_Description].

|

| Usage Rules:

|     1) Text may continue for up to 4 lines.

|     2) The end of this section is determined by finding the keyword

|        [End_Cn_Model_Description]

|     3) This keyword may appear multiple times in the file however it may

|        appear only once between each pair of [Begin_Cn_Model]/[End_Cn_Model]

|        keywords.

|     4) Do not put spaces or indent a the beginning of the line as this

|        text will be formatted in text display controls by CAE tools.

|------------------------------------------------------------------------------

| [Begin_Cn_Model_Description]

| High Density 0.1 center square pin connector designed

| for use on EIA RS99999Example bus.

| [End_Cn_Model_Description]

|

|

|==============================================================================

| Keyword:        [Cn_Number_of_Conductors]

| Required:       Yes

| Description:

|     Number of current carrying conductors used in the model.

|     It is assumed that all matrices in for the associated connector
|     model will have the SAME number of indices.

|     This also defines the size of the expected Matrix(s) unless the

|     optional Begin_Cn_Swath keyword is used.

|

| Usage Context:

|     1) This keyword MUST BE USED BETWEEN KEYWORDS

|        [Begin_Cn_Model] and [End_Cn_Model].

|     2) It is required that this keyword be the 2nd keyword

|        following the keyword [Begin_Cn_Model] (excluding the

|        the keywords [Begin_Cn_Model_Description] and

|        [End_Cn_Model_Description]).

|

| Usage Rules:

|     1) If the [Begin_Cn_Pin_Map] keyword is in use in this model then a

|        single numeric value from 1 to 100000 is recommended. Larger values

|        shall create warnings by the parser.  Smaller values shall create

|        errors by the parser.

|     2) If the [Begin_Cn_Auto_Map] keyword is in use in this model then

|        a range MUST be specified from 1 to 100000 is recommended. Larger

|        values shall create warnings by the parser.  Smaller values shall

|        create errors by the parser.

|     3) If the [Begin_Cn_Auto_Map] keyword is also used for this connector

|        model than a range MUST be specified as a minimum values to

|        a maximum value (see example below).  If the [Begin_Cn_Auto_Map]
|        is not used than a single fixed number of conductors must be

|        specified.

|==============================================================================

|Fixed sized connector example:

|[Cn_Number_of_Conductors]  8

|

|Variable sized connector example:

|[Cn_Number_of_Conductors] 9 to 25

| The range of 9 to 25 indicates the connector may 
| have any number of pins with a minimum size of

| 9 pins and a maximum size of 25 pins.

|==============================================================================

| Keyword:        [Cn_Columns_of_Pins]

| Required:       Yes

| Description:

|     The number of columns of pins

|     For a non-rectangular connectors the maximum number of pins

|     "across" the connector should be specified.

|

| Usage Context:

|     1) This keyword pair MUST BE USED BETWEEN KEYWORDS

|        [Begin_Cn_Model] and [End_Cn_Model].

|     2) It is required that this keyword be the 3rd or 4th keyword

|        following the keyword [Begin_Cn_Model] (excluding the

|        the keywords [Begin_Cn_Model_Description] and

|        [End_Cn_Model_Description]).

|        That is it MUST BE specified after the keyword

|        [Cn_Number_of_Conductors] for this model.

|

| Usage Rules:

|     -If the keyword [Begin_Cn_Auto_Map] is present in this connector

|      model than the keyword VARIABLE MUST be used to indicate a variable

|      width connector.

|      Else a numeric value from 1 to 100000  is recommended. Larger values

|      shall create warnings by the parser.  Smaller values shall create

|      errors by the parser.

|     -If the keyword VARIABLE is used it must be all upper case.

|==============================================================================

|For fixed sized connector models use a single numeric value as shown below

[Cn_Columns_of_Pins]  4

|

|For variable size connector models using the [Begin_Cn_Auto_Map] keyword

| the special keyword VARIABLE may be used to indicate this axis is variable

| width.

|[Cn_Columns_of_Pins] VARIABLE

|==============================================================================

| Keyword:        [Cn_Rows_of_Pins]

| Required:       Yes

| Description:

|     The number of rows of pins.

|     For a non-rectangular connectors the maximum number of pins

|     "down" the connector should be specified.

|

| Usage Context:

|     1) This keyword pair MUST BE USED BETWEEN KEYWORDS

|        [Begin_Cn_Model] and [End_Cn_Model].

|     2) It is required that this keyword be the 3rd or 4th keyword

|        following the keyword [Begin_Cn_Model] (excluding the

|        the keywords [Begin_Cn_Model_Description] and

|        [End_Cn_Model_Description]).

|        That is it MUST BE specified after the keyword

|        [Cn_Number_of_Conductors] for this model.

|

| Usage Rules:

|     1) A numeric value is required following the keyword to specify

|        the number of rows this connector model requires.

|        A numeric value from 1 to 100000  is recommended. Larger values

|        shall create warnings by the parser.  Smaller values shall create

|        errors by the parser.

|==============================================================================

[Cn_Rows_of_Pins]  2

|

|********** Variable size connectors **************************************

| Keywords:       [Begin_Cn_Auto_Map]  [End_Cn_Auto_Map]

| Required:       If the [Begin_Cn_Pin_Map] keyword is NOT present
|                 than this keyword is required
|                 else it shall not be used.

| Description:

|   Using the [Begin_Cn_Auto_Map] keyword indicates that this connector is

|   variable size and will be automatically created by a CAE tool such as

|   a simulator using the algorithm defined below.

|

|   This keyword allows a tool like a simulator to create the pin map

|   and connector map automatically for a connector by first determining

|   the total number of pins need for the particular schematic/PCB layout

|   or requested from the user interface and then executing the algorithm

|   until the desired number of pins is created.

|

|Notes:

| 1) Variable map creation should only be used for connectors that

|    are non-industry standard pin outs.  For example an ISA, SCSI, or

|    PCI bus connector should not use the [Begin_Cn_Auto_Map]

|    keyword as the signal names would not match generally defined signal

|    names.  It may be desired to create multiple models for some connectors

|    for example: The model for a square pin connector might have two

|    industry standard pin outs like 34 pin floppy and 50 pin SCSI and

|    also a generic 8 to 100 pin model.  The Generic version could use

|    the variable size model and the 34 and 50 pin connectors would use

|    fixed size models.

|

| 2) Variable width connectors may be created with this method.

|    Variable height connectors cannot be created with this method.

|

| 3) Signal names are automatically generated and may not be useful for

|    identifying nets.  The CAE tools should give priority to identifying

|    connectivity with pin names if this keyword is used.

|

| 4) This is not a free form equation.  The syntax must be followed exactly.

|    The result must produce sequential index numbers starting at 1 or

|    the model is not valid.

|

| 5) An internal “Pin Map” is created using the specified algorithm.

|    The number of pins in the “Pin Map” depends on the number of
|    pins needed for a particular design file’s usage e.g. a board
|    design needs a 50 pin header connector and the [Begin_Cn_Auto_Map] 
|    feature allows 10 to 100 pin connector models to be created.  In

|    this case the algorithm would be followed to create the 50 pin

|    connector model.

|

| 6) An internal “Physical Map” is create using the specified algorithm
|    and an implicit ordering of the index numbers that starts at the

|    physical top left of the connector and goes down the physical left

|    most column than starts at the top of the next column to the right

|    and continues until the request number of pins are assigned physical

|    locations.  This way the lowest index number pins are assigned to

|    the left most physical column and the highest index number pins 
|    are assigned to the right most physical column.

|

|

| Usage Context:

|	1) MUST BE USED between keywords [Begin_Cn_Model] and [End_Cn_Model].

|	2) MUST BE specified after [Cn_Number_of_Conductors], [Cn_Rows_of_Pins],

|      and [Cn_Columns_of_Pins] have been specified for this connector model.

|

|Usage:

|	1) The keyword [Begin_Cn_Auto_Map] replaces both the 
|         [Begin_Cn_Pin_Map] and the [Begin_Cn_Phy_Map] keywords.  These
|         other keywords are not valid in the same model as the
|         [Begin_Cn_Auto_Map] keyword.

|	2) If the keyword [Begin_Cn_Auto_Map] is used than the connector
|         model must be defined using the [Begin_Cn_Swath] keyword.

|

|Algorithm Form:

| -Variable pin mapping MUST follow one of the following algorithm forms:

|      $$(i*n+m) OR $$(LASTCOL*n+i+m)

| where:

|   -Algorithm formulas must be enclosed in right and left parenthesis.

|   -One to 8 characters may optionally precede the algorithm formulas

|    in the $$ positions shown above.  Spaces are not allowed between the

|    characters and the formula left parenthesis.

|   -Two variables are provided i and LASTCOL.  These are case sensitive

|    and must appear exactly as shown.

|   -The variable "i" and constant values n and m are required in all

|    algorithms between the left and right parenthesis.

|    [Begin_Cn_Auto_Map] and [End_Cn_Auto_Map] and may not be substituted

|    with any other character.

|   -The variable "i" always begins at 0 and increments by 1 until

|    all pins are defined.

|   -The value n should be any unsigned integer from 1 to 100000.  Smaller

|    integer numbers cause parser errors.  Larger numbers cause parser

|    warnings.

|   -The value m should be any unsigned integer from 1 to 99999.  Smaller

|    integer numbers cause parser errors.  Larger numbers cause parser

|    warnings.

|   -If present the LASTCOL variable must be entered as an upper case keyword.

|   -The constant LASTCOL is computed internally by the CAE tool by taking

|    the desired number of pins for this connector divided by the number

|    of rows.

|   -All Index's,  Pin names, and signal names created with algorithm

|    specified in the model must be result in unique identifiers or

|    the model is invalid. (i.e. all algorithms must be unique)

|   -The number of pins in the connector must be an EXACT MULTIPLE of the

|    number of row equation lines defined between [Begin_Cn_Auto_Map]

|    and [End_Cn_Auto_Map].  If not this model should not be created.

|   -The number of algorithm lines must EXACTLY match the number rows

|    defined for this connector in [Cn_Rows_of_Pins].  If not this model

|    has an error and should not be created.

|   -The number of pins created must be greater than or equal to the

|    minimum number of pins specified and less than or equal to the

|    maximum number of pins specified in the [Cn_Number_of_Conductors]

|    keyword line.

|   -One or more characters may optionally precede the algorithm formulas

|    however spaces are not allowed between the characters and the formula.

|   -Only multiply "*" and add "+" characters are allowed for math operators.

|    other operators shall be considered errors.

|   -All three fields index, pin, and signal fields must be defined.

|   -signal names may also be defined by either algorithm or using one of the

|    fixed reserved names:

|        PWRGND- reserved model name, used with power or ground pins,

|        POWER - reserved model name, used with power supply pins,

|        RET   - reserved word indicating either power or ground used

|        GND   - reserved model name, used with ground pins,

|        NC    - reserved model name, used with no-connect pins,

|        NA    - used where data not available.

|

|***** an alphanumeric signal name example of 9 pin connector ******

|[Cn_Number_of_Conductors]  9 to 300

|[Cn_Columns_of_Pins]       VARIABLE

|[Cn_Rows_of_Pins]          3

|[Begin_Cn_Auto_Map]

|Index   Pin      Signal

|(i*3+1)  A(i+1)    S(i*3+1)

|(i*3+2)  B(i+1)    S(i*3+2)

|(i*3+3)  C(i+1)    S(i*3+3)

|[End_Cn_Auto_Map]

|

|If the previous keyword values and algorithms are defined and

| the CAE tool or user requested a model for a 9 pin connector the

| following would be created automatically:

|Index Pin    Signal

|1    A1    S1

|2    B1    S2

|3    C1    S3

|4    A2    S4

|5    B2    S5

|6    C2    S6

|7    A3    S7

|8    B3    S8

|9    C3    S9

|

|connector physical map

|A1 A2 A3

|B1 B2 B3

|C1 C2 C3

|

|***** a numeric pin and alphanumeric signal name example of 12 pin ******

|[Cn_Number_of_Conductors]  9 to 300

|[Cn_Columns_of_Pins]       VARIABLE

|[Cn_Rows_of_Pins]          3

|[Begin_Cn_Auto_Map]

|note: LASTCOL will be calculated as 12/3 = 4

|Index    Pin            Signal

|(i*3+1)  (i+1)           Xsig(i+1)

|(i*3+2)  (LASTCOL+i+1)   Y(LASTCOL+i+1)

|(i*3+3)  (LASTCOL*2+i+1) Z(LASTCOL*2+i+1)

|[End_Cn_Auto_Map]

|

|If the previous keyword values and algorithms are defined and

| the CAE tool or user requested a model for a 12 pin connector the

| following would be created automatically:

|Index Pin  Signal

|1    1     Xsig1

|2    5     Y5

|3    9     Z9

|4    2     Xsig2

|5    6     Y6

|6    10    Z10

|7    3     Xsig3

|8    7     Y7

|9    11    Z11

|10   4     Xsig4

|11   8     Y8

|12   12    Z12

|

|connector physical map

|1 2  3  4

|5 6  7  8

|9 10 11 12

|

|***** an alphanumeric pin, alphanumeric signal name example of 12 pin *****

|[Cn_Number_of_Conductors]  8 to 100

|[Cn_Columns_of_Pins]       VARIABLE

|[Cn_Rows_of_Pins]          2

|[Begin_Cn_Auto_Map]

|note: LASTCOL will be calculated as 12/2 = 6

|Index    Pin            Signal

|(i*2+1)  A(i+1)           sig(i+1)

|(i*2+2)  B(LASTCOL+i+1)   GND

|[End_Cn_Auto_Map]

|

|If the previous keyword values and algorithms are defined and

| the CAE tool or user requested a model for a 12 pin connector the

| following would be created automatically:

|Index Pin  Signal

|1    1     sig1

|2    7     GND

|3    2     sig2

|4    8     GND

|5    3     sig3

|6    9     GND

|7    4     sig4

|8    10    GND

|9    5     sig5

|10   11    GND

|11   6     sig6

|12   12    GND

|

|connector physical map

|1  2  3  4  5  6

|7  8  9  10 11 12

|==============================================================================

| Keywords:       [Begin_Cn_Swath]  [End_Cn_Swath]

| Required:       No, Yes if [Begin_Cn_Auto_Map] is used

| Description:

|    Describes a "swath" which is a smaller matrix used to

|    simulate connectors of equal or larger size.  To visualize this

|    consider the swath matrix being a smaller version of the connector.  It’s

|    edges show the same edge effects as the full size connector has.

|    It’s center area shows the same effects the center of the full size

|    connector has.  It is the responsibility of the simulator to expand
|    this “mini” connector to model a larger connector.

|

|    For example a 2x8 matrix might be used to model a connector

|    that is 2x100.  The "swath" can also be combined with the

|    [Begin_Cn_Auto_Map] keyword to create multiple connector models from

|    a single description.  For example a 2x10 matrix might be used

|    to model connectors ranging from 2x10 to 2x200 with a single

|    descriptive model.

|

| Definition:  "EDGE ONLY"

|    For reference in this document the phrase "EDGE ONLY" is used to

|    reference rows or columns from the swath matrix that are used

|    only at the indicated edge of a connector. Such rows or columns

|    should instead be connected through a resistor to 0.0 volts.

|    The resistors shall have a value of resistance equal to the value

|    defined with the [Cn_Z] keyword.  This load will provide

|    the matrix with an approximately feel of neighbor pins and circuit

|    loading to approximate a much larger connector matrix.

|

| Note: The keyword [Begin_Cn_Phy_Map] is required if the [Begin_Cn_Swath]

|       keyword is used.  The [Begin_Cn_Phy_Map] keyword provides the

|       information needed by the simulator to apply the matrix data

|       to the physical connector.

|

|

| Usage Context:

|   1) MUST BE USED between keywords [Begin_Cn_Model] and [End_Cn_Model].

|   2) MUST BE specified after [Cn_Number_of_Conductors], [Cn_Rows_of_Pins],

|      and [Cn_Columns_of_Pins] have been specified for this connector model.

|   3) MUST be followed by 6 numbers in order as follows:

|      Rows in the swath matrix

|      Columns in the swath matrix

|      EDGE ONLY columns in the swath matrix used to represent right edge

|      EDGE ONLY columns in the swath matrix used to represent left edge

|      EDGE ONLY rows in the swath matrix used to represent top edge

|      EDGE ONLY rows in the swath matrix used to represent bottom edge

|   4) All matrices used to represent a connector model with swath must
|      have exactly (SwathRows * SwathCols) of indices.

|

| Usage:

|   1) Only one [Begin_Cn_Swath] shall be used per connector model.

|   2) MUST include exactly one [Cn_Z] definition line.

|------------------------------------------------------------------------------

|==============================================================================

|                  S W A T H      E X A M P L E

|==============================================================================

|                SwathRows SwathCols RightEdge LeftEdge TopEdge BottomEdge

| [Begin_Cn_Swath]     2         8        1          1       0        0

| [Cn_Z] 50

| [End_Cn_Swath]

|

|

|==============================================================================

| Keywords:       [Cn_Z]

| Required:       If the [Begin_Cn_Swath] keyword is present than this

|                 keyword is required, else this keyword shall not be used.

| Description:

|     The impedance value defined by [Cn_Z] is used as a load by the simulator.

|     This load must be connected within the simulation to portions of the

|     matrix defined as "NO_SIM" locations (see below) to insure proper

|     electric loading of the simulation matrix.

|

| Usage Context:

|   1) MUST BE USED between keywords [Begin_Cn_Swath] and [End_Cn_Swath].

|

| Usage:

|   1) The impedance value following [Cn_Z] must be specified in ohms.

|------------------------------------------------------------------------------

|Example:

|[Cn_Z] 50

|

|

|==============================================================================

| Keywords:       [Begin_Cn_Pin_Map], [End_Cn_Pin_Map]

| Required:       If the [Begin_Cn_Auto_Map] keyword is NOT present
|                 than this keyword is required
|                 else it shall not be used.

| Description:

|     Describes the index into the matrix for which a given pin applies.

|

| Usage Context:

|    1) This keyword pair MUST be placed outside the keyword pair

|       [Begin_Cn_Model]/[End_Cn_Model] keywords to allow it

|       to be reused. if desired.

|    2) This keyword pair MUST appear between the keyword pair

|       [Begin_Cn_Family_Description]  [End_Cn_Family_Description]

|

| Usage Rules:

|      1) The keyword [Begin_Cn_Pin_Map] must be followed by one required

|         parameter which is the ModelPinMapName.

|         e.g. [Begin_Cn_Pin_Map] ModelPinMapName

|

|	2) The field ModelPinMapName is required and must be less than

|	   20 characters. The ModelPinMapName MUST appear after the

|	   [Begin_Cn_Pin_Map] and may optionally appear as a parameter

|	   following the [End_Cn_Pin_Map] keyword.   This name is used

|	   by the [Begin_Cn_Model] keyword to reference a pin map.

|

|	3) More than one [Begin_Cn_Model] record may reference the same

|	   pin map.

|

|	4) The keyword NA indicates that there is not a pin in the

|	   location (e.g. PGA socket with corner pin missing)

|	   An end of line character indicates the end of the information for

|	   the row

|

|	5) The lines between [Begin_Cn_Pin_Map] and [Begin_Cn_Pin_Map] shall

|         contain one line for each pin in this connector model.  The pin

|         map line shall contain the following parameters.

|	   PIN TO INDEX MAPPING

|	   Index  (required)

|         Is the index into the matrix to associate with the pin.

|         This must be a number between 1 and 100000

|         Where "Index" refers to the matrix diagonal

|         (i.e. Index is represented by C11, | L11, and R11)

|         Pin    (required)

|         Is a pin name specified by the manufacture

|         This allows CAE tools to align pin names in the model

|         with pin names in the net list.

|         Signal (Optional, based on application but recommended if applicable)

|         Is the signal name used; required if this is an industry-

|         standard connector like RS232, PCI, EISA etc.

|         NOTE: NA should be used to indicate this field is not available

|         This can be used by CAE tools or by users interactively

|         to easily identify the pins of the connector.

|         This is an alternate method for CAE tools to align signal  names

|         in the model with net names from the design's net list if the

|         pin names cannot be aligned.

|         Maximum of 20 characters allowed for Pin Name

|         Maximum of 20 characters allowed for Signal Name

|         Pin or Signal names longer than 20 should be truncated to 20 to fit

|         this specification.

|

|      6) It is acceptable to have more than one pin map in a file.  
|         Therefore the keywords Begin_Cn_Pin_Map and End_Cn_Pin_Map
|         may appears as a pair more than once in a file.

|

|         Simulators should consider pin names with case in-sensitive compares.

|         To make human comparison easy it is recommended that all uses of the

|         pin name in the file utilize the same case.

|

|NOTICE: The special Signal names POWER, RET, GND, and PWRGND shall be used to

|     indicate that this pin should be connected to either a Power pin like

|     +5V, a ground pin, or (either a power or ground pin).  This information

|     may be used by the simulator or PCB layout tool to provided a warning

|     that the connector is not following an established industry standard

|     layout.  The keyword RET is available to indicate it is a return pin

|     and could be either power or ground.

|==============================================================================

| [Begin_Cn_Pin_Map] 20PinHdiPin

| Index Pin    Signal

| 1      A1     GND

| 2      A2     D0

| 3      A3     CLK

| 4      A4     D1

| 5      B1     STROBE1

| 7      B2     A0

| 8      B3     POWER

| 9      B4     PWRGND

| [End_Cn_Pin_Map]

|

|==============================================================================

| Keywords:       [Begin_Cn_Phy_Map], [End_Cn_Phy_Map]

| Required:       If the [Begin_Cn_Auto_Map] keyword is NOT present
|                 than this keyword is required
|                 else it shall not be used.

|

| Description:

|     Describes the physical layout of the connector for use with

|     Swath method or general documentation.

|     [PhyRow] x

|        x= 1, 2, 3, ...   One row for each conductor required

|     Layout connector rows / col  -  kind of a refinement to the

|     "Column" and "Row" Definitions above.   ASCII Pictorial Description

|     This has two purposes:
|      1) When swath matrices are used the physical map provides

|         the information about how to map the swath matrix into the physical

|         connector.

|      2) When swath is NOT used then the matrix size shall match
|         the connector size.  In this case the physical map is not

|         needed to determine interconnect with between the pins and the
|         matrix but it may still serve as a checking mechanism to insure that
|         the two sides of the connector are properly mechanically mated.

|         This feature helps to insure that pins are not swapped between two

|         uses of the connector (i.e. side A and side B do actually align
|         physically as well as in the net list).

|

| Usage Context:

|  1) This keyword MUST be placed outside the [Begin_Cn_Model]/

|     [End_Cn_Model] keywords to allow it to be reused. if desired.

|

| Usage Rules:

|  1) The keyword [Begin_Cn_Phy_Map] must be followed by one required

|     parameter which is the ModelPhyMapName.

|

|     e.g. [Begin_Cn_Phy_Map] ModelPhyMapName

|

|  2) The field ModelPhyMapName is required and must be less than

|     20 characters. The ModelPhyMapName MUST appear after the

|     [Begin_Cn_Phy_Map] and may optionally appear as a parameter

|     following the [End_Cn_Phy_Map] keyword.   This name is used

|     by the [Begin_Cn_Model] keyword to reference a physical map.

|

|  3) More than one [Begin_Cn_Model] record may reference the same

|     physical map.

|

|  4) Pin names MUST match the names used in the [Begin_Cn_Pin_Map]

|     section exactly.

|

|  5) All pin names specified in [Begin_Cn_Pin_Map] must also be specified

|     exactly the same in [Begin_Cn_Phy_Map] that used by the same

|     [Begin_Cn_Model].

|

|  6) Simulators should consider pin names with case in-sensitive compares.

|     To make human comparison easy it is recommended that all uses of the

|     pin name in the file utilize the same case.

|

|  7) It is acceptable to have more than one physical map in a file.  Therefore

|     the keywords Begin_Cn_Phy_Map and End_Cn_Phy_Map may appears as a pair

|     more than once in a file.

|==============================================================================

|[Begin_Cn_Phy_Map] 20PinHdiPhy

|[PhyRow] 1

|A1 A2 A3 A4 A5 A6 A7 A8 A9 A10

|[PhyRow] 2

|B1 B2 B3 B4 B5 B6 B7 B8 B9 B10

|[End_Cn_Phy_Map]

|

|==============================================================================

| Keyword:        [Begin_Cn_Section], [End_Cn_Section]

| Required:       No

| Description:

|  Indicates the beginning of a set of R,L,C matrix data.

| Sub parameters:

|     A matrix name must follow both the [Begin_Cn_Section] and

|     [End_Cn_Section] keywords.

|

| Usage Context:

|  -MUST be placed outside the context of keywords

|   [Begin_Cn_Model] and [End_Cn_Model] as sections may be reused

|   by more than one connector model.

|  -MUST be placed inside the context of the family

|  -The following keywords are required:

|     [Inductance Matrix], [Capacitance Matrix]

|  -The following keywords are optional:

|     [Resistance Matrix], [Bandwidth], [Row]

|

| Usage:

|  -Each matrix used for given connector model MUST have exactly

|   the same number conductors.

|  -If the swath method is used the matrix may have fewer conductors

|   then the connector model.

|  -If the swath method is not used the matrix MUST have exactly the

|   same number of conductors as the connector has pins.

|  -The matrices used for the R,L,C description can be defined using

|   any of the matrix description methods allowed in the specification.

|   The matrix description methods can also be mixed within a given section.

|   For example the resistance matrix might use the Diagonal_matrix and the

|   inductance matrix might use a Full_matrix.

|------------------------------------------------------------------------------

| See example "Matrix Example" lower in this document.

|

|=============================================================================

|=============================================================================

|

|                                 Section 7

|

|                          M A T R I X    D A T A

|

|=============================================================================

|=============================================================================

|==============================================================================

| Keywords:    [Resistance Matrix], [Inductance Matrix], [Capacitance Matrix]

| Required:    Yes

|              If a simple single line model is defined then the following

|              matrices are required:

|                 case 1: [Resistance Matrix] only

|                               OR

|                 case 2: [Inductance Matrix] and [Capacitance Matrix] only

|                         for which the [Resistance Matrix] is assumed to

|						  be zero ohms

|                               OR

|                 case 3: [Resistance Matrix] and [Inductance Matrix]

|                         and [Capacitance Matrix]

|              NOTE: In case 1,2,3 the matrices must be Diagonal_matrix.

|

|              If a coupled model is defined then the following matrices are

|              required:

|                 case 4: [Inductance Matrix] and [Capacitance Matrix] only

|                         for which the [Resistance Matrix] is assumed to

|						  be zero ohms

|                               OR

|                 case 5: [Resistance Matrix] and [Inductance Matrix]

|                         and [Capacitance Matrix]

|              NOTE: In case 4,5 the matrices shall be any combination of

|              matrix types (Diagonal, Banded, Sparse, Full).

|

| Description:

|     The sub-parameters mark the beginning of a matrix, and

|     specify how the matrix data is formatted.

|

| Sub-Params:  Diagonal_matrix, Banded_matrix, Sparse_matrix, or Full_matrix

|

|

| Usage Context:

|     1) MUST BE USED between keywords [Begin_Cn_Section] and [End_Cn_Section].

|

| Usage Rules:

|     1) For each matrix keyword, use only one of the sub-parameters.

|     2) After each of these sub-parameters, insert the matrix data in

|        the appropriate format.  (These formats are described in detail

|        below.)

|

| Notes:

|     Numeric Accuracy:

|     The numeric accuracy used in the file should not be significantly

|     greater than the known accuracy of the model.

|     Refer to usage rule 13 (top of document) for more details.

|

|     Currents and Sign Conventions:

|     The resistance, inductance, and capacitance matrices may also

|     be referred to as "RLC matrices" within this specification.

|     When measuring the entries of the RLC matrices, either with

|     laboratory equipment or field-solver software, currents are

|     defined as ENTERING the pins of the package from the board

|     (General Syntax Rule #11).  The corresponding voltage drops are

|     to be measured with the current pointing "in" to the "+" sign

|     and "out" of the "-" sign.

|

|                         I1   +-----+    I2

|                       -----> |     | <------

|               board o--------| Pkg |---------o board

|                      +  V1 - |     | -  V2  +

|                              +-----+

|

|     It is important to observe this convention in order to get the

|     correct signs for the mutual inductance's and resistances.

|

|     For each [Resistance Matrix], [Inductance Matrix], or [Capacitance

|     Matrix] a different format can be used for the data.  The choice of

|     formats is provided to satisfy different simulation accuracy and speed

|     requirements.  Also, there are many packages in which the resistance

|     matrix can have no coupling terms at all.  In this case, the most

|     concise format (Banded_matrix) can be used.

|

|    **************************************************************************

|    *                    RESISTANCE MATRIX DESCRIPTION                       *

|    *                                                                        *

|    * UNITS:       OHMS                                                      *

|    *                                                                        *

|    * DESCRIPTION:                                                           *

|    *   The resistance matrix represents the total DC resistance between     *

|    *   the two ends of this section of the connector.                       *

|    * NOTES:                                                                 *

|    *   1) The resistance matrix is optional.  Simulation tools shall        *

|    *      use 0.0 ohms for all entries if this matrix is not present.       *

|    *   2) DO NOT USE ohms/meter as this is the total value and includes     *

|    *      the length of this section.                                       *

|    *   3) Values should all appear in the diagonal terms of the matrix.     *

|    *      For example: value 1,1 is the resistance for pin 1 and            *

|    *      value 2,2 is the resistance for pin 2.                            *

|    *   4) All off diagonal terms should 0.0 if entered however all          *

|    *      simulators shall ignore any off diagonal terms present.           *

|    *   5) A "Diagonal_matrix" matrix type is the best choice for            *

|    *      describing the resistance matrix.                                 *

|    *      (see matrix  descriptions below).                                 *

|    *                                                                        *

|    **************************************************************************

|    *                    INDUCTANCE MATRIX DESCRIPTION                       *

|    *                                                                        *

|    * UNITS:      HENRIES                                                    *

|    *                                                                        *

|    * DESCRIPTION:                                                           *

|    * Two inductance matrix methods are supported:                           *

|    *                                                                        *

|    *       SLM, Single-Line, Loop-Inductance Matrix    (non-coupled)        *

|    *       MLM, Multi-Line,  Partial-Inductance Matrix (coupled)            *

|    *                                                                        *

|    * If ModelType is SLM then all sections must use single-line             *

|    * loop-inductance and capacitance matrices.                              *

|    *                                                                        *

|    * If ModelType is MLM then all sections must use                         *

|    * partial-inductance and capacitance matrices.                           *

|    *                                                                        *

|    * Both capacitance and inductance matrix of a given section must use     *

|    * the same method.                                                       *

|    *                                                                        *

|    *	Single-Line Loop-Inductance Matrix:                                   *

|    * -----------------------------------                                    *

|    * DESCRIPTION:                                                           *

|    * To define inductance requires a loop.  The loop consists of a closed   *

|    * path including a signal and all possible return paths.  The inductance *

|    * is a coefficient between the emf induced in the loop and the change of *

|    * magnetic flux through the loop.                                        *

|    *                                                                        *

|    *                    connector or section                                *

|    *                ____________________________                            *

|    *               |                            |                           *

|    *             Return .....................  Return                       *

|    *    (side A)   |                            |       (side B)            *

|    *             Signal .....................  Signal                       *

|    *               |                            |                           *

|    *             Return .....................  Return                       *

|    *               |                            |                           *

|    *                ----------------------------                            *

|    * The return path must include all pins that are power or ground.        *

|    * In a loop inductance matrix, the diagonal terms represent the loop     *

|    * inductance of a specific pin with return currents assumed returned     *

|    * through all pins defined to be power or ground.                        *

|    *                                                                        *

|    * NOTES (Single-Line Loop-Inductance Matrix):                            *

|    * 1) This method must be used if the ModelType is SLM.                   *

|    * 2) Loop inductance can be experimentally measured.                     *

|    * 3) Loop inductance matrices can be created using fieldsolvers.         *

|    * 4) An SGR value is normally specified for an SLM model that indicates  *

|    *    the ratio of signal pins to return path pins (i.e. power or ground) *

|    *    pins.                                                               *

|    * 5) SLM models make have zero values for power or ground R,L,C values   *

|    *    which prevents there use in analyzing connector ground bounce       *

|    *    effects.                                                            *

|    * 6) Typical uses for SLM model might include for example a 2x17 header  *

|    *    connector where the bottom row is all grounded.  The SGR would be   *

|    *    specified in this case as 1:1.  A model would be developed with     *

|    *    these assumptions made.  Then all pins would be filled with average *

|    *    values.  To further expand this example see the figure below:       *

|    *                                                                        *

|    *    Pin out:                                                            *

|    *             1  ................. 17                                    *

|    *             18 ................. 34                                    *

|    *                                                                        *

|    *    Measurement or Fieldsolver connection:                              *

|    *             Sig Sig Sig ... Sig Sig                                    *

|    *             Gnd Gnd Gnd ... Gnd Gnd                                    *

|    *                                                                        *

|    *    Measured or Computed values:                                        *

|    *             3nH 4nH 4nH ....4nH 3nH                                    *

|    *                                                                        *

|    *    Matrix data:                                                        *

|    *             3nH 4nH 4nH ....4nH 3nH                                    *

|    *             3nH 4nH 4nH ....4nH 3nH                                    *

|    *                                                                        *

|    *    This allows the user to specify any of the following power/ground   *

|    *    connections and still use this model:                               *

|    *    Simulator Connection A:                                             *

|    *             Sig Sig Sig ... Sig Sig                                    *

|    *             Gnd Gnd Gnd ... Gnd Gnd                                    *

|    *                                                                        *

|    *    Simulator Connection B:                                             *

|    *             Gnd Gnd Gnd ... Gnd Gnd                                    *

|    *             Sig Sig Sig ... Sig Sig                                    *

|    *                                                                        *

|    *    Simulator Connection C:                                             *

|    *             Sig +5V Sig ... Gnd Sig                                    *

|    *             Gnd Sig Gnd ... Sig +5V                                    *

|    *                                                                        *

|    *    Given the following connection a simulation should generate a       *

|    *    warning to the user indicate the ground-signal-ratio has been       *

|    *    violated.                                                           *

|    *    Simulator Connection D:                                             *

|    *             Sig Sig Sig ... Sig Sig                                    *

|    *             Gnd Sig Sig ... Sig Sig                                    *

|    *                                                                        *

|    * 7) All connector pins must be specified with non-zero L and C values   *

|    *    in at least one of the sections.  The simulator will zero out       *

|    *    portions of all sections corresponding to the ground or power pins. *

|    *                                                                        *

|    * 8) SLM models are most useful where                                    *

|    *    coupling analysis is not critical and where the rise time of the    *

|    *    signal is long compared to the electrical delay through the         *

|    *    connector.                                                          *

|    *                                                                        *

|    *                                                                        *

|    *	Multi-Line Partial-Inductance Matrix:                                 *

|    * -------------------------------------                                  *

|    * DESCRIPTION:                                                           *

|    * A partial inductance is an approximation of the loop inductance in     *

|    * situations where the current return path cannot be explicitly stated   *

|    * in the field extraction stages.                                        *

|    * Partial inductance can be numerically calculated by introducing a      *

|    * current in a circuit element that originates on one end and terminates *

|    * on the other end. There is no physical loop per se, instead charges    *

|    * will build up on each side of the circuit element.  In practice, this  *

|    * situation cannot exist more than instantaneously.  For the purposes of *

|    * simulation the condition does not affect the inductance values.        *

|    * Then from the currents found the magnetic fields and inductances can   *

|    * be calculated.  In this way, the self and mutual inductances of any    *

|    * section of a closed circuit can be  calculated.                        *

|    * This representation of inductance in terms of partial self and mutual  *

|    * inductance allows the analysis of any combination of current paths.    *

|    * The signal and return current paths can be defined in circuit after    *

|    * the model is established. In this way the circuit under simulation     *

|    * accounts for the impact on signal-integrity, cross-talk and            *

|    * ground-bounce from currents passing through any combination of pins.   *

|    *                                                                        *

|    *                    connector or section                                *

|    *                ----------------------------                            *

|    *               |                            |                           *

|    *             Signal .....................  Signal                       *

|    *    (side A)   |                            |       (side B)            *

|    *             Signal .....................  Signal                       *

|    *               |                            |                           *

|    *             Signal .....................  Signal                       *

|    *               |                            |                           *

|    *                ----------------------------                            *

|    *                                                                        *

|    * There are many ways to create this information using either 2D or 3D   *

|    * fieldsolvers.  It may also be possible to create this information      *

|    * using other methods which are left to the model developer.             *

|    *                                                                        *

|    *                                                                        *

|    * NOTES (Multi-Line Partial-Inductance Matrix):                          *

|    *                                                                        *

|    * 1) For an MLM model it is required that at least one section include   *

|    *    mult-line inductance and capacitance matrices.                      *

|    *                                                                        *

|    *                                                                        *

|    **************************************************************************

|    *                    CAPACITANCE MATRIX DESCRIPTION                      *

|    *                                                                        *

|    * UNITS:      FARAD                                                      *

|    *                                                                        *

|    * DESCRIPTION:                                                           *

|    * Two Maxwell capacitance matrix methods are supported:                  *

|    *                                                                        *

|    *       SLM, Single-Line, Loop-capacitance Matrix    (non-coupled)       *

|    *       MLM, Multi-Line,  Partial-capacitance Matrix (coupled)           *

|    *                                                                        *

|    * If ModelType is SLM then all sections must use single-line             *

|    * loop-capacitance and inductance matrices.                              *

|    *                                                                        *

|    * If ModelType is MLM then all sections must use                         *

|    * partial-capacitance and inductance matrices.                           *

|    *                                                                        *

|    * Both capacitance and inductance matrix of a given section must use     *

|    * the same method.                                                       *

|    *                                                                        *

|    *	Single-Line Loop-Capacitance Matrix:                                  *

|    * -----------------------------------                                    *

|    * DESCRIPTION:                                                           *

|    * To define capacitance requires conductors separated by a dielectric.   *

|    * The conductors are the those found in a loop consisting of a closed    *

|    * path including a signal and all possible return paths.                 *

|    * The Maxwell Capacitance matrix relates the total charges on the        *

|    * conductors to the voltages applied to them.                            *

|    * [Q]=[Cm][V]                                                            *

|    * Where [Cm] is the Maxwell matrix, [Q] is the vector of total conductor *

|    * charges, and [V] is a vector of applied voltages. The negative values  *

|    * of the off-diagonal entries can be explained on physical grounds:      *

|    * if you put +1 Volt on a single conductor and 0 Volts on the others,    *

|    * then you induce negative charges on those other conductors to terminate*

|    * the field lines emanating from the conductor at 1 Volt.                *

|    *                                                                        *

|    * In the Maxwell Capacitance matrix, the diagonal term is the maximum    *

|    * capacitance that the conductor encounters, meaning the capacitance to  *

|    * ground plus the capacitances to all the other conductors.              *

|    *                                                                        *

|    * In the Maxwell Capacitance matrix, the OFF-diagonal term is the        *

|    * capacitance that the conductor encounters relative to a second         *

|    * conductor.  This capacitance value is effected by other nearby         *

|    * conductors during the field extraction phase.  As such it should not   *

|    * normally be used by itself to determine the capacitance of a random    *

|    * pair of conductors in a given structure.                               *

|    *                                                                        *

|    *	Also see "Single-Line Loop-Inductance Matrix" section above for more  *

|    * information on Single-Line Matrices.                                   *

|    *                                                                        *

|    *	Multi-Line Partial-Capacitance Matrix:                                *

|    * --------------------------------------                                 *

|    * DESCRIPTION:                                                           *

|    * A partial capacitance is an approximation of the capacitance in        *

|    * situations where the return path cannot be explicitly stated           *

|    * in the field extraction stages.                                        *

|    *                                                                        *

|    *	See "Multi-Line Partial-Capacitance Matrix" section above for more    *

|    * information on Multi-Line Matrices.                                    *

|    **************************************************************************

|    *                    LOSS MATRIX DESCRIPTION                             *

|    *                                                                        *

|    * This release does not support frequency dependent loss matrices.       *

|    * A future release is expected to add support for the numerous           *

|    * frequency dependent loss matrix effects which may occur including:     *

|    * skin effect, dielectric frequency dependence, surface roughness etc.   *

|    **************************************************************************

|

|    **************************************************************************

|    *             MATRIX DATA DERIVATION NOTES                               *

|    *                                                                        *

|    * UNCOUPLED MODELS:                                                      *

|    * -----------------                                                      *

|    * These are also called Single-Line-Models or SLM models.                *

|    * These models are useful for most simulations where coupled effects     *

|    * can be ignored safely.  These models are most accurate if closely      *

|    * spaced ground pins or a ground plane is present.                       *

|    * A "Diagonal_matrix" matrix type is the best choice for describing      *

|    *    all matrices R,L and C for SLM models.                              *

|    *    (see matrix  descriptions below).                                   *

|    * Several methods can be used:                                           *

|    * 1) 2D field solver with return path pins defined.  Only the diagonal   *

|    *    matrix data is used.  The coupling information is ignored.          *

|    *    The R,L,C matrix results from the field solver usually in           *

|    *    per-unit-length form must be multiplied times the actual length     *

|    *    of this section so that the final matrix represents the specific    *

|    *    length of this section.                                             *

|    *    (Does not allow simulation of coupling)                             *

|    * 2) 2D field solver without specific return path pins.  A return plane  *

|    *    is specified as a reference only and is placed far field.           *

|    *    from the pin field.  Only the diagonal matrix data is used and the  *

|    *    off diagonal information must be appropriately reduced to a single  *

|    *    diagonal matrix. No coupling data is present in the final matrix    *

|    *    The R,L,C matrix results from the field solver usually in	     *

|    *    per-unit-length form must be multiplied times the actual length	     *

|    *    of this section so that the recorded matrix data includes the	     *

|    *    length of the section.							     *
|    *                                                                        *

|    *    (Does not allow simulation of coupling)		                  *

|    * 3) Closed form round wire inductance equation.			 	     *

|    *    (Does not allow simulation of coupling)	                        *

|    * 4) Experimental lab data may be obtained from TDR or other means	    *

|    *    (Does not allow simulation of coupling)	                        *

|    *										 	    *

|    *										 	    *

|    * COUPLED (Preferred method):           					    *

|    * --------------------------          				           *

|    * These are also called Multi-Line-Models or MLM models.	 	    *

|    * A "Diagonal_matrix" matrix type is the best choice for describing      *

|    *    all matrices R,L and C for SLM models.                              *

|    *    (see matrix  descriptions below).                                   *

|    * Several methods can be used:                                           *

|    * 1) 3D field solver can be used to create partial inductance and        *

|    *    capacitance matrices that allow accurate simulations without        *

|    *    the need to pre-define the ground pins.                             *

|    * 2) 2D field solver without specific return path pins.  A return plane  *

|    *    is specified as a reference only and is placed far field.           *

|    *    The R,L,C matrix results from the field solver usually in           *

|    *    per-unit-length form must be multiplied times the actual length	     *

|    *    of this section so that the recorded matrix data includes the	     *

|    *    length of the section.							     *

|    *    (Does not include 3D effects)                                       *

|    * 3) 2D field solver with return path pins specifically defined.         *

|    *    The R,L,C matrix results from the field solver usually in           *

|    *    per-unit-length form must be multiplied times the actual length     *

|    *    of this section so that the recorded matrix data includes the       *

|    *    length of the section.                                              *

|    *    (Does not include 3D effects)                                       *

|    *    found in structures like U bends on connector fingers.              *

|    * 4) Experimental lab data may be obtained from TDR or other means.      *

|    *    This method will generally not allow the most accurate models       *

|    *    as the inner sections of the connector generally cannot be          *

|    *    probed.                                                             *

|    **************************************************************************

|

|     One common aspect of all the different formats is that they exploit

|     the symmetry of the matrices they describe.  This means that the

|     entries below the main diagonal of the matrix are identical to the

|     corresponding entries above the main diagonal.  Therefore, only

|     roughly one-half of the matrix needs to be described.  By convention,

|     the main diagonal and the UPPER half of the matrix are provided.

|

|     In the following text, we use the notation [I, J] to refer to the entry

|     in row I and column J of the matrix.  Note that I and J are allowed

|     to be alphanumeric strings as well as integers.  An ordering of these

|     strings is defined in the [Pin Numbers] section.  In the following text,

|     "Row 1", means the row corresponding to the first pin.

|

|     Also note that the numeric entries of the RLC matrices are standard

|     IBIS floating point numbers.  As such, it is permissible to use

|     metric "suffix" notation.  Thus, an entry of the C matrix could be

|     given as 1.23e-12 or as 1.23p or 1.23pF.

|

|     Full_matrix

|        When the Full_matrix format is used, the couplings between

|        every pair of elements is specified explicitly.  Assume that

|        the matrix has N rows and N columns.  The Full_matrix is specified

|        one row at a time, starting with Row 1 and continuing down to Row N.

|

|     Each new row is identified with the Row keyword.

|------------------------------------------------------------------------------

| See example "Matrix Example" lower in this document.

|==============================================================================

| Keyword:        [Row]

| Required:       Yes if a matrix is being defined.

| Description:

|     Indicates the beginning of a new row of the matrix.

|

| Usage Context:

|     1) MUST be used between keywords [Begin_Cn_Section] and [End_Cn_Section].

|     2) MUST follow any usage of one of the following keywords:

|       Banded_matrix, Sparse_matrix, Full_matrix

|

| Usage:

|     The argument must be a sequential number beginning at 1

|     and continuing to the last row for the matrix

|

|     Following a [Row] keyword is a block of numbers that represent the

|     entries for that row.  Suppose that the current row is number M.  Then

|     the first number listed is the diagonal entry, [M,M].  Following this

|     number are the entries of the upper half of the matrix that belong to

|     row M:

|        [M, M+1], [M, M+2], ... up to [M,N].

|

|     For even a modest-sized matrix, this data will not all fit on one line.

|     You can break the data up with new-line characters so that this limit is

|     observed.

|

|     An example: suppose the matrix has 40 conductors and that we are

|     currently working on Row 19.  There is 1 diagonal entry, plus

|     40 - 19 = 21 entries in the upper half of the matrix to be specified,

|     for 22 entries total.  The data might be formatted as follows:

|

|     In the example below, the entry 5.67e-9 is on the diagonal of row 19.

|

|     Observe that Row 1 always has the most entries, and that each

|     successive row has one fewer entry than the last; the last row always

|     has just a single entry.

|==============================================================================

| See example "Matrix Example" lower in this document.

|

|==============================================================================

| Keyword:        Diagonal_matrix

| Required:       No

| Description:

|     The Diagonal_matrix is used to specify uncoupled model.  This keyword

|     is exactly the same as using the Banded_matrix described below with a

|     Bandwidth=0.

|

| Usage Context:

|     1) MUST be used between keywords [Begin_Cn_Section] and [End_Cn_Section].

|

| Usage:

|     A Diagonal_matrix is one whose entries are guaranteed to be zero if they

|     are not on the main diagonal.  Let the matrix size be N x N,

|     An entry [I,J] of the matrix is zero if: | I - J | != 0

|     where |.| denotes the absolute value.

|

|     Specify the Diagonal_matrix one row at a time, starting with row 1 and

|     working up to higher rows.  Diagonal elements are entered one per line.

|==============================================================================

| See example "Matrix Example" lower in this document.

|

|==============================================================================

| Keyword:        Banded_matrix

| Required:       No

| Description:

|     The Banded_matrix is used to specify the coupling effects up to B pins

|     on either side.  Two variations are supported.  One allows for the

|     coupling to circle back on itself.  This is technically a simple form

|     of a bordered block diagonal matrix.  However, its data can be

|     completely specified in terms of a Banded_matrix for an N x M matrix

|     consisting of N rows and M = N + B columns.

|     The second variation is just in terms of an N x N matrix where no

|     circle back coupling needs to be specified.

|

| Usage Context:

|     1) MUST be used between keywords [Begin_Cn_Section] and [End_Cn_Section].

|

| Usage:

|     A Banded_matrix is one whose entries are guaranteed to be zero if they

|     are farther away from the main diagonal than a certain distance, known

|     as the "bandwidth."  Let the matrix size be N x M, and let the bandwidth

|     be B.  An entry [I,J] of the matrix is zero if:

|        | I - J | > B

|     where |.| denotes the absolute value.

|

|     The bandwidth for a Banded_matrix must be specified using the

|     [Bandwidth] keyword:

|

|     Specify the banded matrix one row at a time, starting with row 1 and

|     working up to higher rows.  Mark each row with the [Row] keyword, as

|     above.  As before, symmetry is exploited: do not provide entries below

|     the main diagonal.

|

|     For case where coupling can circle back on itself, consider a matrix of

|     N pins organized into N rows 1 ... N and M columns 1 ... N, 1 ... B.

|     The first row only needs to specify the entries [1,1] through [1,1+B]

|     since all other entries are guaranteed to be zero.  The second row will

|     need to specify the entries [2,2] through [2,2+B], and so on.  For row K

|     the entries [K,K] through [K,K+B] are given when K + B is less than or

|     equal to the size of the matrix N.  When K + B exceeds N, the entries

|     in the last columns 1 ... B specify the coupling to the first rows.  For

|     row K, the entries [K,K] ... [K,N] [K,1] ... [K,R] are given where R =

|     mod(K + B - 1, N) + 1.  All rows will contain B + 1 entries.  To avoid

|     redundant entries, the bandwidth is limited to B <= int((N - 1) / 2).

|

|     For the case where coupling does not circle back on itself, the process

|     is modified.  Only N columns need to be considered.  When K + B finally

|     exceeds the size of the matrix N, the number of entries in each row

|     starts to decrease; the last row (row N) has only 1 entry.  This

|     construction constrains the bandwidth to B < N.

|

|     As in the Full_matrix, if all the entries for a particular row do not

|     fit into a single 120-character line, the entries can be broken across

|     several lines.

|

|     It is possible to use a bandwidth of 0 to specify a diagonal matrix

|     (a matrix with no coupling terms.)  This is sometimes useful for

|     resistance matrices.

|==============================================================================

| See example "Matrix Example" lower in this document.

|==============================================================================

| Keyword:        [Bandwidth]

| Required:       Yes if a Banded_matrix matrix is being defined.

| Description:

|     Indicates the bandwidth of the matrix.  The bandwidth field

|     must be a nonnegative integer.  This is the number of off-diagonal matrix

|     rows for which data is provided.

|

| Usage Context:

|     1) MUST be used between keywords [Begin_Cn_Section] and [End_Cn_Section].

|     2) MUST follow any usage of one of the following keyword: Banded_matrix

|

| Usage:

|==============================================================================

| See example "Matrix Example" lower in this document.

|==============================================================================

| Keyword:        Sparse_matrix

| Required:       No

| Description:

|     A Sparse_matrix is expected to consist mostly of zero-valued entries,

|     except for a few nonzero.  Unlike the Banded_matrix, there is no

|     restriction on where the nonzero entries can occur.  This feature is

|     useful in certain situations, such as for Pin Grid Arrays (PGAs)

|     sockets.

|

|     As usual, symmetry can be exploited to reduce the amount of data by

|     eliminating from the matrix any entries below the main diagonal.

|

| Usage Context:

|     1) MUST be used between keywords [Begin_Cn_Section] and [End_Cn_Section].

|

| Usage:

|     An N x N Sparse_matrix is specified one row at a time, starting with

|     row 1 and continuing down to row N.  Each new row is marked with [Row]

|     keyword, as in the other matrix formats.

|

|     Data for the entries of a row is given in a slightly different format,

|     however.  For the entry [I, J] of a row, it is necessary to explicitly

|     list the name of pin J before the value of the entry is given.  This

|     specification serves to indicate to the parser where the entry is put

|     into the matrix.

|

|     Note that each of the column indices listed for any row must be

|     greater than or equal to the row index, because they always come from

|     the upper half of the matrix.  When alphanumeric pin names are used,

|     special care must be taken to ensure that the ordering defined in the

|     [Pin Numbers] section is observed.

|

|     With this convention, please note that the N'th row of an

|     N x N matrix has just a single entry (the diagonal entry).

|

|     The proper location is not otherwise obvious because of the lack of

|     restrictions on where nonzero entries can occur.  Each (Index, Value)

|     pair is listed upon a separate line.  An example follows.  Suppose

|     that row 10 has nonzero entries [10,10], [10,11], [10,15],

|     and [10,25].  The following row data would be provided:

|==============================================================================

| See example "Matrix Example" below.

|

|=============================================================================

|==============================================================================

|                  M A T R I X      E X A M P L E

|==============================================================================

|=============================================================================

|

| The resistance matrix for this package has no coupling

|

[Begin_Cn_Section]       ExampleMatrix00

[Resistance Matrix]     Diagonal_matrix

10.0

15.0

15.0

10.0

10.0

15.0

15.0

10.0

[End_Cn_Section] ExampleMatrix00

[Begin_Cn_Section]       ExampleMatrix01

|

| The resistance matrix for this package has no coupling

| and is exactly the same as the Diagonal_matrix shown above

| it is described using a trivial version of the Banded_matrix

| as an example only.  The Diagonal_matrix should be used for

| this example to conserve file size.

|

[Resistance Matrix]     Banded_matrix

[Bandwidth]             0

[Row]   1

10.0

[Row]   2

15.0

[Row]   3

15.0

[Row]   4

10.0

[Row]   5

10.0

[Row]   6

15.0

[Row]   7

15.0

[Row]   8

10.0

|

| The inductance matrix has coupling between pins

|

[Inductance Matrix]     Full_matrix

[Row]   1

3.04859e-07      4.73185e-08      1.3428e-08     6.12191e-09

1.74022e-07      7.35469e-08     2.73201e-08     1.33807e-08

[Row]   2

3.04859e-07      4.73185e-08      1.3428e-08     7.35469e-08

1.74022e-07      7.35469e-08     2.73201e-08

[Row]   3

3.04859e-07      4.73185e-08     2.73201e-08     7.35469e-08

1.74022e-07      7.35469e-08

[Row]   4

3.04859e-07      1.33807e-08     2.73201e-08     7.35469e-08

1.74022e-07

[Row]   5

4.70049e-07      1.43791e-07     5.75805e-08     2.95088e-08

[Row]   6

4.70049e-07      1.43791e-07     5.75805e-08

[Row]   7

4.70049e-07      1.43791e-07

[Row]   8

4.70049e-07

|

| The capacitance matrix has sparse coupling

|

[Capacitance Matrix]    Sparse_matrix

[Row]   1

1       2.48227e-10

2       -1.56651e-11

5       -9.54158e-11

6       -7.15684e-12

[Row]   2

2       2.51798e-10

3       -1.56552e-11

5       -6.85199e-12

6        -9.0486e-11

7       -6.82003e-12

[Row]   3

3       2.51798e-10

4       -1.56651e-11

6       -6.82003e-12

7        -9.0486e-11

8       -6.85199e-12

[Row]   4

4       2.48227e-10

7       -7.15684e-12

8       -9.54158e-11

[Row]   5

5       1.73542e-10

6       -3.38247e-11

[Row]   6

6       1.86833e-10

7       -3.27226e-11

[Row]   7

7       1.86833e-10

8       -3.38247e-11

[Row]   8

8       1.73542e-10

|

[End_Cn_Section] ExampleMatrix01

|==============================================================================

| Keyword:        [End]

| Required:       Yes

| Description:

|     Specifies end of file

|[End]

|

|=============================================================================

|==============================================================================

|                   C O N N E C T O R      E X A M P L E S

|==============================================================================

|=============================================================================

|

[IBIS_Cn_Model_Ver]  1.0
[Date]          Jan 1, 2000          | The latest file revision date

|

[Begin_Cn_Model_Family]
[Begin_Cn_Model_List]
DB9               Mated         10ns       db9mated.jpg

Header2x5     Mated         500ps     Header100.jpg 
Header2x5s   Mated         500ps     Header100.jpg

[End_Cn_Model_List]

|
|EXAMPLE: single SLM DB9 connector  (NOTE: DATA IS “MAKE BELIEVE”)

|

[Begin_Cn_Model] DB9 PinMapDB9 PhyMapDB9 SLM 1:9

[Cn_Number_of_Conductors]  9
[Cn_Columns_of_Pins]            5
[Cn_Rows_of_Pins]                 2
Cn_Section                               SectionDB9
[End_Cn_Model] DB9 PinMapDB9 PhyMapDB9   |side B pin and physical maps
|
EXAMPLE: single SLM DB9 connector with swath (NOTE: DATA IS “MAKE BELIEVE”)

|

[Begin_Cn_Model] DB25swath PinMapDB25 PhyMapDB25 SLM 1:25

[Cn_Number_of_Conductors]   25
[Cn_Columns_of_Pins]             13
[Cn_Rows_of_Pins]                  2
Cn_Section                                SectionDBSwath
|               SwathRows SwathCols RightEdge LeftEdge TopEdge BottomEdge

[Begin_Cn_Swath]          2                     3                 1                     1               0                 0

[Cn_Z] 50
[End_Cn_Swath]
[End_Cn_Model] DB25swath PinMapDB25 PhyMapDB25 |side B pin and physical maps
|
|Pin and Physical maps for DB9

[Begin_Cn_Pin_Map] PinMapDB9
|Index Pin    Signal   
1          1      CD
2          2      RXD
3          3      TXD
4          4      DTR
5          5      GND
6          6      DSR
7          7      RTS
8          8      CTS
9          9      RI
 [End_Cn_Pin_Map]

[Begin_Cn_Phy_Map] PhyMapDB9

[PhyRow] 1

1 2 3 4 5

[PhyRow] 2

6 7 8 9

[End_Cn_Phy_Map]

|

|Pin and Physical maps for DB25

[Begin_Cn_Pin_Map] PinMapDB25
|Index Pin    Signal   
1          1      CD
2          2      RXD
3          3      TXD
4          4      DTR
5          5      GND
6          6      DSR
7          7      RTS
8          8      CTS
9          9      RI
10        10      CD
11        11      CD
12        12      RXD
13        13      TXD
14        14      DTR
15        15      GND
16        16      DSR
17        17      RTS
18        18      CTS
19        19      RI
20        20      RI
21        21      CD
22        22      RXD
23        23      TXD
24        24      DTR
25        25      GND
 [End_Cn_Pin_Map]

[Begin_Cn_Phy_Map] PhyMapDB25

[PhyRow] 1

1 2 3 4 5 6 7 8 9 10 11 12 13

[PhyRow] 2

14 15 16 17 18 19 20 21 22 23 24 25

[End_Cn_Phy_Map]

|

|EXAMPLE========= M A T R I X   S E C T I O N S ==========================

[Begin_Cn_Section] SectionDB9
[Resistance Matrix] Diagonal_matrix
.05
.05
.05
.05
.05
.05
.05
.05
.05
[Inductance Matrix] Diagonal_matrix
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
[Capacitance Matrix] Diagonal_matrix
0.3pF          |Edge pin has less capacitance	
0.5pF
0.5pF
0.5pF
0.3pF          |Edge pin has less capacitance	
0.3pF          |Edge pin has less capacitance
0.5pF
0.5pF
0.3pF          |Edge pin has less capacitance	
[End_Cn_Section] SectionDB9

|
[Begin_Cn_Section] SectionDBswath
[Resistance Matrix] Diagonal_matrix
.05
.05
.05
.05
.05
.05
 [Inductance Matrix] Diagonal_matrix
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
1.5nH
 [Capacitance Matrix] Diagonal_matrix
0.3pF          |Edge pin has less capacitance	
0.3pF          |Edge pin has less capacitance
0.5pF
0.5pF
0.3pF          |Edge pin has less capacitance	
0.3pF          |Edge pin has less capacitance
 [End_Cn_Section] SectionDBswath

|

| [Endn_Cn_Model_Family]
|

[End]