=============================================================================

# I B I S    I N T E R C O N N E C T   S P E C I F I C A T I O N

=============================================================================

Specification Revision level 1.0 draft

=============================================================================

## Table of Contents

=============================================================================

I B I S    I N T E R C O N N E C T   S P E C I F I C A T I O N	1

Table of Contents	1

REVISION HISTORY	2

1.0 GENERAL INTRODUCTION	2

2.0 STATEMENT OF INTENT	2

3.0 GENERAL SYNTAX RULES AND GUIDELINES	2

4.0 KEYWORD TREE DIAGRAM	4

5.0 HEADER KEYWORDS	5

Keyword: [Begin Header]	5

Keyword: [IBIS ICM Model Ver]	6

Keyword: [File Name]	6

Keyword: [File Rev]	6

Keyword: [Date]	6

Keyword: [Source]	7

Keyword: [Notes]	7

Keyword: [Disclaimer]	7

Keyword: [Copyright]	8

Keyword: [Support]	8

Keyword: [Redistribution]	8

Keyword: [Redistribution Text]	9

Keyword: [End Header]	9

6.0 GLOBAL KEYWORDS	9

Keyword: [Comment Char]	10

7.0 INTERCONNECT MODEL FAMILY KEYWORDS	10

Keyword: [Begin ICM Model Family]	10

Keyword: [Manufacturer]	10

Keyword: [ICM Family Description]	11

Keyword: [ICM Model List]	11

Keyword: [Begin ICM Model]	13

Keyword: [ICM Model Description]	14

Keyword: [Tree Path Description]	15

Keyword: [Nodal Path Description]	20

Keyword: [ICM Swath Parameters]	26

Keyword: [ICM Swath Pin Numbers]	30

Keyword: [End ICM Model]	31

Keyword: [ICM Pin Map]	32

Keyword: [ICM Node Map]	34

Keyword: [End ICM Family]	38

8.0 MATRIX KEYWORDS	38

Keyword: [Begin ICM Section]	38

Keyword: [Derivation Method]	39

Keyword: [Resistance Matrix], [Inductance Matrix], [Conductance Matrix],                [Capacitance Matrix]	40

Matrix Descriptions	41

Resistance Matrix Description:	42

Inductance Matrix Description:	42

Loop-Inductance Matrix Description (SLMs ONLY):	42

Partial Self Inductance Matrix Description (MLMs ONLY):	44

Capacitance Matrix Description	44

Conductance Matrix Description:	45

Loss Matrix Description:	46

Keyword: [Bandwidth]	46

Keyword: [Row]	46

Keyword: [ICM S-parameter]	47

Keyword: [End ICM Section]	48

Matrix Examples:	48

9.0 End Keyword	49

Keyword: [End]	50

=============================================================================

# REVISION HISTORY

09-18-2002, v1.0  Draft Specification

=============================================================================

=============================================================================

# 1.0 GENERAL INTRODUCTION

=============================================================================

This document contains a general purpose specification for interconnect

modeling in an IBIS (I/O Buffer Information Specification) compatible format.

It was written to provide means for modeling all electrical interconnect

types, including connectors, cables, packages, and printed circuit boards.

This section gives a general overview of the remainder of this document.

Sections 2 to 5 contain general information about the IBIS versions and the

general rules and guidelines.  The remaining sections describe the

interconnect keywords and usage rules.

=============================================================================

# 2.0 STATEMENT OF INTENT

=============================================================================

In order to enable an industry standard method to electronically transport

IBIS interconnect modeling data between interconnect design and simulation

tools.  The intention of this specification is to specify a consistent format

that can be parsed by software, allowing simulation vendors to derive models

compatible with their own products.

One goal of this template is to represent the current state of IBIS data,

while allowing a growth path to more complex models / methods (when deemed

appropriate).  This would be accomplished by a revision of the base

template, and possibly the addition of new keywords or categories.

Another goal of this template is to ensure that model creation is simple

enough for interconnect vendors and customers to use, while ensuring that

it is rigid enough for simulation vendors to write reliable parsers.

This template is meant to contain a complete description of the

elements of an entire interconnect.

This specification is intended to provide a basis for future versions

which are expected to be backward compatible with this initial version.

=============================================================================

# 3.0 GENERAL SYNTAX RULES AND GUIDELINES

=============================================================================

1) Only ASCII characters, as defined in ANSI Standard X3.4-1986, may be used in

an IBIS-Interconnect file.  The use of characters with codes greater than

hexadecimal 07E is not allowed.  Also, ASCII control characters (those

numerically less than hexadecimal 20) are not allowed, except for TAB

characters and line termination sequences.

NOTE: the use of TAB characters is strongly discouraged.

2) Except for keywords (words enclosed by a square bracket []), the content of

an IBIS Interconnect Model (ICM) file is case sensitive.

3) Keywords must be enclosed in square brackets, [], and must start in column 1

of the line.  No space is allowed immediately after the opening bracket “[“ or

immediately before the closing bracket “]”.  If used, only one space (“ “) or

underscore (“_”) character separates the parts of a multi-word keyword. Spaces

and underscores are equivalent within square bracket. Some keywords may be

followed by an argument. Keyword arguments are of three types: either a text

string, a file name, or a text block.  Text string and file name arguments

start on the same line as the keyword and are terminated with a line

termination sequence. A text block starts on the same line as the keyword, may

extend over multiple lines, and is terminated by the occurrence of the next

keyword.

4) Keywords and subparameters must begin with a letter, and shall consist only

of alphanumeric characters and the underscore “_”.  Spaces are not allowed in

subparameter names.

5) Each IBIS Interconnect data file is divided into sections.  Each section is

delimited by keywords of the form [Begin "section name"] and [End "section

name"]. Within each section only specific keywords are legal (note, however, a

keyword may be legal in more than one section).  Unless otherwise noted, a

section's keywords can appear in any order.

6) To facilitate portability between operating systems, file names used in the

IBIS Interconnect file must only have lower case characters.  File names should

have a basename followed by a period (“.”), followed by a file name extension

of no more than three characters. There is no length restriction on the

basename. The basename and extension must use characters from the following set

(space, “ “, 0x20 is not included):

a b c d e f g h i j k l m n o p q r s t u v w x y z

0 1 2 3 4 5 6 7 8 9 _ -

7) A line of the file may have at most 120 characters, followed by a line

termination sequence.  The line termination sequence must be one of the

following two sequences: a linefeed character, or a carriage return followed by

a linefeed character.

8) Anything following the comment character is ignored and considered a comment

on that line.  The default "|" (pipe) character can be changed by the keyword

[Comment Char] to any other character. The [Comment Char] keyword can be used

throughout the file as desired.

9) Valid scaling factors are:

T = tera	k = kilo	n = nano

G = giga	m = milli	p = pico

M = mega	u = micro	f = femto

When no scaling factors are specified, the appropriate base units are assumed.

(These are volts, amperes, ohms, farads, henries, and seconds).  The parser

looks at only one alphabetic character after a numerical entry; therefore it is

enough to use only the prefixes to scale the parameters.  However, for clarity,

it is allowed to use full abbreviations for the units, (e.g., pF, nH, mA,

mOhm).  In addition, scientific notation IS allowed (e.g., 1.2345e-12).

10) All temperatures are represented in degrees Celsius.

11) All lines that occur before the [Begin Header] keyword or after the [End]

keyword shall be treated as comments and ignored by the parser. The intent

of this rule is to facilitate the inclusion of HTML tags, revision control

headers, etc. at the beginning of the file.

12) The following words are reserved words and must not be used for

any other purposes in the document:

POWER - reserved model name, used with power supply pins,

GND   - reserved model name, used with ground pins,

NC    - reserved model name, used with no-connect pins,

NA    - used where data not available.

=============================================================================

# 4.0 KEYWORD TREE DIAGRAM

=============================================================================

/-- Start of File

|   -------------

|

|--/--[Begin Header]

|  |  |--[IBIS ICM Model Ver]

|  |  |--[Comment Char]

|  |  |--[File Name]

|  |  |--[File Rev]

|  |  |--[Date]

|  |  |--[Source]

|  |  |--[Notes]

|  |  |--[Disclaimer]

|  |  |--[Copyright]

|  |  |--[Support]

|  |  |--[Redistribution]

|  |  |--[Redistribution Text]

|  \--[End Header]

|

|

|--/--[Begin ICM Family]         | (Chapter 7)

|  |  |--[Manufacturer]

|  |  |--[ICM Family Description]

|  |  |--[ICM Model List]

|  |

|  |--/--[Begin ICM Model]

|  |  |    ICM_Model_Type

|  |  |    SGR

|  |  |    Ref_impedance

|  |  |--[ICM Model Description]

|  |  |--[Tree Path Description]

|  |  |    Model_PinMap

|  |  |    Fork, End_fork

|  |  |    Section

|  |  |--[Nodal Path Description]

|  |  |    Model_NodeMap

|  |  |    N_section

|  |  |--[ICM Swath Parameters]        | (optional for Swath)

|  |  |    Left_edge

|  |  |    Right_edge

|  |  |    Top_edge

|  |  |    Bottom_edge

|  |  |--[ICM Swath Pin Numbers]       | (optional for Swath)

|  |  \--[End ICM Model]

|  |

|  |--[ICM Pin Map]                    | Pins to Physical

|  |    Pin_order

|  |    Num_of_columns

|  |    Num_of_rows

|  |--[ICM Node Map]                   | Nodes to Physical

|  \--[End ICM Family]

|

|

|--/--[Begin ICM Section]              | (Chapter 8)  Define Matrices

|  |--[Derivation Method]

|  |--[Resistance Matrix]

|  |    /-- [Bandwidth]

|  |    \-- [Row]

|  |--[Inductance Matrix]

|  |    /-- [Bandwidth]

|  |    \-- [Row]

|  |--[Conductance Matrix]

|  |    /-- [Bandwidth]

|  |    \-- [Row]

|  |--[Capacitance Matrix]

|  |    /-- [Bandwidth]

|  |    \-- [Row]

|  |

|  |--[ICM S-parameter]

|  |    Pin_order

|  |    Num_of_columns

|  |    Num_of_rows

|  |

|  \--[End ICM Section]

|

|

\--[End]

=============================================================================

# 5.0 HEADER KEYWORDS

=============================================================================

## Keyword: [Begin Header]

Required: Yes

Uses: [IBIS ICM Model ver], [File Name], [File Rev], [Date], [Source],

[Notes], [Disclaimer], [Copyright], [Support], [Redistribution],

[Redistribution Text], [End Header]

Description: This keyword denotes the beginning of the file header section.

Usage Rules: The [Begin Header] keyword may appear only once per ICM file and

must be the first keyword in the file.  This keyword may be

proceeded by lines of general text and/or comments which should

be treated as comments by the parser.

-----------------------------------------------------------------------------

[Begin Header]

===========================================================================

## Keyword: [IBIS ICM Model Ver]

Required: Yes

Argument: Text String

Used By: [Begin Header]

Description: This keyword allows electronic parsers to immediately determine

that this file contains an IBIS interconnect model.

The version number is used to inform the parser what keywords

are valid for this model and to allow backward support as new

keywords are added.

Usage Rules: Only one [IBIS ICM Model Ver] keyword is allowed per file, which

must appear directly after the [Begin Header] keyword.

-----------------------------------------------------------------------------

[IBIS ICM Model Ver]  1.0

===========================================================================

## Keyword: [File Name]

Required: Yes

Argument: Text String (file name)

Used by: [Begin Header]

Description: Specifies the name of the IBIS Interconnect Model (ICM) file

Usage Rules: The purpose of this keyword is to document a single filename

the file uses even if the file is transferred from one

computer to another.  The file name must conform to the rules

given in section 4, "General Syntax Rules and Guidelines".

The [File Name] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[File Name]     iconm_hdi_202.icm

=============================================================================

## Keyword: [File Rev]

Required: Yes

Argument: Text String

Used By: [Begin Header]

Description: Tracks the revision level of a particular ICM file

Usage Rules: Revision level is set at the discretion of the engineer

defining the file.  The following guidelines are suggested:

0.x     interconnect and/or file in development

1.x     pre-manufacturing interconnect and/or model

2.x     mature interconnect and/or model, no more changes

likely

The [File Rev] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[File Rev]     1.0                   |  Used for ICM file versions

=============================================================================

## Keyword: [Date]

Required: No

Argument: Text String (Date)

Used By: [Begin Header]

Description: Date this file was last modified

Usage Rules: This keyword is provided to insure the last changed date for

this file is not lost if the file is transmitted between

computer systems. The format is fixed.  The pattern to be used

is “Month Day, Year”.  Month is fully written out.  Day uses two

digits.  Year uses four digits.  See example.

The [Date] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Date]          July 01, 2000         | The latest file revision date

=============================================================================

## Keyword: [Source]

Required: No

Argument: Text Block

Used by: [Begin Header]

Description: Records the originating source of model data

Usage Rules: Use this keyword to record how the model information was

obtained (physical measurement of device, simulations, data

book, etc.). This field contain the name of the company or

entitity that created the model.

The [Source] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Source] Put originator and the source of information here. For example:

Results from field simulation

=============================================================================

## Keyword: [Notes]

Required: No

Argument: Text Block

Used By: [Begin Header]

Description: Optional notes regarding the file

Usage Rules: The keyword provides a place for the model maker to record

important notes about the file or model data that are not

included elsewhere.  Such information may include notes on

validation level, model limits, usage assumptions, etc.

The [Notes] keyword can only be used once.

The [Notes] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Notes]       Use this section for any special notes related to the file.

=============================================================================

## Keyword: [Disclaimer]

Required: No

Argument: Text Block

Used By: [Begin Header]

Description: Legal disclaimer and copyright information

Usage Rules: This keyword provides a place for the user to add a legal

disclaimer.

NOTE: It is recommended that the argument to the [Disclaimer]

keyword be limited to a maximum of 96 lines of text.

The [Disclaimer] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Disclaimer]  This information is for modeling purposes only, and is not

guaranteed.

=============================================================================

## Keyword: [Copyright]

Required: No

Argument: Text Block

Used By: [Begin Header]

Description: Legal copyright information

Usage Rules: Because model writers may consider the information in

these keywords essential to users, and sometimes legally

required, design automation tools should make this information

available. Any text following the [Copyright] keyword must be

included in any derivative models verbatim.

The [Copyright] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Copyright]   Copyright 2000, XYZ Corp., All Rights Reserved

=============================================================================

## Keyword: [Support]

Required: No

Argument: Text Block (URL path name)

Used By: [Begin Header]

Description: Specifies a web site that can be visited to get the latest

version of the file

Usage Rules: Following the [Support] keyword is the URL of a web site the

user may visit for more information on the model or model(s).

The entire link, including any file extension, is required. The

[Support] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

NOTE:  It is recommended that a separate IBIS model web site

(not page) be maintained to prevent accidental changes of page

name from breaking this link.

The [Support] keyword may appear only once between the [Begin

Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Support] http://www.VendorNameIbisModels.com     |example ibis web site root

=============================================================================

## Keyword: [Redistribution]

Required: Yes

Argument: Text String (Yes, No, Specific)

Used By: [Begin Header]

Description: Indicates to EDA tool companies and model users who may use and

redistribute this file

Usage Rules: Following the [Redistribution] keyword is one of three

arguments: "Yes", "No" and "Specific".

An argument value of "Yes" means that a EDA tool vendor or end

user may freely distribute the model as long as no fee is

charged.  A fee may be charged if authorized by the model

creator.

An argument value of "No" means that the model may not be

redistributed or retransmitted in any form.

An argument value of "Specific" means that specific license

information is contained in the [Redistribution Text] field.

Use of the "Specific" or “No” argument prevents automated

redistribution.

The [Redistribution] keyword may appear only once between the

[Begin Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Redistribution]  Yes

=============================================================================

## Keyword: [Redistribution Text]

Required: Yes, ONLY if [Redistribution] value is “Specific”, otherwise it

is optional

Argument: Text block

Used By: [Begin Header]

Description: Allows model creator to supply specific redistribution

information

Usage Rules: If the argument to the [Redistribution] keyword is “Specific”,

then the user must include additional information on licensing

details, or where to find them. Any text following the

[Redistribution Text] keyword must be included in any derivative

models verbatim.

Note: It is recommended that the argument to the [Redistribution

Text] keyword be limited to a maximum of 24 lines of text.

The [Redistribution Tesxt] keyword may appear only once between

the [Begin Header]/[End Header] keyword pair.

-----------------------------------------------------------------------------

[Redistribution Text]  Your text goes here.

===============================================================================

## Keyword: [End Header]

Required: Yes

Used By: [Begin Header]

Description: Marks the end of an IBIS Interconnect header section

Usage Rules: Only one [End Header] keyword is allowed per file, which must be

the last keyword in any IBIS Interconnect header section.

-----------------------------------------------------------------------------

[End Header]

=============================================================================

# 6.0 GLOBAL KEYWORDS

=============================================================================

## Keyword: [Comment Char]

Required: No

Argument: Text String

Description: Defines a new comment character to replace the default

"|" (pipe) character, if desired

Usage Rules: The new comment character to be defined must be followed by

the underscore character and the letters "char".  For example:

"|_char" redundantly redefines the comment character to be

the pipe character.  The new comment character is in effect

only following the [Comment Char] keyword.  The following

characters MAY be used:

! " # $ % & ' ( ) * , : ; < > ? @ \ ^ ` { | } ~

The [Comment Char] keyword can only appear after the

[IBIS ICM Model Ver] keyword. A file may contain multiple

[Comment Char] keywords.

-----------------------------------------------------------------------------

[Comment Char]  |_char

=============================================================================

# 7.0 INTERCONNECT MODEL FAMILY KEYWORDS

=============================================================================

## Keyword: [Begin ICM Model Family]

Required: Yes

Argument: Text String

Uses: [Manufacturer], [ICM Family Description], [ICM Model List],

[ICM Model], [End ICM Family]

Description: This keyword marks the beginning of the interconnect family

description section.  An interconnect "family" consists of one

or more similar interconnects.  Note that each individual

interconnect in a family will have a separate interconnect

model.

Usage Rules: Following the keyword is the name of the interconnect family.

Only one [Begin ICM Model Family] keyword is allowed per file,

and this keyword may appear only after the [End Header] keyword.

Interconnect family descriptions are terminated by the

occurrence of the [End ICM Family] keyword.

-----------------------------------------------------------------------------

[Begin ICM Family] High_Speed_Interconnect

=============================================================================

## Keyword: [Manufacturer]

Required: Yes

Argument: Text String

Used By: [Begin ICM Family]

Description: This keyword gives the name of the manufacturer of the

physical connectors or interconnect being described in this file.

Usage Rules: Following the keyword is the interconnect manufacturer's name.

Blank characters are allowed.

Only one [Manufacturer] keyword is allowed per file, and this

keyword may appear only between the [Begin ICM Model Family] and

[ICM Model List] keywords.

-----------------------------------------------------------------------------

[Manufacturer]  XYZ Incorporated

=============================================================================

## Keyword: [ICM Family Description]

Required: Yes

Argument: Text Block

Used By: [Begin ICM Family]

Description: Provides a human-readable description of this interconnect

family

Usage Rules: Only one [ICM Family Description] keyword is allowed per file,

and this keyword may appear only between the [Begin ICM Model

Family] and [ICM Model List] keywords.

Note: It is recommended that the family description be limited

to a maximum of 4 lines.

-----------------------------------------------------------------------------

[ICM Family Description]

High Density 0.1 center square pin connector designed for use on IEEE 99999

buses.

=============================================================================

## Keyword: [ICM Model List]

Required: Yes

Used By: [Begin ICM Family]

Description: This keyword lists the model name, mating conditions, minimum

slew time and picture link for each individual interconnect model

described in this file.

Usage Rules: The [ICM Model List] keyword may appear only once between the

[Begin ICM Model Family]/[End ICM Model Family] keyword pair, and

must be placed before the [Begin ICM Model] keyword.

Following the keyword are four columns of data, as shown in

the example below (the column headings are shown as comments

and are not required).  The "Name", "Mating" and

"Min_Slew_Time" columns are required, and all columns must be

in the left to right order shown.

The "name" column lists the model name for each interconnect

model contained in the file.  The model name must match one of

the model names given as an argument to the [Begin ICM Model]

keyword(s).

The "mating" column specifies the mating condition under which

each model's data is valid.  Valid mating choices are "Mated",

"Unmated_side_A" and "Unmated_side_B".  These choices are

defined as follows:

Mated: Both halves of a two piece interconnect are mated

together for the model.

Unmated_side_A: A model of non-mated interconnect.  Typically,

this would be the connector half that is

attached to a baseboard, or the socket which

holds the socket of an IC.

Unmated_side_B: A model of non-mated interconnect.  Typically,

this would be the connector half that is

attached to a daughter card, cable, edgecard,

or package of an IC.

Note that using two unmated models in series DOES NOT provide a

correct model of a mated interconnect.  Also, if the

interconnect model describes a package, one should always make

it "mated" since unmated packages don't have any practical use.

The Min_Slew_Time column specifies the minimum slew time (i.e.

fastest edge rate) for which each model is valid.  Slew time

is defined as the time it takes a signal to transition from

20% to 80% of its final value.  This term does not indicate

the bandwidth of the actual interconnect.  Rather, this term

is used to define a usage limit of each model.

Note: It is strongly recommended that an EDA tool issue a

warning if a signal with a lesser slew time (i.e. faster edge

rate) is applied to the model during a simulation.

The optional image column specifies an associated file that

contains an image of each interconnect.  The image file shall

be either a .jpg or .txt (ASCII) formatted picture.  The

interconnect model MUST be considered valid by EDA tools even

if an image file is specified but is missing.

If present, images should be maintained in the same directory

as the interconnect model file or stored in the same

compressed file with the interconnect to ensure that the files

remain paired.

The same image file may be repeated for all model types if

desired.

Note: it is suggested that EDA tools provide a viewer for the

.jpg format and that model builders provide at least one

picture for an interconnect.

Suggested .jpg image attributes:

size = 160 horizontal by 120 vertical (pixels)

color = 24 bit

Image quality = 85%

Larger or smaller images may be used but are not recommended.

A standard image size will allow EDA tools to provide uniform

look and feel for images from all vendors.  The size was chosen

to be a small disk file size and be useful as either a

thumb-nail in a model selection window or scaled up to give a

larger picture.  The 160x120 was chosen as a perfect 4:1 scale

in both axis of a 640x480 picture.

|----------------------------------------------------------------------------

|

[Begin ICM Model List]

|        Name             Mating      Min_Slew_Time 	Image

|------------------------------------------------------------------------------

HDI_TEST_202              Mated          100ps       HDI_TEST_202_Mated.jpg

HDI_TEST_202_UnMatedA     Unmated_side_A 100ps       HDI_TEST_202_UnMatedA.jpg

HDI_TEST_202_UnMatedB     Unmated_side_B 100ps

HDI_TEST_202_ThruHole_to_Cable Mated     25ps        HDI_TEST_202_Mated.jpg

HDI_TEST_202_SMT_to_Cable    Mated       25ps        HDI_TEST_202_Mated.jpg

HDI_TEST_202_SMT_to_ThruHole Mated       25ps        HDI_TEST_202_Mated.jpg

=============================================================================

## Keyword: [Begin ICM Model]

Required: Yes

Argument: Text String (Model name)

Used By: [Begin ICM Family]

Uses: [ICM Model Description], [Tree Path Description], [Nodal Path

Description], [ICM Swath Parameters], [End ICM Model]

Description: This keyword marks the beginning of each individual

interconnect model.

Sub-params: ICM_Model_Type, SGR, Ref_impedance

Usage Rules: Following the keyword is a single argument specifying the

name of this particular interconnect model.  The name must be

one of the model names listed under the "model name" column of

the preceding [ICM Model List] keyword.  The [Begin ICM Model]

keyword must appear after the [ICM Model List] keyword.

Note that for every model name listed by the [ICM Model List]

keyword there must be a corresponding model as defined by a

[Begin ICM Model]/[End ICM Model] keyword pair.

The [Begin ICM Model] keyword may appear only between the [Begin

ICM Model Family]/[End ICM Model Family] keyword pair, and and

must be placed after after the [ICM Model List] keyword.

Subparameters:

ICM_Model_Type

Following the keyword line is a required subparameter

ICM_Model_Type.  This subparameter defines the type of the

data (RLGC or S-parameter), and the conditions under which the

RLGC data was gathered.

The legal values for this subparameter are "SLM_general"

"SLM_quiescent", "SLM_even_mode", "SLM_odd_mode", "MLM", and

"S-parameter".

The subparameter and its value are separated by a single

whitespace.  Only one ICM_Model_Type is allowed per

interconnect model.

An ICM_Model_Type subparameter value of "SLM_quiescent" indicates

that the model data was gathered with all other neighboring

conductors non-switching and terminated by a reference impedance

as given by the optional Ref_Impedance subparameter.

A value of "SLM_even_mode" indicates that all other neighboring

conductors are transitioning at the same time and in the same

direction as the conductor being modeled (i.e. under even mode

switching conditions).  A value of "SLM_odd_mode" indicates

that all other neighboring conductors are transitioning at the

same time but in the opposite direction as the conductor being

modeled (i.e. under odd mode switching conditions).

A value of "SLM_general" indicates that the model data was

gathered with all other neighboring conductors non-switching

and the ground to signal ratio is specified by SGR and the

return path is unknown.

Note that the use of an SLM_* value indicates that this is a

"single line model" (SLM) and no electrical data is supplied

to model the coupling between pins.  Specifically, the

corresponding RLGC matrix must contain only a single diagonal

term.  SLMs can be used to evaluate for Prop Delay, bandwidth,

and impedance where ideal ground can be assumed. SLMs should

not be used when lossy return paths contributions are required

for simulations.

"SLM_quiescent", "SLM_even_mode" and "SLM_odd_mode" have a

defined reference or return path.  SLM_general does not have

specified Return Path given in the matrix.

An ICM_Model_Type value of "MLM" indicates that this model is a

"multi line model" and the electrical data suitable for

modeling pin to pin coupling is supplied.  Any combination of

Diagonal_matrix, Sparse_matrix, Banded_matrix or Full_matix may

be used in an MLM model.  In general, MLMs should not be used in

simulations with perfect ground planes on both sides of the

interconnect model.

An ICM_Model_Type value of "S-parameter" indicates that this

model is a "multi line model" and the electrical data suitable

for modeling pin to pin coupling is supplied in a frequency

domain format.  Specifically, the model is a secondary file in

"Touchstone" format.

Example:  ICM_Model_type SLM_quiescent

SGR

If the ICM_Model_Type subparameter is equal to SLM_general,

then this SGR subparameter is required.  This subparameter

specifies the signal to ground ratio used when generating the

SLM_general model.  The signal to ground ratio is indicated as

"number of signal pins per ground pin", followed by a colon (:),

followed by the number one (1).  For example, if the data for an

18 pin connector was taken with 6 of the pins grounded, the

signal to ground ratio is 18/6 or 3 to 1.  The SGR subparameter

argument would then be listed as 3:1.

This subparameter is only valid when the ICM_Model_Type sub-

parameter has a value of "SLM_general".

If this subparameter is present and the ICM_Model_Type sub-

parameter is NOT "SLM_general" a warning should be issued.

Ref_impedance

An optional subparameter that can be used to specify the

impedance of the system used to extract SLM values.  If not

given the Ref_Impedance value is assumed to be 50 ohms.  The

reference impedance for an "S-parameter" model is located in

the model file.

=============================================================================

## Keyword: [ICM Model Description]

Required: No

Argument: Text Block

Used By: [Begin ICM Model]

Description: Provides a concise yet easily human-readable description of

this interconnect model

Usage Rules: The [ICM Model Description] keyword may appear multiple times in

the file, however it may appear only once between each pair of

[Begin ICM Model]/[End ICM Model] keywords.

-----------------------------------------------------------------------------

[Begin ICM Model Description]

High Density 0.1 center square pin with PCB effects

=============================================================================

## Keyword: [Tree Path Description]

Required: Yes, if [Nodal Path Description] does not exist

Sub-params: Model_PinMap, Section, Fork, End_fork

Used By: [Begin ICM Model]

Uses: [ICM Pin Map]

Description: This keyword describes the connection path between the pin or

pins on one side of the interconnect and the corresponding set

of pins on the other side.  This pin to pin interconnect is

divided into one or more "sections", where each section

consists of RLGC and/or S-parameter data.  Sections are

generally cascaded in series, but the Fork and End_fork

subparameters allow sections to branch off the main

interconnect path as a stub or tee.

Usage Rules:  A [Tree Path Description] is used when all of the sections in a

given interconnect have one to one mapping.  These

straight through connections are relative to the matrices.  This

description is useful when all of the matrices used in each

section can be connected in a conductor by conductor fashion.

In all cases, all the RLGC matrixes must have the same number of

diagonal terms, regardless whether  they are swath or full

matrices.  Further, all diagonal terms must be in the same order.

The [Tree Path Description] keyword may appear multiple times in

the file, however it may appear only once between each pair of

[Begin ICM Model]/ [End ICM Model] keywords..

The [Nodal Path Description] and [Tree Path Description]

keywords are mutually exclusive and only one of them can be

used between a [Begin ICM Model] and [End ICM Model] keyword

pair.  However, these keywords may appear more than once and

may be used together within one file.

A path description begins with the subparameter Model_PinMap,

which points to the [ICM Pin Map] used for one end of the

interconnect.  This is followed by one or more Section

subparameters.  The electrical properties of an interconnect

are contained in the [ICM Section] matrix(s) pointed to by the

Section subparameter.  Branches or "T" connections are

indicated by the "Fork" and "End_fork" subparameters.

Finally, a path description is terminated by a Model_PinMap

subparameter.

These subparameters are described in detail below.

Model_PinMap

A path description begins with the required subparameter

Model_PinMap.  This subparameter is followed by the name

of a "pin map" which names the pins on that end of the

interconnect.  A second Model_PinMap is used at the end of the

path description to reference the pin map used for the other

end of the interconnect.  The Model_PinMap subparameter is

also used to reference a pin map for the pins at the end of

any Fork.  Note that the pin map name given as an argument

to a Model_PinMap subparameter must match the name of a pin

map defined by a subsequent [ICM Pin Map] keyword.

The argument to the Model_PinMap subparameter and the sub-

parameter itself are separated by whitespace.  There can be as

many Model_PinMap subparameters as required to represent an

interconnect, but each must be on a single line.

In general, an interconnect will have a single pin map that

is referenced at the beginning and end of a path description.

However, two different pin maps will be used if the

interconnect contains cross over pins.

Section

Following the opening Model_PinMap subparameter is one or more

instances of the required subparameter Section.  This sub-

parameter defines which [ICM Section] matrix is part of this

interconnect model.  Matrix sections are serially

interconnected and may be re-used to form a complete

interconnect model.

Arguments "multiplier" and "section name" must follow the

Section parameter.

The "multiplier" argument contains a single numeric value

which allows a matrix section to be re-used as a scaled value.

This argument can either be a multiplier factor determining

how many times the section should be repeated in series

(cascaded), or defines what its length is.  The value of the

second argument may be any positive value greater than zero.

If a section is intended to be repeated (cascaded) a certain

amount of times, the syntax of this argument is:

Mult = value

where "value" must be an integer number.

If this argument contains the length of a section, its syntax

is:

Len = value

where "value" is a floating point decimal real number.  (Note

that spaces around the "=" sign are not required).

Some sections within a particular path description can use Mult

while others use Len, if so desired.  If "Mult=" is used the

[Derivation Method] keyword for the corresponding [ICM

Section] must have a value of "Lumped" else it is an error.  If

"Len=" is used the [Derivation Method] keyword for the

corresponding [ICM Section] must have a value of "Distributed"

else it is an error.

The scale factor "Len" is applied to each parameter of all

matrices of a section resulting in scaled RLGC matrices.  When

S-parameters are used to describe a section, the usage of

scale factor "Len" is not allowed.

Mult and Len are separate arguments.  Mult and Len are not

allowed to be used in the same section.

The "section name" is the name of the section as specified under

the [Begin ICM Section] keyword.

Section may be used as many times as required to describe

the interconnect.  Each scale factor and section name MUST be

separated by at least one space.

Fork, End_fork

The Fork and End_fork subparameters are used if there

are forks or "stubs" in the interconnect being modeled.

Fork marks the beginning of the stub while End_fork

marks the end of the stub.  These subparameters do not have

any arguments.

At a fork, a "tee" section is created.  Matrix sections are

connected to the previous section and interconnected to zero

or more sections between Fork and End_fork.  If no pin

map name is placed between an Fork and End_fork block,

then the fork constitutes a stub.  However, if a pin map name

is placed at the end of the Fork and End_fork block, an

externally available port is created on the interconnect.

Any number of Fork subparameters may be used per path

description, however only one subparameter is allowed per line.

Example:  Fork

End_fork

-----------------------------------------------------------------------------

"ICM Model" Examples

-----------------------------------------------------------------------------

Example 1:  (A simple single line model with one section).

[Begin ICM Model] MyModelExample1

ICM_Model_Type SLM_general

SGR 3:1

Ref_Impedance 50

[Tree Path Description]

Model_PinMap  Example1_pinmap

Section Mult=1 Diagonal_matrix1

Model_PinMap  Example1_pinmap

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 2:  (2 ports per pin, 5 sections straight through).

SectA       SectB      SectC     SectD      SectE

Port1 >--------< >--------< >-------< >--------< >----------< Port2

[Begin ICM Model] MyModelExample2

ICM_Model_Type SLM_quiescent

[Tree Path Description]

Model_PinMap MyModelPinMapA

Section Mult=1 SectA

Section Mult=1 SectB

Section Mult=1 SectC

Section Mult=1 SectD

Section Mult=1 SectE

Model_PinMap MyModelPinMapB

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 3: (2 ports per pin, 2 series sections with one stub).

SectionA          SectionB

Port1 >------------< >--------------< Port2

|

StubSection1

[Begin ICM Model] MyModelExample3

ICM_Model_Type MLM

[Tree Path Description]

Model_PinMap MyModelPinMapA

Section Mult=1 SectionA

Fork

Section Mult=1 StubSection1

End_fork

Section Mult=1 SectionB

Model_PinMap MyModelPinMapB

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 4:  (2 ports per pin, 2 stub sections, one stub matrix is re-used,

and another stub is repeated).

A            B            C

Port1 >----------< >-----------< >-----------< Port2

|stub1        |stub4

|stub2        |stub4

|stub1

[Begin ICM Model] MyModelExample4

ICM_Model_Type MLM

[Tree Path Description]

Model_PinMap MyModelPinMapA

Section Mult=1 A

Fork

Section Mult=1 stub1

Section Mult=1 stub2

Section Mult=1 stub1

End_fork

Section Mult=1 B

Fork

Section Mult=2 stub4

End_fork

Section Mult=1 C

Model_PinMap MyModelPinMapB

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 5:  (2 ports per pin, 3 stubs).

A        B        C         D

Port1 >------< >------< >------< >-------< Port2

|Stub1   |Stub2   |Stub1

|Stub3

|Stub4

[Begin ICM Model] MyModel4

ICM_Model_Type MLM

[Tree Path Description]

Model_PinMap MyModelPinMapA

Section Mult=1 A

Fork

Section Mult=1 Stub1

End_fork

Section Mult=1 B

Fork

Section Mult=1 Stub2

Section Mult=1 Stub3

Section Mult=1 Stub4

End_fork

Section Mult=1 C

Fork

Section Mult=1 Stub1

End_fork

Section Mult=1 D

Model_PinMap MyModelPinMapB

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 6:  (3 ports per pin, 2 stubs).

A        B        C         D

Port1 >------< >------< >------< >-------< Port 2

|Stub1   |Stub2   |Stub1

|Stub3

|Stub4

|Port3

[Begin ICM Model] MyModelExample6

ICM_Model_Type MLM

[Tree Path Description]

Model_PinMap MyModelPinMapA

Section Mult=1 A

Fork

Section Mult=1 Stub1

End_fork

Section Mult=1 B

Fork

Section Mult=1 Stub2

Section Mult=1 Stub3

Section Mult=1 Stub4

Model_PinMap MyModelPinMapC

End_fork

Section Mult=1 C

Fork

Section Mult=1 Stub1

End_fork

Section Mult=1 D

Model_PinMap MyModelPinMapB

.

.

.

[End ICM Model]

-----------------------------------------------------------------------------

Example 7:  (3 ports per pin, 5 sections straight through).

SectionA     SectionB    SectionC     SectionD     SectionE

Port1 >----------< >----------< >---------< >----------<|>------------< Port2

|  SectionF

|>------------< Port3

[Begin ICM Model] MyModelExample7

ICM_Model_Type SLM_quiescent

[Tree Path Description]

Model_PinMap = MyModelPinMapA

Section Mult=1 SectionA

Section Mult=1 SectionB

Section Mult=1 SectionC

Section Mult=1 SectionD

Fork

Section Mult=1 SectionF

Model_PinMap MyModelPinMapB

End_fork

Section Mult=1 SectionE

Model_PinMap MyModelPinMapC

.

.

.

[End ICM Model]

=============================================================================

## Keyword: [Nodal Path Description]

Required: Yes, if [Tree Path Description] does not exist

Sub-params: N_section, Model_NodeMap

Used By: [Begin ICM Model]

Uses: [ICM Node Map]

Description: This keyword marks the beginning of a nodal path description.

It contains the information on how two or more nodes are

connected.  The nodal description style allows for any

arbitrary topologies to be described.

Nodes define the connection points where the conductors of two

or more sections are joined together, or where the path is

interfaced to the outside world.  Please note that single

connections (dangling ends) at nodes are also permitted.  Thus

nodes can represent any connection points, including pins (on

either side of connectors, packages, or sockets), pads and

bumps of a die (where the connections to the package or

printed circuit board are made), or the ends of either side of

interconnect sections of printed circuit boards.

An interconnect path may have one or more sections where each

section is described by RLGC, or S-parameter matrices.

Multiple sections within a path description may be connected

in any arbitrary manner by using the usual node naming

techniques.  Please note that the swath features cannot be

used with this keyword.

Usage Rules: The [Nodal Path Description] keyword may appear multiple times in

the file, however it may appear only once between each pair of

[Begin ICM Model]/ [End ICM Model] keywords.

The [Nodal Path Description] and [Tree Path Description]

keywords are mutually exclusive and only one of them can be

used between a [Begin ICM Model] and [End ICM Model] keyword

pair.  However, these keywords may appear more than once and

may be used together within one file.

Following the [Nodal Path Description] keyword line are the

subparameters that describe the topology and electrical

characteristics of the pin to pin connections through the

interconnect.

Path description sections are terminated by the occurrence

of the next keyword.

The [Nodal Path Description] keyword line is followed by at

least one Model_NodeMap and one N_section subparameters which

describe the pin to node mapping, and the topology of the

interconnect.  The order in which the Model_NodeMap and

N_section subparameters are listed in a path description is

irrelevant.  However, for visual clarity it is a good practice

to follow the physical layout of the interconnect with the

order in which they are listed.  The electrical properties of

an interconnect section are described in the matrices under

the [Begin ICM Section] keyword which are referenced by the

N_section subparameter(s), discussed below.

Connections between the nodes of the various N_sections,

branching or T-ing are achieved using the usual node naming

techniques.

Detailed descriptions of these subparameters follow below.

N_section

The N_section subparameter is used to describe the

connectivity and electrical characteristics of an interconnect

segment in the path description.

A path description must have at least one occurrence of the

required subparameter N_section, but can have as many

N_sections as needed to adequately describe an interconnect.

Each N_section subparameter must start on a separate line and

must end on the line on which the closing parentheses are

found.  (This allows long node lists to span over multiple

lines without causing parsing problems).

The subparameter name N_section is followed by three

arguments, separated by at least one white space.

The first argument is a list of nodes and must be enclosed in

parentheses.  The length of each node name cannot exceed 20

characters.  The node names within the parentheses are

separated by at least one white space.  The allowable

characters for node names are a-z, A-Z, 0-9, and the

underscore character "_".

There must be an even number of nodes in a node list when

using an RLGC matrix to describe a section.  There will be

half as many rows in an RLGC matrix as there are nodes.  When

an N_section is described by S-parameter matrices an odd

number of nodes is allowed in the node list.

The order of the node names in the first half of the node

list must match the order of the rows in the corresponding

RLGC matrix data, or the order of the node names in the node

list must be the same as the order of the rows in the

corresponding S-parameter matrix.

A port for the S-parameter data is defined between a node and

the ideal ground.

Matching node names inside a path description section are

assumed to be connected with an ideal short.  Nodes having the

same names in different [Nodal Path Description]s are assumed

to be not connected.

The second argument contains a single numeric value which

allows a matrix section to be re-used as a scaled value.  This

argument can either be a multiplier factor determining how

many times the section should be repeated in series

(cascaded), or defines what its length is.  The value of the

second argument may be any positive value greater than zero.

If a section is intended to be repeated (cascaded) a certain

amount of times, the syntax of this argument is:

Mult = value

where "value" must be an integer number.

If this argument contains the length of a section, its syntax

is:

Len = value

where "value" is a floating point decimal real number.  (Note

that spaces around the "=" sign are not required).

Mult and Len are separate arguments.  Mult and Len are not

allowed to be used in the same section.

Some sections within a particular path description can use M

while others use Len, if so desired.  If "Mult=" is used the

[Derivation Method] keyword for the corresponding [ICM

Section] must have a value of "Lumped" else it is an error.  If

"Len=" is used the [Derivation Method] keyword for the

corresponding [ICM Section] must have a value of "Distributed"

else it is an error.

The scale factor "Len" is applied to each parameter of all

matrices of a section resulting in scaled RLGC matrices.  When

S-parameters are used to describe a section, the usage of

scale factor "Len" is not allowed.

The third argument contains the name of the N_section which

must match the name of a subsequent [Begin ICM Section]

keyword.  The electrical characteristics of sections are

described in the form of RLGC or S-parameter matrices under

the [Begin ICM Section] keyword(s).  A [Begin ICM Section]

keyword may be referenced by more than one N_section

subparameter even if they are in different path descriptions.

Model_NodeMap

The required subparameter Model_NodeMap is used to reference

the [Node Map] keyword which provides a mapping between the

actual data book pin (or signal) names and the node names used

in the path description.

A path description must have at least one occurrence of the

required subparameter Model_NodeMap, but can have as many

Model_NodeMap subparameters as needed to adequately describe

the interconnect.  The number of Model_NodeMap subparameters

used in a path description would be typically the same as the

number of ends or sides in a connector or interconnect.  Each

Model_NodeMap subparameter must be on a single line.

The Model_NodeMap subparameter is followed by one argument,

which the name of a node map.  The argument must match the

name of a node map defined by a subsequent [ICM Node Map]

keyword.  The argument and the subparameter must be separated by

at least one white space.

-----------------------------------------------------------------------------

Please note that the following examples can be easily applied to connectors

as well as cable assemblies, packages and printed circuit boards.

-----------------------------------------------------------------------------

Example 1:

A simple model for a DB9 male to female adapter with a straight through

wiring modeled as one section.  The pin numbering increases left to right

when facing the male side, and right to left when facing the female side.

Note that this connector could have been also described by the [Path

Description] keyword due to its regularity and the one to one mapping.

1   2   3   4   5

6   7   8   9       Male side

| | | | | | | | |

| | | | | | | | |

| | | | | | | | |

1   2   3   4   5     Female side

6   7   8   9

[Begin ICM Model] DB9M_F

ICM_Model_Type MLM

[Nodal Path Description]

Model_NodeMap DB9_Male_side

N_section (M1 M2 M3 M4 M5 M6 M7 M8 M9

F1 F2 F3 F4 F5 F6 F7 F8 F9) Mult=1 RLGC_matrix_9x9

Model_NodeMap DB9_Female_side

[End ICM Model]

-----------------------------------------------------------------------------

Example 2:

This is a model of the same adapter as in example 1, except that both sides

are male which results in a crossed pinout.  Note that this connector could

have been also described by the [Tree Path Description] keyword due to its

regularity and the one to one mapping.

1   2   3   4   5

6   7   8   9       Male side A

| | | | | | | | |

| | | | | | | | |

| | | | | | | | |

5   4   3   2   1     Male side B

9   8   7   6

[Begin ICM Model] DB9M_M

ICM_Model_Type MLM

[Nodal Path Description]

Model_NodeMap DB9_side_A

N_section (A1 A2 A3 A4 A5 A6 A7 A8 A9

B5 B4 B3 B2 B1 B9 B8 B7 B6) Mult=1 RLGC_matrix_9x9

Model_NodeMap DB9_side_B

[End ICM Model]

-----------------------------------------------------------------------------

Example 3:

This is a model of a PS2 6-pin female mini DIN to 5-pin male DIN adapter.

The model consist of three sections to provide a more detailed model.

The drawing of this adapter was found on the following web site:

http://www.grand-tek.com/SPEC-7/GM06F-R05MM0-04.pdf

1 2 3 4 5 6 shell   PS2 6-pin female mini DIN

| | | | | |   |     Section for connector pins

|   | | |     |     Section for wiring

| | | | |     |     Section for connector pins

2 3 4 5 1   shell   5-pin male DIN

[Begin ICM Model] PS2_6_DIN_5

ICM_Model_Type MLM

[Nodal Path Description]

Model_NodeMap PS2_6_DIN_5_PS2_side

N_section ( PS1  PS2  PS3  PS4  PS5  PS6  PS7

n11  n12  n13  n14  n15  n16  n17) Mult=1 RLGC_matrix_7x7

N_section ( n11       n13  n14  n15       n17

n21       n23  n24  n25       n27) Mult=1 RLGC_matrix_5x5

N_section ( n21  n22  n23  n24  n25       n27

DIN2 DIN3 DIN4 DIN5 DIN1      DIN7) Mult=1 RLGC_matrix_6x6

Model_NodeMap PS2_6_DIN_5_DIN_side

[End ICM Model]

-----------------------------------------------------------------------------

Example 4:

This is a model of a PS2 keyboard Mouse Y-splitter adapter.  The model

consist of only one section for simplicity, but includes connections between

the shells (not shown on the drawing of the web site).  The specifications

for this adapter were found on the following web site:

http://www.hardwarebook.net/adapter/userinput/ps2keyboardygateway.html

1 2  3   4  5 6 shell       6-pin male mini DIN (computer)

| | +-+ +-+ | |   +---+

| | | | | | | |   |   |     Section

2 | 3 | 4 | 6 | shell |     6-pin female DIN (keyboard)

2   3   4   6     shell   6-pin female DIN (mouse)

[Begin ICM Model] PS2_splitter

ICM_Model_Type MLM

[Nodal Path Description]

Model_NodeMap PS2_splitter_Computer_side

N_section (C1 C3 C4 C5 Cs C2 C3 C4 C6 Cs

K2 K3 K4 K6 Ks M2 M3 M4 M6 Ms) Mult=1 RLGC_matrix_10x10

Model_NodeMap PS2_splitter_Keyboard_side

Model_NodeMap PS2_splitter_Mouse_side

[End ICM Model]

-----------------------------------------------------------------------------

Example 5:

2 ports per pin, 5 sections straight through.  Note that this interconnect

could have been also described by the [Tree Path Description] keyword due to its

regularity and the one to one mapping.

SectA     SectB     SectC     SectD     SectE

Port_A >-------< >-------< >-------< >-------< >-------< Port_B

[Begin ICM Model] MyModelExample5

ICM_Model_Type SLM_quiescent

[Nodal Path Description]

Model_NodeMap Port_A

N_section (A1 A2 A3 A4  11 12 13 14) Len=1.0 SectA

N_section (11 12 13 14  21 22 23 24) Len=1.0 SectB

N_section (21 22 23 24  31 32 33 34) Len=1.0 SectC

N_section (31 32 33 34  41 42 43 44) Len=1.0 SectD

N_section (41 42 43 44  B1 B2 B3 B4) Len=1.0 SectE

Model_NodeMap Port_B

[End ICM Model]

-----------------------------------------------------------------------------

Example 6:

2 ports per pin, 2 series sections with one stub that taps into line_2 and

line_4.

SectionA         SectionB

Port_A1 >----------<----->----------< Port_B1

Port_A2 >----------<---o->----------< Port_B2

Port_A3 >----------<---+->----------< Port_B3

Port_A4 >----------<-o-+->----------< Port_B4

Port_A5 >----------<-+-+->----------< Port_B5

| |

| |

StubSection1

[Begin ICM Model] MyModelExample6

ICM_Model_Type MLM

[Nodal Path Description]

N_section   (A1 A2 A3 A4 A5  11 12 13 14 15) Len=1.0 SectionA

N_section (   12    14        x2    x4   ) Len=1.0 StubSection1

N_section   (11 12 13 14 15  B1 B2 B3 B4 B5) Len=1.0 SectionB

[End ICM Model]

-----------------------------------------------------------------------------

Example 7:

2 ports per pin, 2 stub sections, one stub matrix is re-used, and another

stub is repeated.

A           B           C

Port_A >---------< >---------< >---------< Port_B

|stub1      |stub4

|stub2      |stub4

|stub1

|Port_C

[Begin ICM Model] MyModelExample7

ICM_Model_Type MLM

[Nodal Path Description]

N_section   (A1 A2 A3 A4 A5  11 12 13 14 15) Len=1.0 A

N_section (11 12 13 14 15  s1 s2 s3 s4 s5) Len=1.0 stub1

N_section (s1 s2 s3 s4 s5  t1 t2 t3 t4 t5) Len=1.0 stub2

N_section (t1 t2 t3 t4 t5  C1 C2 C3 C4 C5) Len=1.0 stub1

N_section   (11 12 13 14 15  21 22 23 24 25) Len=1.0 B

N_section (21 22 23 24 25  x1 x2 x3 x4 x5) Mult=2   stub4

N_section   (21 22 23 24 25  B1 B2 B3 B4 B5) Len=1.0 C

[End_ICM_Model]

=============================================================================

## Keyword: [ICM Swath Parameters]

Required: No, unless an interconnect is described by a swath.

Sub-Params: Left_edge, Right_edge, Top_edge, Bottom_edge

Used By: [Begin ICM Model]

Uses: [ICM Swath Pin Numbers]

Description: Describes a smaller matrix which can be used to represent

interconnects of equal or larger size.  To visualize this,

consider the swath matrix being a smaller version of a

connector.  The edges of the matrix represent the same edge

effects that exists in the full size connector.  The center

area of the matrix represents the same effects as the center

of the full size connector.

It is the responsibility of the EDA tool to use the matrix and

the required keyword information as a "mini" interconnect that

is centered about the electrical paths of interest and/or to

expand this information into a larger sized swath up to the

full sized interconnect itself.  For example a 2x8 matrix might

be expanded to represent a connector that is 2x100.

Usage Rules: Only rectangular interconnects without missing conductor may be

represented by a swath matrix.  If the interconnect is non-

rectangular or the pin field has missing pins then the matrix

must describe the full interconnect and this keyword cannot be

used.  This keyword MUST be present if the number of rows in

the matrix does not equal the number of pin names as given by

the [ICM Pin Map] keyword used for this model.

The [ICM Swath Parameters] keyword may appear multiple times in

the file, however it may appear only once between each pair of

[Begin ICM Model]/ [End ICM Model] keywords.  This keyword is

only required when using only a part of the matrix or when

expanding the matrix.

MUST be followed by 4 required subparameters:

1) Left_edge:   Columns in the swath matrix used to represent

left edge

2) Right_edge:  Columns in the swath matrix used to represent

right edge

3) Top_edge:    Rows in the swath matrix used to represent

top edge

4) Bottom_edge: Rows in the swath matrix used to represent

bottom edge

These subparameters use the physical representation of the

pins to define the minimum coupling bandwidth.

The model maker decides on the minimum number of columns (for

a column swath) or rows (for a row swath) that should be used

to define the banded matrix, then assigns values to the

appropriate subparameter that will, by subtraction of the

columns or rows, create the appropriate maximum matrix

size for the swath.

The basic goal is to define how far the edge effects extend

into the coupling fields.

None of the values of these numbers may exceed the number of

rows or column of pins that are defined by the matrices of a

given model.  The swath is applicable to any model supported

by the specification.

At this time, there must be at least two subparameters with

a value not equal to zero.

Please see the example below to see how these are used with

the keyword.

The edge effects describe the edge rows or columns that do

not contain all of the coupling effects (from both sides)

that the centermost rows or columns.  This information is

useful for "swath expansion" when determining what larger

size to use.

When the swath represents a SLM or else documents all of the

pins in a physical interconnect, the off diagonal entries are

0 by convention.  If the swath describes a section with fewer

columns, but the same number of rows as the physical

interconnect, then the row edge effects are 0 by convention.

Similarly, if the swath describes a section with fewer rows,

but the same number of columns as the physical interconnect,

then the column edge effects are 0 by convention.

In a fully coupled swath where all pins couple to all

others, all of the rows or columns except for the one

designated as the centermost row or column can have edge

simplification.  So the number of rows or columns can

extend to all but the center row or column.  On the other

hand, a coupling pattern that extends over a few columns may

result in a smaller number of edges designated.  Below is an

example of a 3 row by 5 column swath, where the lines in a

rectangular connector represent the electrical coupling:

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

Notice that columns 2, 3, and 4 have the maximum amount of

coupling from each side.  In this case the edge effect

columns 1 and 5 do not have coupling from both sides.

Application of Swath Matrices:

For the purposes of this discussion a 3 row by 5 column

swath is used and the full sized interconnect is 3 rows by 9

columns.   The swath matrix data can be organized by

[ICM Swath Parameters].  Three methods can be used to apply

the Swath matrix information:

(1) Centering the Swath around the pins of interest

1  2  3  4  5

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|  Swath

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

*--*--*--*--*--*--*--*--*

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*   Full Interconnect

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*

1  2  3  4  5  6  7  8  9

The Swath above is centered about column 4.  The paths of

interest including those of columns 3 and 5 are also

described by swath matrix data that has coupling from both

sides.  However, because of edge effects, columns 2 and 6

are less accurately approximated.

Centering is a way to map the swath matrix pins directly

into the corresponding pins of the actual interconnect.

When the signals of interest are at the left or right

edges, the swath would normally be positioned just to the

edges.  For example, column 1 of the swath matrix would

line up with column 1 of the interconnect.

(2) Expansion and Centering: Expanding the Swath matrix into a

larger sized Swath matrix and centering it about the paths

of interest.

This approach is similar to centering.  However, the

signal pins of interest might have more columns than the

non-edge section of the swath.  One solution is to expand

the swath to a larger swath and then use the larger swath

for centering.  This is a subset of expanding the swath

into the full sized interconnect.

A larger swath might be constructed to span the number of

columns of interest plus the edge effect columns to assure

that the most accurate coupling information from both

sides is used for the columns of interest.

(3) Expansion to Full Sized Interconnect:  Expanding the swath

matrix to the full sized interconnect matrix.

There are three steps to this procedure (a, b, and c

below).  This is illustrated using the 3 row, 5 column

swath and showing how it is expanded into a 3 row, 9

column full sized interconnect.

(a) Copy the left column pattern

1  2  3  4  5

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|   Swath

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

*--*--*--*--*--*--*--*--*

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*   Full Interconnect

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*

1  2  3  4  5  6  7  8  9

The matrix data for swath columns 1 and 2 (including

forward references) copied into the new full

interconnect matrix.

(b) Repeat the center pattern,

1  2  3  4  5

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|   Swath

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

*--*--*--*--*--*--*--*--*

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*   Full Interconnect

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*

1  2  3  4  5  6  7  8  9

Starting at the center of the swath, the coupling pattern

of just the center column (column 3 in this example) is

copied for the interconnect matrix.  In this case, the

coupling pattern extends forward to the next column.  The

pattern is copied by mapping the swath pin numbers into

the interconnect pin numbers and copying the center

column forward referenced data.  In this example, the

operation starts at the full interconnect column 3 and

ends upon completion at the full interconnect column 7.

(c) Copy the right column patterns

1  2  3  4  5

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|   Swath

*--*--*--*--*

|\/|\/|\/|\/|

|/\|/\|/\|/\|

*--*--*--*--*

*--*--*--*--*--*--*--*--*

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*   Full Interconnect

|\/|\/|\/|\/|\/|\/|\/|\/|

|/\|/\|/\|/\|/\|/\|/\|/\|

*--*--*--*--*--*--*--*--*

1  2  3  4  5  6  7  8  9

The final step is similar to Step (a), but concluded

using the right side of the swath.  When the swath is

positioned against the right edge as shown, then the

coupling patterns for the swath columns are duplicated

for the final columns of the full sized interconnect.  So

the patterns of swath columns 4 and then 5 are duplicated

for the full sized interconnect columns 8 and 9.

A similar process could be created for expanding the number of

rows.  Some details are left to the EDA tool to implement.

For example, if the swath contains an even number of columns,

then either the left of center or the right of center column

may be used for center section expansion.

-----------------------------------------------------------------------------

[ICM Swath Parameters]

Left_edge = 1

Right_edge = 1

Top_edge = 0

Bottom_edge = 0

==============================================================================

## Keyword: [ICM Swath Pin Numbers]

Required: Yes, if [ICM Swath Parameters] is present, else it must not be

used.

Used By: [ICM Swath Parameters]

Description: Lists the pins of the matrix in the exact order that the

swath matrix rows are entered and gives the corresponding

physical location of the pin

Usage Rules: The [ICM Swath Pin Numbers] keyword is followed by a list of

entries that apply for all of the sections as referenced by the

[Tree Path Description] keyword.

The number of entries in the list is equal to the product of

the rows and columns of the [ICM Swath Pin Numbers] keyword.

The [ICM Swath Pin Numbers] keyword may appear multiple times in

the file, however it may appear only once between each pair of

[Begin ICM Model]/ [End ICM Model] keywords.

Each line must contain three columns documenting the row

names and physical row and column locations for each entry of

the matrices under the corresponding [Begin ICM Section]

keyword.  The entries consist of [Row] name designated

row_name, physical row number designated physical_row, and

physical column number designated physical_column.  All three

columns are required and must be separated by at least one

white space.

The physical row number consists of an integer value from 1 to

the number of rows documented in the [ICM Swath Pin Numbers]

keyword.  Similarly, the physical column number consists of an

integer value from 1 the number of columns documented in the

[ICM Swath Pin Numbers] keyword.

These physical row and column numbers give the relative

physical location of each of the swath matrix elements as if

they were the pin numbers of the interconnect itself.  Only one

unique physical row and physical column number pair are allowed

in the list.  In other words, no two or more row_name entries

shall refer to the same physical_row, physical_column pair.

Other Notes: It is required that all of the sections that are referenced

by the [Begin ICM Section] keyword have swath matrix entries

that are ordered in the same manner.  This would be expected

from the field solver working on a single interconnect swath,

but issuing its results as separate sections.

The EDA tool that processes this keyword has enough information

to reorder the swath matrix, if needed, to position its

electrical contents among the pins of interest in the actual

interconnect.

--------------------------------------------------------------------------

Consider a 2 row by 3 column Swath matrix that was extracted for the pins

as shown below:

A B C

D E F

The field extractor may have produced matrices ordered:  C E B F A D

[Swath Pin Numbers]

| row_name     physical_row    physical_column

C            1               3

E            2               2

B            1               2

F            2               3

A            1               1

D            2               1

=============================================================================

## Keyword: [End ICM Model]

Required: Yes

Used By: [Begin ICM Model]

Description: Defines the end of the interconnect models for the

interconnect family

Usage Rules: The [End ICM Model] keyword may appear only between the [Begin

ICM Model Family]/[End ICM Model Family] keyword pair.  Every

[Begin ICM Model] keyword must have a corresponding [End ICM

Model] keyword, which must be the last keyword of an ICM Model

section.

-----------------------------------------------------------------------------

[End ICM Model]

=============================================================================

## Keyword: [ICM Pin Map]

Required: No, unless the [Tree Path Description] keyword exists.

Argument: Text String (Model_PinMap name)

Sub-params: Pin_order, Num_of_columns, Num_of_rows

Used By: [Tree Path Description]

Description: This keyword gives the names of the pins on one side of an

interconnect, then defines a pin ordering such that pins can

be associated with rows in the RLGC or S-parameter section

matrixes.  This keyword also defines whether the pins are listed

in column ordered or row ordered fashion.

Usage Rules: Following the keyword is the name of this pin map.  The pin map

name must match the pin map name used as an argument to one of

the Model_PinMap subparameters in a model's [Tree Path

Description] keyword.

The [ICM Pin Map] keyword may appear multiple times in the file,

however it may appear only between the last [End ICM Model] and

before the [End ICM Model Family] keywords.

Following the [ICM Pin Map] line are three subparameters:

Pin_order, Num_of_rows, Num_of_columns.  Each subparameter is

placed on a seperate line, and when used they must appear in

the above order.  Following the subparameters is a list of pin

names that are covered by this pin map.

Subparameters and the pin list are described in detail below.

Pin_order

The Pin_order subparameter is required.  Legal arguments to the

Pin_order subparameter are Row_ordered, Column_ordered or Un-

ordered.  The subparameter name and its argument are seperated

by one or more white spaces.

Row_ordered and Column_ordered refers to the order in which

the pins in the accompanyng pin list are listed.  Row_ordered

means that all the pins in the top row of an interconnect are

listed in order, then the second row of pins is listed, and so

on.  Likewise, Column_ordered means that all the pins in the

first column of an interconnect are listed, then the second

column, and so forth.

If the Row_ordered or Column_ordered argument is specified then

the Num_of_rows and Num_of_columns subparameters are also

required.

If the Un-ordered argument is specified, the pins are not

listed in row or column order and the Num_of_rows and

Num_of_columns subparameters must not be used.  Un-ordered also

implies that the interconnect is not swathable.

Num_of_columns

This optional subparameter specifies the number of columns that

are in the physical interconnect.  The subparameter name is

followed by a single positive integer.

Num_of_rows

This optional subparameter specifies the number of rows that

are in the physical interconnect.  The subparameter name is

followed by a single positive integer.

Note that the pin order and number information are required in

order for EDA software to properly position and expand a swath

matrix around a given pin.

Pin List

Following the subparameters is a list of the pin names covered

by this pin map.  The pin list is required.  Each pin name is

listed on a separate line on the file.  The pin names must be

organized as specified by the Pin_order subparameter. There is a

second column which contains a name for the pin

IMPORTANT NOTE:  The intent of the [ICM Pin Map] keyword pin

order information is to give an EDA tool enough information to

map pin names to the rows of an interconnect's section matrix.

In the case of a full-matrix/full-pinout description of an

interconnect, it is required that the rows of a section matrix

appear in the same row or column order as the pin names.  For

example, suppose the pins names of a six pin connector were

listed as follows:

A1  SIG

A2  SIG

A3  SIG

B1  SIG

B2  SIG

B3  GND

A six pin connector can be described by a 6x6 matrix.  It is

required that the first row of the matrix contain the data for

pin A1, the second row contain the data for pin A2, and so on.

Other Notes: More than one [ICM Pin Map] keyword is allowed in a

file.

This keyword must occur outside the [Begin ICM Model]/[End ICM

Model] keyword pair.  This is to allow a [ICM Pin Map] to be

referenced by multiple interconnect models.

“GND” is a reserved word.  The use of “GND” is required for

models designated as “SLM_quiescent", "SLM_even_mode" and

"SLM_odd_mode".  For SLM_general and MLMs “GND” cannot be used.

“GND” is optional for model type s-parameter.

All diagonal values in any of the parametric matrix positions

that are referenced as “GND” must contain the value of “0”.  See

the “Matrix Description” section of this document for further

information.

The examples below use the connector represented by:

A1 A2 A3 A4

B1 B2 B3 B4

-----------------------------------------------------------------------------

[ICM Pin Map] Baseboard_side

Pin_order = row_ordered

Num_of_columns = 4

Num_of_rows = 2

|Pin  Type

A1  SIG

A2  SIG

A3  SIG

A4  SIG

B1  SIG

B2  SIG

B3  SIG

B4  SIG

[ICM Pin Map] Baseboard_side2

Pin_order = column_ordered

Num_of_columns = 4

Num_of_rows = 2

|Pin

A1 SIG

B1 SIG

A2 SIG

B2 SIG

A3 SIG

B3 SIG

A4 SIG

B4 SIG

[ICM Pin Map] Daughtecard_side

Pin_order = Un_ordered

|Pin

A1 SIG

A2 SIG

A3 SIG

A4 SIG

B1 SIG

B2 SIG

B3 SIG

B4 SIG

=============================================================================

## Keyword: [ICM Node Map]

Required: No, unless the [Nodal Path Description] keyword exists.

Argument: Text String (Model_NodeMap name)

Used By: [Nodal Path Description]

Description: This keyword is used to provide a mapping between the actual

data book pin names and the node names used in the path

description.  This may be useful, for example, when the data

book pin names are identical on either side of a connector, or

when only a signal name is available for a conductor on

package or BCP, because the node names on the two ends of these

interconnects must be unique in the path description (to

prevent shorting the pins of the two sides).  Typically there

would be as many Model_NodeMap subparameters in a path

description as there are ends or sides.

Usage Rules: Following the keyword is the name of this node map.  The node

map name must match the node map name used as an argument to

one of the Model_NodeMap subparameters in a model's [Nodal

Path Description] keyword.

The [ICM Node Map] keyword may appear multiple times in the file,

however it may appear only between the last [End ICM Model] and

before the [End ICM Model Family] keywords.

The [ICM Node Map] line is followed by a two-column table.  The

first column contains the list of pin or signal names and the

second column contains the corresponding node names.  The node

names listed here must match the node names by the N_section

subparameter(s) under the [Nodal Path Description] keyword to

make a connection.

Note that the first columns (i.e. the pin or signal names) of

different [ICM Node Map] keywords may be identical in certain

situations.  This does not result in an electrical short

between those pins or signals even if these [ICM Node Map]

keywords are called by the Model_NodeMap subparameters of the

same [Nodal Path Description] keyword, because it is the

associated node names which make the electrical connections.

Also, the node names defined in the [ICM Node Map] keyword are

not required to be nodes having a single connection only in the

path description.

There is a third column, which contains a name for the pin.

“GND” is a reserved word.  The use of “GND” is required for

models designated as “SLM_quiescent", "SLM_even_mode" and

"SLM_odd_mode".  For SLM_general and MLMs “GND” cannot be used.

“GND” is optional for model type s-parameter.

All diagonal values in any of the parametric matrix positions

that are referenced as “GND” must contain the value of “0”.  See

the “Matrix Description” section of this document for further

information.

Other Notes: More than one [ICM Node Map] keyword is allowed in a

file.

This keyword must occur outside the [Begin ICM Model] / [End

ICM Model] keyword pair.  This is to allow a [ICM Node Map] to

be referenced by multiple interconnect models.

[ICM Node Map] sections are terminated by the occurrence of the

next keyword.

-----------------------------------------------------------------------------

The following examples are to be used with the examples following the

description of the [Nodal Path Description] keyword.

-----------------------------------------------------------------------------

Example 1:

[ICM Node Map] DB9_Male_side

|  pin    node   name

1     M1     SIG

2     M2     SIG

3     M3     SIG

4     M4     SIG

5     M5     SIG

6     M6     SIG

7     M7     SIG

8     M8     SIG

9     M9     SIG

[ICM Node Map] DB9_Female_side

|  pin    node

1     F1     SIG

2     F2     SIG

3     F3     SIG

4     F4     SIG

5     F5     SIG

6     F6     SIG

7     F7     SIG

8     F8     SIG

9     F9     SIG

-----------------------------------------------------------------------------

Example 2:

[ICM Node Map] DB9_side_A

|  pin    node

1     A1     SIG

2     A2     SIG

3     A3     SIG

4     A4     SIG

5     A5     SIG

6     A6     SIG

7     A7     SIG

8     A8     SIG

9     A9     SIG

[ICM Node Map] DB9_side_B

|  pin    node

1     B1     SIG

2     B2     SIG

3     B3     SIG

4     B4     SIG

5     B5     SIG

6     B6     SIG

7     B7     SIG

8     B8     SIG

9     B9     SIG

-----------------------------------------------------------------------------

Example 3:

[ICM Node Map] PS2_6_DIN_5_PS2_side

|  pin    node

1     PS1    SIG

2     PS2    SIG

3     PS3    SIG

4     PS4    SIG

5     PS5    SIG

6     PS6    SIG

shell PS7    SIG

[ICM Node Map] PS2_6_DIN_5_DIN_side

|  pin    node

1     DIN1   SIG

2     DIN2   SIG

3     DIN3   SIG

4     DIN4   SIG

5     DIN5   SIG

shell DIN7   SIG

-----------------------------------------------------------------------------

Example 4:

[ICM Node Map] PS2_splitter_Computer_side

|  pin    node

1     C1     SIG

2     C2     SIG

3     C3     SIG

4     C4     SIG

5     C5     SIG

6     C6     SIG

shell Cs     SIG

[ICM Node Map] PS2_splitter_Keyboard_side

|  pin    node

2     K2     SIG

3     K3     SIG

4     K4     SIG

6     K6     SIG

shell Ks     SIG

[ICM Node Map] PS2_splitter_Mouse_side

|  pin    node

2     M2     SIG

3     M3     SIG

4     M4     SIG

6     M6     SIG

shell Ms     SIG

-----------------------------------------------------------------------------

Example 5:

[ICM Node Map] Port_A

|   pin         node

Port_A1     A1     SIG

Port_A2     A2     SIG

Port_A3     A3     SIG

Port_A4     A4     SIG

[ICM Node Map] Port_B

|   pin         node

Port_B1     B1     SIG

Port_B2     B2     SIG

Port_B3     B3     SIG

Port_B4     B4     SIG

-----------------------------------------------------------------------------

Example 6:

[ICM Node Map] Port_A

|   pin         node

Port_A1     A1     SIG

Port_A2     A2     SIG

Port_A3     A3     SIG

Port_A4     A4     SIG

Port_A5     A5     SIG

[ICM Node Map] Port_B

|   pin         node

Port_B1     B1     SIG

Port_B2     B2     SIG

Port_B3     B3     SIG

Port_B4     B4     SIG

Port_B5     B5     SIG

-----------------------------------------------------------------------------

Example 7:

[ICM Node Map] Port_A

|   pin         node

Port_A1     A1     SIG

Port_A2     A2     SIG

Port_A3     A3     SIG

Port_A4     A4     SIG

Port_A5     A5     SIG

[ICM Node Map] Port_B

|   pin         node

Port_B1     B1     SIG

Port_B2     B2     SIG

Port_B3     B3     SIG

Port_B4     B4     SIG

Port_B5     B5     SIG

[ICM Node Map] Port_C

|   pin         node

Port_C1     C1     SIG

Port_C2     C2     SIG

Port_C3     C3     SIG

Port_C4     C4     SIG

Port_C5     C5     SIG

=============================================================================

## Keyword: [End ICM Family]

Required: Yes

Used By: [Begin ICM Family]

Description: Defines the end of the interconnect model family

Usage Rules: Every [Begin ICM Model] keyword must have a corresponding [End

ICM Model] keyword, which must be the last keyword of an ICM

Model Family section.

-----------------------------------------------------------------------------

[End ICM Family]

=============================================================================

# 8.0 MATRIX KEYWORDS

=============================================================================

## Keyword: [Begin ICM Section]

Required: Yes

Argument: Text String

Used By: [Tree Path Description] or [Nodal Path Description]

Uses: [Derivation Method], [Resistance Matrix], [Inductance Matrix],

[Conductance Matrix], [Capacitance Matrix], [ICM S-parameter],

[End ICM Section]

Description: Indicates the beginning and the name of a set of RLGC

or S-parameter matrix data

Usage Rules: This keyword is followed by the name of a section.  The section

must be used in one of the models in the document.

The [Begin ICM Section] keyword may appear multiple times in the

file, however it may appear only after the [End ICM Model Family]

keyword.

When using [Tree Path Description], every matrix used for given

interconnect model MUST have exactly the same number of diagonal

terms.  When using a [Nodal Path Description], the number of

diagonal terms in the matrices may vary.

If the swath method is used the matrix may have fewer

conductors then the interconnect model.  When the swath method is

not used, the matrix MUST have exactly the same number of

diagonal terms as the interconnect has conductors.

The matrices used for the RLGC description can be defined

using any of the matrix description methods allowed in the

specification. The matrix description methods can also be

mixed within a given section. For example the resistance

matrix might use the Diagonal_matrix and the inductance matrix

might use a Full matrix.

For an example of [Begin ICM Section] see matrix examples that

follow.

=============================================================================

## Keyword: [Derivation Method]

Required: Yes

Argument: Text String (Lumped or Distributed)

Used By: [Begin ICM Section]

Description: This keyword describes how the data in the RLGC or S-parameter

matrixes should be interpreted.  This keyword also places rules

on what individual matrixes must be present in a section.

Usage Rules:  Following the keyword is one of two arguments: "Lumped" or

"Distributed".

The [Derivation Method] keyword may appear multiple times in the

file, however it may appear only once between the [Begin ICM

Section]/[End ICM Section] keyword pair, and must be placed

before any [*** Matrix] or [ICM Sparameter] keywords.

If “Lumped” is specified then the RLGC matrix data represents

discrete values suitable for creating a circuit of

interconnected inductor, capacitor and resistor elements.  A

section may be replicated, but the values in each matrix are

fixed. For sections described using lumped data any single,

any pair, or all of the following keywords could be used to

describe a section: [Resistance Matrix], [Inductance Matrix],

[Conductance Matrix], and [Capacitance Matrix].

If “Distributed” is specified then the RLGC matrix data

represents per-unit-length values suitable for creating a

transmission line representation of the section.  A

multiplier value representing the physical length of a section

may be applied to the data in the RLGC matrixes.  For sections

described using distributed data both the [Inductance Matrix]

and [Capacitance Matrix] keywords are required. The

[Conductance Matrix] and [Resistance Matrix] keywords are

optional.

For a section described using S-parameter data the argument

to the [Derivation Method] keyword shall be “Lumped”.

All matrices within the same section must share the same

derivation method.

For any SLM lumped model, Resistance and Inductance are series

components, Capacitance and Conductance are shunted to the

return path.

The assumed circuit topology is RLGC for any derivation method.

If a different circuit topology is desired (for example, CLRG)

then start the model by creating a section that has only the

capacitance element, then follow this by a standard LRG section.

------------------------------------------------------------------------------

[Derivation Method] Lumped

=============================================================================

## Keyword: [Resistance Matrix], [Inductance Matrix], [Conductance Matrix], 
              [Capacitance Matrix]

Required: Yes, at least one

Argument: Text String (Matrix Type)

Used By: [Begin ICM Section]

Description: Each keyword marks the beginning of its respective matrix data

set and also specifies how the matrix data is formatted.

Usage Rules: Following each matrix keyword, one of the following arguments is

required: Diagonal_matrix, Banded_matrix, Sparse_matrix, or

Full_matrix.  No more than one of each matrix type

is allowed per section. If model type is SLM only a diagonal

matrix is allowed.

The [Resistance Matrix], [Inductance Matrix], [Conductance

Matrix], and [Capacitance Matrix] keywords may appear multiple

times in the file, however they may appear only once between the

[Begin ICM Section]/[End ICM Section] keyword pair, and they must

be placed after the [Derivation Method] keyword.

After each of these keywords, insert the matrix data in

the appropriate format.  (These formats are described in

detail below).

If the location in the matrix is defined as a return path (“GND”)

in the [ICM Pin Map] or [ICM Node Map], the values associated

with the “GND” designation in the matrix diagonal must be 0 for

any of the matrices.

Arguments:

Diagonal_matrix, Banded_matrix, Sparse_matrix, or Full_matrix

#### Diagonal_matrix

The Diagonal_matrix is used to specify uncoupled

models.

This subparameter is exactly the same as using

the Banded_matrix described below with a

Bandwidth=0.  It has the added benefit of

requiring half as many lines.

A Diagonal_matrix is one whose entries are

guaranteed to be zero if they are not on the

main diagonal.

Specify the Diagonal_matrix one row at a time,

starting with row 1 and working up to higher

rows.  Diagonal elements are entered one per

line. Mark each row with the [Row] keyword.

#### Banded_matrix

The Banded_matrix is used to specify the

coupling effects up to “X” pins on either side

for a given pin.

A Banded_matrix is one whose entries are

guaranteed to be zero if they are farther away

from the main diagonal than a certain distance,

known as the "bandwidth."

The bandwidth for a Banded_matrix must be

specified using the [Bandwidth] keyword.

Specify the banded matrix one row at a time,

starting with row 1 and working up to higher

rows.  Mark each row with the [Row] keyword.

Symmetry is exploited to reduce the amount

of data by eliminating, from the matrix, any

entries below the main diagonal.

#### Sparse_matrix

A Sparse_matrix is expected to consist mostly of

zero-valued entries, except for a few nonzero.

Unlike the Banded_matrix, there is no

restriction on where the nonzero entries can

occur.

An NxN Sparse_matrix is specified one row at a

time, starting with row 1 and continuing down to

row N.  Each new row is marked with [Row]

keyword, as in the other matrix formats.

Symmetry is exploited to reduce the amount

of data by eliminating, from the matrix, any

entries below the main diagonal.

#### Full_matrix

Description: When the Full_matrix format is used, the

coupling between every pair of elements is

specified explicitly.  Assume that the Matrix

has N rows and N columns.

The Full_matrix is specified one row at a time,

starting with Row 1 and continuing down to Row

N.  Each new row is identified with the [Row]

keyword.

Symmetry is exploited to reduce the amount

of data by eliminating, from the matrix, any

entries below the main diagonal.

------------------------------------------------------------------

See example "Matrix Example" lower in this document.

### Matrix Descriptions

This insert provides additional information regarding the

[Resistance Matrix], [Inductance Matrix], [Conductance Matrix],

and [Capacitance Matrix] keywords.  These will be referred to as

"RLGC" matricies.

When measuring the entries of the RLGC matrices, either with

laboratory equipment or field solver software, currents are

defined as ENTERING the pins of the package from the board.  The

corresponding voltage drops are to be measured with the current

pointing "in" to the "+" sign and "out" of the "-" sign.

I1   +-----+    I2

-----> |     |  <------

board o--------| ICM |---------o board

+  V1 - |     |-  V2  +

+-----+

It is important to observe this convention in order to

get the correct signs for the mutual inductance and

resistances.

For each [Resistance Matrix], [Inductance Matrix], [Conductance

Matrix] or [Capacitance Matrix] a different matrix type can be

used for the data.

### Resistance Matrix Description:

The resistance matrix represents the total DC resistance in ohms

between the two ends of this section of the interconnect.

The resistance matrix is optional.  Simulation tools shall

use 0.0 ohms for all diagonal entries and “infinite” for all

off diagonal entries if this matrix is not present.

When the Section or N_section subparameters use the “Mult”

variable then DO NOT USE ohms/meter as this is the total

value and includes the length of this section.

When the Section or N_section subparameters use the “Len”

variable then USE ohms/meter.

Values should all appear in the diagonal terms of the

matrix.  For example: value 1,1 is the resistance for pin 1

and value 2,2 is the resistance for pin 2.

A "Diagonal_matrix" matrix type is the best choice for

describing the resistance matrix.  Diagonal elements

may be dominant.  Comparably, the off diagonal elements may

be negligible thus leading to off diagonal element values

of zero leading to the [Diagonal_matrix] format.

### Inductance Matrix Description:

Two inductance matrix methods are supported:

Loop-Inductance Matrix for SLMs ONLY    (non-coupled)

Partial-Inductance Matrix for MLMs ONLY (coupled)

If ModelType is SLM then all sections must use loop-inductance.

If ModelType is MLM then all sections must use partial-

inductance.

When the Section or N_section subparameters use the “Mult”

variable then DO NOT USE henries/meter as this is the total

value and includes the length of this section.

When the Section or N_section subparameters use the “Len”

variable then USE henries/meter

### Loop-Inductance Matrix Description (SLMs ONLY):

To define inductance requires a loop.  The loop consists of a

closed path including a signal and all possible return paths.

The inductance is a coefficient between the emf induced in the

loop and the change of magnetic flux through the loop.

interconnect or section

Return .....................| Return

(side A)                             |           (side B)

Signal .....................| Signal

|

Return .....................| Return

The return path must include all pins that are power or ground.

In a loop inductance matrix, the diagonal terms represent the

loop inductance of a specific pin given a specific return path.

The loop inductance method must be used if the ModelType is SLM.

Loop inductance can be experimentally measured.

Loop inductance matrices can be created using field solvers.

An SGR value is normally specified for an SLM model that

indicates the ratio of signal pins to return path pins (i.e.,

power or ground) pins.

Typical uses for SLM_general model might include:

a 2x17 header connector where the bottom row is all

grounded.  The SGR would be specified in this case

as 1:1.  A model would be developed with these

assumptions made.  Then all pins could be filled

with average values.  To further expand this example

see the figure below:

Pin out:

1  ................. 17

18 ................. 34

Measurement or Field solver connection:

Sig Sig Sig ... Sig Sig

Gnd Gnd Gnd ... Gnd Gnd

Measured or Computed values:

3nH 4nH 4nH ... 4nH 3nH

Matrix data:

3nH 4nH 4nH ... 4nH 3nH

3nH 4nH 4nH ... 4nH 3nH

This allows the user to specify any of the following

power/ground connections and still use this model:

Simulator Connection A:

Sig Sig Sig ... Sig Sig

Gnd Gnd Gnd ... Gnd Gnd

Simulator Connection B:

Gnd Gnd Gnd ... Gnd Gnd

Sig Sig Sig ... Sig Sig

Simulator Connection C:

Sig +5V Sig ... Gnd Sig

Gnd Sig Gnd ... Sig +5V

Given the following connection a simulation should generate a

warning to the user if the indicated ground-signal-ratio has

been violated.

Simulator Connection D:

Sig Sig Sig ... Sig Sig

Gnd Sig Sig ... Sig Sig

All interconnect conductors must be specified with non-zero L

and C values in at least one of the sections.  The

simulator will zero out portions of all sections

corresponding to the ground or power pins.

SLM models are most useful where coupling analysis

is not critical and where the rise time of the

signal is long compared to the electrical delay

through the interconnect.

### Partial Self Inductance Matrix Description (MLMs ONLY):

A partial self inductance is an approximation of the loop

inductance in situations where the current return path cannot be

explicitly stated in the field extraction stages.

Partial self inductance can be numerically calculated by

introducing a current in a circuit element that originates on

one end and terminates on the other end. There is no physical

loop per se, instead charges will build up on each side of the

circuit element.  In practice, this situation cannot exist more

than instantaneously.  For the purposes of simulation the

condition does not affect the resultant loop inductance values.

Then from the currents found the magnetic fields and

inductance can be calculated.  In this way, the self and

mutual inductances of any section can be calculated.

This representation of inductance in terms of partial self and

mutual inductance allows the analysis of any combination of

current paths. The signal and return current paths can be

defined in circuit after the model is established. Assuming that

the self inductance is included in the return path, the circuit

under simulation can account for the impact on signal-integrity,

cross-talk and ground-bounce from currents passing through any

combination of pins.

section

Signal .....................  Signal

(side A)                                |       (side B)

Signal .....................  Signal

|

Signal .....................  Signal

There are many ways to create this information using either 2D

or 3D field solvers.  It may also be possible to create this

information using other methods which are left to the model

developer.

For an MLM model partial inductance values are required.

### Capacitance Matrix Description

A capacitance is an approximation of the capacitance in

situations where the return path cannot be explicitly stated

in the field extraction stages.

See "Multi-Line Capacitance Matrix" section above for more

information on Multi-Line Matrices.

Two capacitance matrix methods are supported:

SLM, Single-Line, Capacitance Matrix (non-coupled)

MLM, Multi-Line, Maxwell Capacitance Matrix (coupled)

If ModelType is SLM then all sections must only contain a Single

Line Capacitance Matrix.

If ModelType is MLM then all sections must use a Maxwell

Capacitance as stated below.

Single-Line Capacitance Matrix Description:

To define capacitance requires conductors separated by a

dielectric.  The conductors are those found in a loop

consisting of a closed path including a signal and all possible

return paths.  The Maxwell Capacitance matrix relates the total

charges on the conductors to the voltages applied to them.

[Q]=[Cm][V] Where [Cm] is the Maxwell matrix, [Q] is the vector

of total conductor charges, and [V] is a vector of applied

voltages. The negative values of the off-diagonal entries can be

explained on physical grounds: if you put +1 Volt on a single

conductor and 0 Volts on the others, then you induce negative

charges on those other conductors to terminate the field lines

emanating from the conductor at 1 Volt.

Maxwell Capacitance Matrix Description

In the Maxwell Capacitance matrix, the diagonal term is the

maximum capacitance that the conductor encounters, meaning the

capacitance to ground plus the capacitances to all the other

conductors.

In the Maxwell capacitance matrix, the off-diagonal term is the

capacitance that the conductor encounters relative to a second

conductor.  This capacitance value is affected by other nearby

conductors during the field extraction phase.  It should

not normally be used by itself to determine the capacitance of a

random pair of conductors in a given structure.

When the Section or N_section subparameters use the “Mult”

variable then DO NOT USE farads/meter as this is the total

value and includes the length of this section.

When the Section or N_section subparameters use the “Len”

variable then USE farads/meter

### Conductance Matrix Description:

The Conductance maxtrix represents the energy transfer

between the conductor and its surroundings, including the

return path.  Conductance includes dielectric and radiation

losses, as well as any DC conductance.

The conductance matrix is optional.  Simulation tools shall use

0.0 Siemens for all entries if this matrix is not present.

When the Section or N_section subparameters use the “Mult”

variable then DO NOT USE siemens/meter as this is the total

value and includes the length of this section.

When the Section or N_section subparameters use the “Len”

variable then USE siemens/meter

### Loss Matrix Description:

The [ICM S-parameter] keyword introduced later provides a format

for describing losses that vary with frequency.

=============================================================================

## Keyword: [Bandwidth]

Required: Yes, if a Banded_matrix matrix is being defined.

Argument: Numeric Value

Used By: [Resistance Matrix], [Inductance Matrix], [Conductance Matrix],

[Capacitance Matrix]

Description: This keyword indicates the size of the width of the coupling

matrix.  This does not have anything in common with electrical

bandwidth.  The bandwidth field must be a nonnegative integer.

This is the number of off-diagonal matrix rows for which data is

provided.

Usage Rules: The bandwidth field must be a non-negative integer. The

[Bandwidth] keyword may appear multiple times in the file,

however it may be used only once per each set of [*** Matrix]

keywords, and it must be placed before the matrix data is given).

------------------------------------------------------------------------------

See example "Matrix Example" lower in this document.

=============================================================================

## Keyword: [Row]

Required: Yes

Argument: Row Number

Used By: [Resistance Matrix], [Inductance Matrix], [Conductance Matrix],

[Capacitance Matrix]

Description: Indicates the beginning of a new row of the matrix

Usage Rules: The argument must be a sequential number beginning at 1

and continuing to the last row for the matrix.

Following a [Row] keyword is a block of numbers that represent

the entries for that row.  Suppose that the current row is

number M.  Then the first number listed is the diagonal entry,

[M,M].  Following this number are the entries of the upper half

of the matrix that belong to row M: [M, M+1], [M, M+2], ... up

to [M,N].

For even a modest-sized matrix, this data will not all fit on

one line.  It is allowed to break the data up with new-line

characters so that the line length limit is observed.

Example: Suppose the matrix has 40 conductors and that we are

currently working on Row 19.  There is 1 diagonal entry, plus

40 - 19 = 21 entries in the upper half of the matrix to be

specified, for 22 entries total.

Row 1 always has the most entries, and that each successive row

has one fewer entry than the last; the last row always has just

a single entry.

The [Row] keyword must be placed directly after the [Bandwidth]

keyword.  The data entries for the row defined by this keyword

begin on the line below the keyword and may span multiple lines.

------------------------------------------------------------------------------

See example "Matrix Example" lower in this document.

==============================================================================

## Keyword: [ICM S-parameter]

Required: No, unless ICM_Model_Type subparameter of [Begin ICM Model] is

equal to “S-parameter”.

Sub-Params: File_name, Port_map

Used By: [Begin ICM Section]

Description: Indicates a reference to an external S-parameter file

Usage Rules: An S-parameter matrix can be used in place of an RLGC matrix.

The [ICM Sparameter] keyword may appear multiple times in the

file, however it may appear only once between the [Begin ICM

Section]/[End ICM Section] keyword pair, and they must be placed

after the [Derivation Method] keyword.

File_name

File_name is to reference the name of the Touchstone

file.  It is required that the ICM and the referenced file be in

the same directory.

Port_map

“Port_map” is to be used to relate the ports in the S-parameter

file to the “fixed” pin map in the ICM file.  It is important

that the “Port_map” uses the pin names from [ICM PinMap] or [ICM

Node Map].  No more or no less entries in the table are

allowed.

There are three columns used in the Port_map.  The first column

is a reference to a “Pin Name” as defined in the [ICM Pin Map] or

[ICM Node Map] keyword.  The second column should reference the

input port name as defined in the Touchstone file that is related

to the “Pin Name”.  The third column should reference the

output port name as defined in the Touchstone file that

is related to the “Pin Name”.

Example:

------------------------------------------------------------------------------

[ICM S-parameter]

File_name anyname.any_extension

Port_map

|Pin    Port A  Port B

A1      1       2

A2      3       4

A3      5       6

A4      7       8

B1      9       10

B2      11      12

B3      13      14

B4      15      16

===============================================================================

## Keyword: [End ICM Section]

Required: Yes

Argument: Text String

Used By: [Begin ICM Section]

Description: Indicates the end of [Begin ICM Section].  This keyword is after

the [Derivation Method], [Resistance Matrix], [Inductance

Matrix], [Conductance Matrix], [Capacitance Matrix], [Bandwidth],

[Row], [ICM S-parameter].

Usage Rules: Every [Begin ICM Section] keyword must have a corresponding

[End ICM Section] keyword, which must be the last keyword of

an ICM Section description.

### Matrix Examples:

===========================================================================

The resistance matrix for this package does not have coupling:

[Begin ICM Section] ExampleMatrix00

[Derivation Method] Distributed

[Resistance Matrix] Diagonal_matrix

10.0

15.0

15.0

10.0

10.0

15.0

15.0

10.0

[End ICM Section] ExampleMatrix00

[Begin ICM Section]       ExampleMatrix01

The resistance matrix for this package does not have coupling

and is exactly the same as the Diagonal_matrix shown above

and is described using a trivial version of the Banded_matrix

as an example only.  The Diagonal_matrix should be used for

this example to conserve file size.

[Resistance Matrix] Banded_matrix

[Bandwidth] 0

[Row]   1

10.0

[Row]   2

15.0

[Row]   3

15.0

[Row]   4

10.0

[Row]   5

10.0

[Row]   6

15.0

[Row]   7

15.0

[Row]   8

10.0

The inductance matrix has coupling between pins:

[Inductance Matrix] Full_matrix

[Row]   1

3.04859e-07      4.73185e-08      1.3428e-08     6.12191e-09

1.74022e-07      7.35469e-08     2.73201e-08     1.33807e-08

[Row]   2

3.04859e-07      4.73185e-08      1.3428e-08     7.35469e-08

1.74022e-07      7.35469e-08     2.73201e-08

[Row]   3

3.04859e-07      4.73185e-08     2.73201e-08     7.35469e-08

1.74022e-07      7.35469e-08

[Row]   4

3.04859e-07      1.33807e-08     2.73201e-08     7.35469e-08

1.74022e-07

[Row]   5

4.70049e-07      1.43791e-07     5.75805e-08     2.95088e-08

[Row]   6

4.70049e-07      1.43791e-07     5.75805e-08

[Row]   7

4.70049e-07      1.43791e-07

[Row]   8

4.70049e-07

The capacitance matrix has sparse coupling:

[Capacitance Matrix] Sparse_matrix

[Row]   1

1       2.48227e-10

2       -1.56651e-11

5       -9.54158e-11

6       -7.15684e-12

[Row]   2

2       2.51798e-10

3       -1.56552e-11

5       -6.85199e-12

6        -9.0486e-11

7       -6.82003e-12

[Row]   3

3       2.51798e-10

4       -1.56651e-11

6       -6.82003e-12

7        -9.0486e-11

8       -6.85199e-12

[Row]   4

4       2.48227e-10

7       -7.15684e-12

8       -9.54158e-11

[Row]   5

5       1.73542e-10

6       -3.38247e-11

[Row]   6

6       1.86833e-10

7       -3.27226e-11

[Row]   7

7       1.86833e-10

8       -3.38247e-11

[Row]   8

8       1.73542e-10

[End ICM Section] ExampleMatrix01

=============================================================================

# 9.0 End Keyword

=============================================================================

## Keyword: [End]

Required: Yes

Description: Specifies end of file.  Any text after the [End] keyword is

ignored.

Usage Rules: The [End] keyword marks the end of the interconnect model in

an ICM file and must be the last ICM keyword in the file.

------------------------------------------------------------------------------

[End]