(Unofficial) IBIS Version 5.0 Expanded Tree

Bob Ross (Teraspeed Consulting Group)

July 25, 2006, Updated September 14, 2007, March 13, 2011

Prior Contributors:

Michael Mirmak (Intel Corporation)

Bob Ross (while at Mentor Graphics)

Atsushi Ogawa (NEC) - Original suggestion

Key:

[Keyword] Bold, subparameters on the right side

Introduced first in Version 1.1 unless marked as (2.1), (3.2), (4.2), or (5.0)

The keyword version number applies to the keywords and subparameters below it,

except when a higher version number is shown.

(m)   Keyword or subparameter may occur multiple times.

(ml)  Keyword may occur multiple times and at any location, currently for the

[Comment Char].

*     Defined selections for keywords or subparameters are at the end.

Underlined [Keyword], File or Section scope the contents below it.

.ibs FILE

│

├── File Header Section

│     ├── [IBIS Ver]

│     ├── [Comment Char] (ml)

│     ├── [File Name]

│     ├── [File Rev]

│     ├── [Date]

│     ├── [Source]

│     ├── [Notes]

│     ├── [Disclaimer]

│     └── [Copyright]             (2.1)

│

├── [Component] (m)

│     │                           (3.2)  Si_location *, Timing_location *

│     ├── [Manufacturer]

│     ├── [Package]                      R_pkg, L_pkg, C_pkg

│     ├── [Pin]                          signal_name, model_name, R_pin,

│     │                                  L_pin, C_pin

│     ├── [Package Model]         (2.1)

│     │     └── [Alternate Package Models]   (m)      (4.2)

│     │           └── [End Alternate Package Models]

│     │

│     ├── [Pin Mapping]           (2.1)  pulldown_ref, pullup_ref,

│     │                                  gnd_clamp_ref, power_clamp_ref,

│     │                           (4.2)  ext_ref

│     ├── [Diff Pin]              (2.1)  inv_pin, vdiff, tdelay_typ,

│     │                                  tdelay_min, tdelay_max

│     ├── [Series Pin Mapping]    (3.2)  pin_2, model_name,

│     │                                  function_table_group

│     ├── [Series Switch Groups]  (3.2)  On (m), Off (m)

│     ├── [Circuit Call]          (4.2)  Signal_pin, Diff_signal_pins,

│     │     │                            Series_pins, Port_map (m)

│     │     └── [End Circuit Call]

│     │

│     └── [Begin EMI Component]   (5.0)  Domain *, Cpd, C_Heatsink_gnd,

│           │                            C_Heatsink_float

│           ├── [Pin EMI]                domain_name, clock_div

│           ├── [Pin Domain EMI]         percentage

│           └── [End EMI Component]

│

├── [Model Selector] (m)          (3.2)

│

├── [Model] (m)                          Model_type *, Polarity *, Enable *,

│     │                                  Vinl, Vinh, C_comp,

│     │                           (2.1)  Vmeas, Cref, Rref, Vref

│     │                           (4.2)  C_comp_pullup, C_comp_pulldown,

│     │                           (4.2)  C_comp_power_clamp, C_comp_gnd_clamp,

│     │                           (4.2)  Rref_diff, Cref_diff

│     ├── [Model Spec]            (3.2)  Vinh, Vinl, Vinh+, Vinh-, Vinl+,

│     │                                  Vinl-, S_overshoot_high,

│     │                                  S_overshoot_low, D_overshoot_high,

│     │                                  D_overshoot_low, D_overshoot_time,

│     │                           (5.0)  D_overshoot_area_h

│     │                           (5.0)  D_overshoot_area_l,

│     │                           (5.0)  D_overshoot_ampl_h,

│     │                           (5.0)  D_overshoot_ampl_l,

│     │                                  Pulse_high, Pulse_low, Pulse_time,

│     │                                  Vmeas,

│     │                           (4.2)  Cref, Rref, Vref, Cref_rising,

│     │                           (4.2)  Cref_falling, Rref_rising,

│     │                           (4.2)  Rref_falling, Vref_rising,

│     │                           (4.2)  Vref_falling, Vmeas_rising,

│     │                           (4.2)  Vmeas_falling,

│     │                           (4.2)  Rref_diff, Cref_diff

│     ├── [Receiver Thresholds]   (4.2)  Vth, Vth_min, Vth_max, Vinh_ac,

│     │                                  Vinh_dc, Vinl_ac, Vinl_dc,

│     │                                  Threshold_sensitivity,

│     │                                  Reference_supply *, Vcross_low,

│     │                                  Vcross_high, Vdiff_ac, Vdiff_dc,

│     │                                  Tslew_ac, Tdiffslew_ac

│     ├── [Add Submodel] *        (3.2)

│     ├── [Driver Schedule]       (3.2)

│     ├── [Temperature Range]     (2.1)

│     ├── [Voltage Range]

│     ├── [Pullup Reference]      (2.1)

│     ├── [Pulldown Reference]    (2.1)

│     ├── [POWER Clamp Reference] (2.1)

│     ├── [GND Clamp Reference]   (2.1)

│     ├── [External Reference]    (4.2)

│     ├── [TTgnd]                 (3.2)

│     ├── [TTpower]               (3.2)

│     ├── [Pulldown]

│     ├── [Pullup]

│     ├── [GND Clamp]

│     ├── [POWER Clamp]

│     ├── [ISSO PU]               (5.0)

│     ├── [ISSO PD]               (5.0)

│     ├── [Rgnd]                  (2.1)

│     ├── [Rpower]                (2.1)

│     ├── [Rac]                   (2.1)

│     ├── [Cac]                   (2.1)

│     ├── [On]                    (3.2)

│     ├── [Off]                   (3.2)

│     ├── [R Series]              (3.2)

│     ├── [L Series]              (3.2)

│     ├── [Rl Series]             (3.2)

│     ├── [C Series]              (3.2)

│     ├── [Lc Series]             (3.2)

│     ├── [Rc Series]             (3.2)

│     ├── [Series Current]        (3.2)

│     ├── [Series MOSFET] (m)     (3.2)  Vds

│     ├── [Ramp]                         dV/dt_r, dV/dt_f,

│     │                           (2.1)  R_load

│     ├── [Rising Waveform] (m)   (2.1)  R_fixture, V_fixture,

│     │     │                            V_fixture_min, V_fixture_max,

│     │     │                            C_fixture, L_fixture, R_dut, L_dut,

│     │     │                            C_dut

│     │     └── [Composite Current] (5.0)

│     │

│     ├── [Falling Waveform] (m)  (2.1)  R_fixture, V_fixture,

│     │     │                            V_fixture_min, V_fixture_max,

│     │     │                            C_fixture, L_fixture, R_dut, L_dut,

│     │     │                            C_dut

│     │     └── [Composite Current] (5.0)

│     │

│     ├── [Test Data]  (m)        (4.2)  Test_data_type *, Driver_model,

│     │     │                            Driver_model_inv, Test_load

│     │     ├── [Rising Waveform Near]

│     │     ├── [Falling Waveform Near]

│     │     ├── [Rising Waveform Far]

│     │     ├── [Falling Waveform Far]

│     │     ├── [Diff Rising Waveform Near]

│     │     ├── [Diff Falling Waveform Near]

│     │     ├── [Diff Rising Waveform Far]

│     │     ├── [Diff Falling Waveform Far]

│     │     └── [Test Load]              Test_load_type *, C1_near, Rs_near,

│     │                                  Ls_near, C2_near, Rp1_near,

│     │                                  Rp2_near, Td, Zo, Rp1_far,

│     │                                  Rp2_far, C2_far, Ls_far, Rs_far,

│     │                                  C1_far, V_term1, V_term2,

│     │                                  Receiver_model,

│     │                                  Receiver_model_inv, R_diff_near,

│     │                                  R_diff_far

│     │

│     ├── [External Model] (m)    (4.2)  Language *, Corner * (m), Parameters (m),

│     │     │                            Ports (m), D_to_A * (m), A_to_D * (m)

│     │     └── [End External Model]

│     │

│     ├── [Algorithmic Model]     (5.0)  Executable * (m)

│     │     └── [End Algorithmic Model]

│     │

│     └── [Begin EMI Model]       (5.0)  Model_emi_type *, Model_Domain *

│           └── [End EMI Model]

│

├── [Submodel] (m)                (3.2)  Submodel_type *

│     ├── [Submodel Spec]                V_trigger_r, V_trigger_f, Off_delay

│     ├── [POWER Pulse Table]

│     ├── [GND Pulse Table]

│     ├── [Pulldown]

│     ├── [Pullup]

│     ├── [GND Clamp]

│     ├── [POWER Clamp]

│     ├── [Ramp]                         dV/dt_r, dV/dt_f, R_load

│     ├── [Rising Waveform] (m)          R_fixture, V_fixture,

│     │                                  V_fixture_min, V_fixture_max,

│     │                                  C_fixture, L_fixture, R_dut, L_dut,

│     │                                  C_dut

│     └── [Falling Waveform] (m)         R_fixture, V_fixture,

│                                        V_fixture_min, V_fixture_max,

│                                        C_fixture, L_fixture, R_dut, L_dut,

│                                        C_dut

│

├── [External Circuit]            (4.2)  Language *, Corner * (m), Parameters (m),

│     │                                  Ports *, D_to_A * (m), A_to_D * (m)

│     └── [End External Circuit]

│

├── [Define Package Model] (m)    (2.1)

│     ├── [Manufacturer]

│     ├── [OEM]

│     ├── [Description]

│     ├── [Number Of Sections]

│     ├── [Number Of Pins]

│     ├── [Pin Numbers]

│     │                           (3.2)  Len (m), L (m), C (m), R (m),

│     │                           (3.2)  Fork (m), Endfork (m)

│     ├── [Model Data]

│     │     ├── [Resistance Matrix]      Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     ├── [Inductance Matrix]      Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     ├── [Capacitance Matrix]     Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     └── [End Model Data]

│     │

│     └── [End Package Model]

│

└── [End]

.pkg FILE                           (2.1)

├── File Header Section           (2.1)

│     ├── [IBIS Ver]

│     ├── [Comment Char] (ml)

│     ├── [File Name]

│     ├── [File Rev]

│     ├── [Date]

│     ├── [Source]

│     ├── [Notes]

│     ├── [Disclaimer]

│     └── [Copyright]

│

├── [Define Package Model] (m)    (2.1)

│     ├── [Manufacturer]

│     ├── [OEM]

│     ├── [Description]

│     ├── [Number Of Sections]

│     ├── [Number Of Pins]

│     ├── [Pin Numbers]

│     │                           (3.2)  Len (m), L (m), C (m), R (m),

│     │                           (3.2)  Fork (m), Endfork (m)

│     ├── [Model Data]

│     │     ├── [Resistance Matrix]      Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     ├── [Inductance Matrix]      Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     ├── [Capacitance Matrix]     Banded_matrix, Sparse_matrix,

│     │     │     │                      Full_matrix

│     │     │     ├── [Bandwidth]

│     │     │     └── [Row] (m)

│     │     │

│     │     └── [End Model Data]

│     │

│     └── [End Package Model]

│

└── [End]

.ebd FILE                           (3.2)

├── File Header Section           (3.2)

│     ├── [IBIS Ver]

│     ├── [Comment Char] (ml)

│     ├── [File Name]

│     ├── [File Rev]

│     ├── [Date]

│     ├── [Source]

│     ├── [Notes]

│     ├── [Disclaimer]

│     └── [Copyright]

│

├── [Begin Board Description]     (3.2)

│     ├── [Manufacturer]

│     ├── [Number of Pins]

│     ├── [Pin List]                      signal_name

│     ├── [Path Description] (m)          Len (m), L (m), C (m), R (m),

│     │                                   Fork (m), Endfork (m), Node (m),

│     │                                   Pin (m)

│     ├── [Reference Designator Map]

│     └── [End Board Description]

│

└── [End]

Reserved Words and Multipliers

Reserved Words

POWER

GND

NC

NA

CIRCUITCALL     (4.2)

Multipliers

M, k, m, u, n, p

T, G, f         (2.1)

Selections (*)

[Add Submodel]        (3.2)  Mode column selections

Driving

Non-Driving

All

Si_location           (3.2)

Pin

Die

Timing_location       (3.2)

Pin

Die

Polarity

Non-Inverting

Inverting

Enable

Active-High

Active-Low

Model_type

3-state

3-state_ECL     (3.2)

3-state_diff    (4.2)

I/O

I/O_ECL         (2.1)

I/O_open_drain  (2.1)

I/O_open_sink   (2.1)

I/O_open_source (2.1)

I/O_diff        (4.2)

Input

Input_ECL       (2.1)

Input_diff      (4.2)

Open_sink       (2.1)

Open_drain

Open_source     (2.1)

Output

Output_ECL      (2.1)

Output_diff     (4.2)

Series          (3.2)

Series_switch   (3.2)

Terminator      (2.1)

Submodel_type         (3.2)

Bus_hold

Dynamic_clamp

Fall_back       (4.2)

Reference_supply      (4.2)

Power_clamp_ref

Gnd_clamp_ref

Pullup_ref

Pulldown_ref

Ext_ref

Test_data_type        (4.2)

Single_ended

Differential

Test_load_type        (4.2)

Single_ended

Differential

Language              (4.2)

SPICE

VHDL-AMS

Verilog-AMS

VHDL-A(MS)

Verilog-A(MS)

Corner, A_to_D corner_name  (4.2)

Typ

Min

Max

Corner, D_to_A corner_name  (4.2)

Typ

Min

Max

Reserved Digital Port Names (4_2)

D_receive

D_drive

D_enable

D_switch

Reserved Analog Port Names  (4.2)

A_signal

A_pos

A_neg

A_signal_pos

A_signal_neg

Reserved Analog Reference Names (4.2)

A_pufref

A_pdref

A_pcref

A_gcref

A_extref

A_gnd

Executable ‘Platform’_’Compile’_’Bits’ column  (5.0)

‘Platform’ (extra characters can be appended)

Linux

Solaris

Windows

HP-UX

‘Compile’ (extra characters can be appended)

gcc

cc

VisualStudio

accA

‘Bits’

32

64

Domain              (5.0)

Analog

Digital

Model_emi_type      (5.0)

Ferrite

Not_a_ferrite

Model_Domain        (5.0)

Analog

Digital