|  |  | 93/XX/CDV COMMITTEE DRAFT FOR VOTE (CDV) PROJET DE COMITÉ POUR VOTE (CDV) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Project number 62014-3 Numéro de projet |  |  |  |
| IEC/TC or SC: TC93 WG2 CEI/CE ou SC: |  | Date of circulation Date de diffusion |  | Closing date for voting (Voting mandatory for P-members) Date de clôture du vote (Vote obligatoire pour les membres (P)) |  |
| Titre du CE/SC: |  |  | TC/SC Title: |  |  |
| Secretary: Gregory LEDENBACH Secrétaire: |  |  |  |  |  |
| Also of interest to the following committees Intéresse également les comités suivants |  |  | Supersedes document Remplace le document |  |  |
| Horizontal functions concerned Fonctions horizontales concernées |  |  |  |  |  |
| Safety Sécurité | EMC CEM |  | Environment Environnement |  | Quality assurance Assurance qualité |
| ce document est toujours à l'Étude et susceptible de modification. il ne peut servir de rÉfÉrence. les rÉcipiendaires du prÉsent document sont invitÉs À prÉsenter, avec leurs observations, LA notification des droits de propriÉtÉ dont ils auraient Éventuellement connaissance et À fournir une documentation explicative. |  |  | this document is still under study and subject to change. it should not be used for reference purposes. recipients of this document are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation. |  |  |

| Titre : CEM des composants, un modèle électrique des circuits intégrés (ICEM) | Title : EMC for Component, Integrated circuits Electrical Model (ICEM). |
| --- | --- |

| Note d'introduction | Introductory note |
| --- | --- |

Release 1.0

FORM 7B (IEC)
1999-10-01

| © | International Electrotechnical Commission, IEC Commission Électrotechnique Internationale, CEI |
| --- | --- |

01 March 2001

TABLE OF CONTENTS

1	SCOPE	4

1.1	General	4

1.2	Philosophy	5

1.2.1	Origin of parasitic emission	5

1.2.2	Conducted emission through power-supply lines	6

1.2.3	Conducted emissions through input/output lines (I/O)	6

1.2.4	Direct radiated emissions	6

2	Normative references	6

3	Definitions	7

3.1	Electro Magnetic Compatibility EMC	7

3.2	Electro Magnetic Emission	7

3.3	Electro Magnetic Radiation	7

4	ICEM  models description	8

4.1	ICEM power-supply line model	8

4.1.1	First and second order effects.	9

4.2	ICEM Input/Output	10

4.2.1	Single supply structure	10

4.2.2	Multiple supplies structure	10

4.3	ICEM direct radiation	11

4.3.1	ICEM direct radiated  model	12

5	ICEM models parts details	12

5.1 Passive parts parameters	12

5.1.1 Measurement of part values	13

5.1.2 Prediction of part values	13

5.2	The current sources Ib and Ii/o.	13

5.2.1	Measurement of Ib and Ii/o	14

Annex 1	15

Implementation in SPICE, IBIS and IMIC modeling data bases	15

Annex2                                                                                                                                                     16

Part1 EMC Task force contributors                                                                                                          16

Part2 MUNICH IBIS Summit meeting presentation                                                                                 17

INTERNATIONAL ELECTROTECHNICAL COMMISSION

____________

IEC 62014-3

Models of Integrated Circuits for EMI

behavioral simulation

FOREWORD

1)	The IEC (International Electrotechnical Commission) is a worldwide organization for standardization comprising all national electrotechnical committees (IEC National Committees). The object of the IEC is to promote international co-operation on all questions concerning standardization in the electrical and electronic fields. To this end and in addition to other activities, the IEC publishes International Standards. Their preparation is entrusted to technical committees; any IEC National Committee interested in the subject dealt with may participate in this preparatory work. International, governmental and non-governmental organizations liaising with the IEC also participate in this preparation. The IEC collaborates closely with the International Organization for Standardization (ISO) in accordance with conditions determined by agreement between the two organizations.

2)	The formal decisions or agreements of the IEC on technical matters express, as nearly as possible, an international consensus of opinion on the relevant subjects since each technical committee has representation from all interested National Committees.

3)	The documents produced have the form of recommendations for international use and are published in the form of standards, technical specifications, technical reports or guides and they are accepted by the National Committees in that sense.

- In order to promote international unification, IEC National Committees undertake to apply IEC International Standards transparently to the maximum extent possible in their national and regional standards. Any divergence between the IEC Standard and the corresponding national or regional standard shall be clearly indicated in the latter.

- The IEC provides no marking procedure to indicate its approval and cannot be rendered responsible for any equipment declared to be in conformity with one of its standards.

- Attention is drawn to the possibility that some of the elements of this International Standard may be the subject of patent rights. The IEC shall not be held responsible for identifying any or all such patent rights.

# SCOPE

The objective of the standard ICEM   (Integrated Circuit Electrical Model) for Components is to propose electrical modeling for  integrated circuit internal activities. This model will be used to evaluate electromagnetic behavior and performances of electronic equipment.

## General

Integrated circuits integrate more and more gates on silicon and the technologies are faster and faster. To predict the electromagnetic behavior of equipment it is required to model I.C. interface switching and their internal activities as well. Indeed IBIS and IMIC models are focused mainly on interface activity predictions (cross-talk, overshoot, ..).

This document describes a model for EMI simulation due to IC internal activities. This model gives more accurately the electromagnetic emissions of electronic equipment by taking into account the influence of internal activities. This model gives general data which could be implemented in different format such as IBIS, IMIC, SPICE, …..

During the design stage of the application that will exploit the IC, it becomes useful to predict and to prevent electromagnetic risks with CAD tool. Accurate IC modeling is necessary to run on these simulation tools.

Three coupling mechanisms of the internal activities  for emission (Figure 1) are proposed in the ICEM model:

- Conducted emissions through supply lines,

- Conducted emissions through input/output lines,

- Direct Radiated emissions.

Figure 1: Mechanisms for parasitic emission covered by ICEM

This document proposes a model that addresses those three types of coupling in a single approach. The elements of the model  should be  kept as simple as possible to ease the identification and simulation process.

## Philosophy

The purpose of this standard is to provide data to enable printed-circuit-board level (PCB) electro-magnetic tools to compute the electromagnetic fields produced by integrated circuits and their associated PCB. These data can be extracted from measurement methods, as described in the IEC61967 standard, or obtained from I.C. simulation tools.

### Origin of parasitic emission

The origin of parasitic emission in I.C is due to the current flowing through all the I.C gates (Ivdd and Ivss) during high to low or low to high transitions as shown in figure 2.

Figure 2: The basic mechanism for parasitic emission is due to the current driving by all the gates.

The combination of several hundred thousands of gates lead to very important peaks of current, mainly at rise and fall edges of the clock circuit. For example Figure 3 plots the number of gates switching versus the time for an I.C integrating 1 million transistors. Consequently, high current spikes are created inside the die and induce voltage drops of the internal voltage references.

Figure 3: Number of switching gates versus time.

### Conducted emission through power-supply lines

The current spikes created inside the die are partially reduced thanks to the on-chip decoupling capacitance. Anyhow, a significant portion of the current spikes is present at the power-supply pins of the chip. This current could be measured  according to the IEC 61967 standard or other methods permitting to have the power-supply currents.

### Conducted emissions through input/output lines (I/O)

The internal voltage drops generated by the current spikes create noise  on the I/Os through direct connection, parasitic capacitive and inductive couplings and/or through common impedance. The PCB wires connected to the I/O can act as antennas and propagate electromagnetic emissions. The measurement set-up is done according to IEC 61967 standard.

### Direct radiated emissions

The internal current flowing in  low impedance loops generates electromagnetic fields which can be measured in near-field according to the IEC 61967 standard.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this International Standard. At the time of publication, the editions indicated were valid. All normative documents are subject to revision, and parties to agreements based on this International Standard are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. Members of IEC and ISO maintain registers of valid International Standards.

IBIS  -I/O Buffer Information Specification version 3.2

IEC 62014-1 : 93/91/CDV, Electronic behavioral specifications of digital integrated circuits I/O Buffer Information Specification (IBIS, Version 2.1)

IEC 93/67/NP : Models of integrated circuits for EMI behavioral simulation

IEC 62200 : 47A/575/NP, Integrated circuits, I/O Interface Model for Integrated Circuit (IMIC)

IEC 61967 part 1: Integrated Circuits, Measurement of electromagnetic emissions, 150KHz to 1GHz. general and definitions.

IEC 61967 part 2: Integrated Circuits, Measurement of radiated emissions, TEM cell method.

IEC 61967 part 4: Integrated Circuits, Measurement of conducted emissions, 1method .

IEC 61967 part 6: Integrated Circuits, Measurement of RF current, Magnetic Probe Method.

# Definitions

## Electro Magnetic Compatibility EMC

Ability of an equipment or system to function satisfactorily in its electromagnetic environment without introducing intolerable electromagnetic disturbance to anything in that environment.

## Electro Magnetic Emission

Phenomenon by which electromagnetic energy emanates from a source.

## Electro Magnetic Radiation

- The phenomena by which energy in the form of electromagnetic waves propagates from a source into space.

- Energy transferred through space in the form of electromagnetic waves.

# ICEM  models description

The proposed model includes 3 sections which describe the 3 coupling mechanisms of the internal activities  for emission introduced in part 1 :

- ICEM power-supply line model for conducted emissions through supply lines

- ICEM Input/Output  for conducted emissions through input/output lines

- ICEM direct radiation  for direct radiated emissions

Models are defined with electrical schematics  described below for each IC pin.

## ICEM power-supply line model

The I.C equivalent model shown in figure 4 is able to determine the peak harmonics spectrum and main resonances.

.

This model consists in:

- Ib:  current generator,

- LpackVdd: package inductance of the positive supply Vdd,

- LpackVss: package inductance of the ground Vss,

- RpackVdd: package resistor of the positive supply Vdd,

- RpackVss: package resistor of the ground Vss,

- Cd: parasitic capacitor between Vdd and Vss package pins,

- Rvdd, series resistor of Vdd, bonding and die connection,

- Rvss, series resistor of Vss , bonding and die connection,

- Lvdd: inductance of Vdd, bonding and die connection,

- Lvss: inductance of Vss, bonding and die connection.

- Cb:  internal die capacitor.

Figure 4 : Model of the IC supply lines

### First and second order effects.

The inductance of the package LpackVdd, LpackVss, in series with the capacitance Cd create a first resonance, while the serial inductances Lvdd, Lvss, in series with the local block capacitance Cb create a second resonance (Figure 5).

Figure 5: Origin of primary and secondary resonance in the IC model

Taking into account second order effects in the proposed model give more accurate simulation results regarding measurement results,as shown in fig 6.

Figure 6: Comparison between simulation and measurements(IEC 61967-4 ,1method )

## ICEM Input/Output

### Single supply structure

Disturbances on I/O are mainly due to the current flow in the supply and ground impedances added to I/O stage. The  ICEM Input/Output is modeled by the superposition of that internal current noise  with functional signals.

Functional signals are described with model as IBIS, IMIC or SPICE. The I/O on which the simulation is performed could be active or not.  When other I/O`s are activated on the same power lines a new equivalent current generator, Ii/o, representing peripheral activity, must be added in parallel to the generator representing the internal activity Ib.

The schematic of the model is reported in figure 7.

Figure 7: Coupling between core and I/Os

Note : Frequency limits of  functional I/O models should be take into account  for EMC high frequency simulation. For instance a second order model should be defined , bonding and package elements, to simulate the resonances.

### Multiple supplies structure

In many cases, the core supply and the I/O supply have separate internal networks. The core model is still identical, but two supplementary components are inserted, as illustrated in figure 8. The first parameter, named Zsub, accounts for the substrate coupling path between the core Vss and the I/O Vss. The second parameter is the decoupling capacitance between I/O supplies, named Cio.

When other I/O`s are activated on the same power lines a new equivalent current generator, Ii/o, representing peripheral activity, must be added to the generator representing the internal activity Ib.

Figure 8: Coupling between core and I/Os in the case of separate supplies

Note : Frequency limits of  functional I/O models should be take into account for EMC high frequency simulation. For instance a second order model should be defined , bonding and package elements, to simulate the resonances.

## ICEM direct radiation

Electromagnetic emission could radiate directly from IC itself. The level is closely linked to current flowing in internal loops on  package and on die.

The radiated electromagnetic emission shall be characterized by measurement methods which measure direct radiation as for instance described in IEC 61967-2 in TEM cell.

Fig9 show measurement results in TEM cell with 2 microcontrollers

Figure 9: IC direct emissions measured in TEM cell

Model parameters  should be electrical parameters as internal currents and geometrical parameters as die size, internal loops area and package characteristics.

### ICEM direct radiated  model

This model should be based on an equivalent representation  with dipoles.

It shall be possible to implement it  in models like IBIS, IMIC or SPICE .

The model is under definition and will be completed in future.

# ICEM models parts details

The model  parts are detailed in this section. We review the current generator Ib, the decoupling capacitance, the serial resistances and inductances and the local block capacitance.

Methods  to determine model  parts values are also considered in this section.

## 5.1 Passive parts parameters

Passive components of the model are: LpackVdd, LpackVss, Cd , Rvdd, RVss, LVdd, LVss, Cb , ,Ci/o, Zsub

LpackVdd, LpackVss are package inductances.

Cd represents the parasitic capacitor between Vdd and Vss package pins.

Rvdd, RVss series resistances of the supply network modelize the metal interconnect that connects the block supply to the main supply ring, which goes to the external supply through specific pads.

LVdd, LVss serie inductances of the supply network modelize the metal interconnect that connects the block supply to the main supply ring.

Cb is the internal die capacitor, placed in parallel with the local current generator. It accounts for the equivalent decoupling capacitance of the block.

Ci/o is the internal die capacitor, placed in parallel with the  I/O block (fig. 8). It accounts for the equivalent decoupling capacitance between I/O`s power supply lines.

Zsub coupling impedance is valid for most CMOS technologies with P-type substrate. It accounts for the substrate coupling path between the core Vss and the I/O Vss.

Value range of these components are reported in the table1 .

| Part name | Min value | Max value |
| --- | --- | --- |
| LpackVdd, LpackVss | 1nH | 10nH |
| Cd | 10pF | 100nF |
| Rvdd, RVss | 0,1 | 10 |
| LVdd, LVss | 1nH | 20nH |
| Cb | 10pF | 100nF |
| Ci/o | 10pF | 100nF |
| Zsub dc value | 0 | 100 |

table1 : Value range of the model parameters

These values are for informative  purpose only. They may vary with new technologies.

In order to perform accurate simulations, the values of these parameters should be accurately determined for each specific case.

### 5.1.1 Measurement of part values

Input impedance should be measured according to time domain recflectometry method (TDR) or with a network analyzer , …

From the measurement results and the already known data (eg  package) it`s possible to extract all the part values using mathematical procedures.

### 5.1.2 Prediction of part values

Part values could be determined with IC design tools which compute the RLC parameters from geometrical and electrical characteristic of IC.

## The current sources Ib and Ii/o.

The main source of parasitic emission considered in the model is the current source Ib. The current shape may consist of the time-domain description of the current in PWL (Piece Wise Linear) format (Figure 10)

Typical values for Ib are several mA, up to 1A for the amplitude, 0.1 to 5ns for duration, and 500ps to 50ns for the period. These values are closely dependant on the software running.

Figure 10: Current source definition as a PWL description versus time

### Measurement of Ib and Ii/o

External current shall be measured, with or without I/O activities, but according to measurement method described in IEC 61967 standard. The software should be clearly described during that test.

A reverse engineering simulation of this external current with previous parameters permits then to extract the internal current value. In case of single supply line, the Ib current is obtained taking account of the Ii/o’s currents values following the interfaces are working or not. In case of multiple supply lines, each current is measured separately. So, the reverse engineering process is used similarly for the two currents.

# Annex 1

# Simulation tools Implementation

The ICEM model described in this document includes additional elements specified to provide a capability to simulate EMC/EMI performances of a complete application or to optimize the PCB layout regarding EMC/EMI phenomena. Such simulation can also help to select active and passive components.

To perform an EMC/EMI simulation the different elements of the model previously defined need  to be implemented in  a software simulation tools.

This implementation and the exact definition of the final model  depend on the software tool used  and also base on the model type required, Spice for real time simulation or IBIS and IIMIC for behavioral modeling.

SPICE MODELING

If the software tools uses  SPICE models, the element of the ICEM models have to be directly added to the spice electrical models so that the simulation can take into account the noise due to internal activity of the ICs.

IBIS  MODELING

If the software tool require IBIS modeling format the element can be described as data files and these additional files need to added to the model description under  IBIS format, for example specific keywords.

Evaluation a new IBIS release including EMC modeling is on going based on this document.

IMIC MODELING

Same comment as the ones done on IBIS modeling may be done on IMIC.

Evaluation to include the EMC elements of the ICEM model in the next release has to done.

# Annex 2

REFERENCES

Part 1

EMC TASK FORCE CONTRIBUTORS

Leader 	PERRIN Jean Claude       TEXAS INSTRUMENTS

Members	HUET Claude		    EADS AIRBUS

LEVANT Jean Luc	   	  ATMEL

MAROT Christian  	  	 SIEMENS AUTOMOTIVE

MAURICE Olivier	  	 VALEO

PERRAUD Richard	   	EADS CCR

SAINTOT  Pierre 	  	 ST MICRO ELECTRONIQUE

SOUBEYRAN Amaury	   EADS MSI

University	SICARD Etienne   		  INSA

RAMDANI Mohamed	  ESEO

LUBINEAU Marc	 	 IERSET

Part 2

MUNICH IBIS SUMMIT MEETING