Date:    March 16, 1994

From:    Derrick Duehren (503) 696-4299, fax (503) 696-4904

Derrick_Duehren@ccm.jf.intel.com

Intel Program Manager

(Will Hobb's IBIS Assistant)

Intel Corporation

5200 NE Elam Young Pkwy, Hillsboro, Oregon 97124 USA

and

Will Hobbs (503) 696-4369,  fax (503) 696-4210

Intel Modeling Manager

Subject: Minutes from IBIS Open Forum 3/11/94

To:

Anacad                        Steffen Rochel

Ansoft                        Henri Maramis

Atmel Corporation             Dan Terry

Cadence Design                Sandeep Khanna, Chris Reed,

Pawel Chadzynski, Kumar*

Contec                        Maah Sango*, Clark Cochran

Digital Equipment Corp.       Barry Katz*

High Design Technology        Michael Smith

HyperLynx                     Steve Kaufer, Kellee Crisafulli*

IBM                           Jay Diepenbrock

IBM-Motorola alliance         Lynn Warriner, Hoa Quoc*, John Burnett

Integrity Engineering         Greg Doyle, Wayne Olhoft

Intel Corporation             Stephen Peters*, Don Telian, Will Hobbs*

Arpad Muranyi, Derrick Duehren*

Interconnectix, Inc.          Bob Ross*

Intergraph                    Ian Dodd*, David Wiens*

IntuSoft                      Charles Hymowitz

Logic Modeling Corp.          Randy Harr

Mentor Graphics               Greg Seltzer, Ravender Goyal

Meta-Software                 Mei Wong, Mei-Ling Wei

MicroSim                      Arthur Wong, Jerry Brown, Graham Bell

National Semiconductor        Syed Huq*

North Carolina State U.       Paul Franzon, Michael Steer, Steve Lipa*

Performance Signal Integrity  Vivek Raghawan, Eric Bracken*

Quad Design                   Jon Powell*

Quantic Labs                  Mike Ventham*, Zhen Mu

Racal-Redac                   John Berrie

Siemens Nixdorf               Werner Rissiek, Olaf Rethmeier

Texas Instruments             Bob Ward*

Thomson-CSF/SCTF              Jean Lebrun

Zeelan Technology             Hiro Moriyasu, George Opsahl

CC:

Intel Corporation             Randy Wilhelm, Jerry Budelman,

Intel IBIS team

In the list above, attendees at the 2/18/94 meeting are indicated by *

Upcoming Meetings:  Date:      Bridge:          Res:

4/1/94     (415) 904-8944   661905

4/22/94    TBD

5/13/94    TBD

6/3/94     TBD

6/24/94    TBD

All meetings are 8:00 AM to 10:00 AM PST (16:00 to 18:00 UTC).  We try to

have agendas out 7 days before each open forum and meeting minutes out

within 7 days after.  When you call into the meeting, ask for the IBIS

Open Forum and give the bridge operator the reservation number.

Please note:  If you know of someone new who wants to join the e-mail

reflector (ibis@vhdl.org) send e-mail to ibis-request@vhdl.org.

------------------------------------------------------------------------------

## 3/11/94 Meeting Agenda

## ----------------------

8:00  Check-in

Intros of new IBIS participants                 Hobbs

Review of 2/18/94 minutes                       Hobbs

Miscellany/Announcements                        Hobbs

Opens for new issues (reshuffle last 5 items?)  All

Treasurer's Report                              Hobbs

Press updates                                   Hobbs/All

Progress toward enlisting new IC vendors        All

IBIS 2.0 Ratification                           Hobbs

-  DAC Conference 6/6 - 6/10

-  Steering Committee (5/20? Who? Where?)

IBIS Cookbook                                   Hobbs

Spice-to-IBIS Converter                         Lipa (NCSU)

8:30  Stnd. driver, quality of support, validation    Hobbs, All

BIRD 8, Spec. of V/I data monotonicity          Crisafulli

BIRD 9, Other model types                       Ross

BIRD 2, VIH, VIL Thresholds for Inputs          Powell

Egg 1, mutual pin coupling (ready to hatch?)    Bracken

9:00  Simulation temperatures (new BIRD?)             Warriner

Ramp measurement                                Reid, Ross, et. al.

Egg 2, ramp table?                              Hobbs

9:30  Canright paper                                  Ward

Formal BNF notation (BIRD?)                     Reed, Harr

High freq. and EMI                              Goyal, et. al.

Phased turn-on/off of multiple devices          Powell

9:55  Wrap-up, Next Meeting Plans                     Hobbs

## Minutes

## -------

### 1.  Intros of new IBIS participants

New participants: None.

There were no corrections made to last month's minutes.  Several open ARs (attention required) are rolled into this meeting minutes.

### 2.  Miscellany/Announcements

No new announcements.

### 3.  Opens for new issues (reshuffle last 5 items?)

Kellee: A large number of new people are creating IBIS models.  Nearly all have put the sign in wrong in at least one of the V/I tables (both diode and pull-up/pull-down tables).  Most of the errors are simple mistakes.  We need a quick update to the IBIS check program to check for correct polarity in these tables.  Kellee then made a proposal for checking these tables and issuing a warning message.

Jon Powell offered to change the parser, Kellee outlined how to do it.  It involves sorting the data and analyzing for polarity.

AR Kellee: Write an emergency BIRD for the change.

Approval will be attempted by the reflector within 1 week.

AR Will: Administer the voting on the emergency BIRD.

AR Jon: Make the change to the parser, once the BIRD is approved by the forum

and pass it to Will Hobbs for distribution to those who have already

paid for the parser source code.  (Note: Will is uncomfortable with

posting the encrypted source to vhdl.org.  Will's address is:

US Mail:                     Private Mail Delivery (FedEx):

Will Hobbs                   Will Hobbs

M/S JF1-57                   M/S JF1-57

5200 NE Elam Young Pkwy      2111 NE 25th St.

Hillsboro, OR 97124          Hillsboro, OR 97124

Jon heard that the RS6000 version of Golden Parser doesn't work, but the update is that it was cockpit error.

### 4.  Treasurer's Report

Will has inquired with the Intel legal dept. regarding the Tax ID issue and is waiting for a response.  A check for the golden parser is pending, waiting for the Tax ID.  Another check has arrived for the GP.  Will wants approval from the buyer before mentioning who it is.

### 5.  Press updates

In the IEEE Spectrum publication, Pg. 54 of March issue; there is an article on managing EMI by Ravender Goyal that references IBIS and most of the participating companies.

### 6.  Progress toward enlisting new IC vendors

Will noted that he is seeing more IC vendors being added to the IBIS reflector.

Jon: First-try PowerPC IBIS models have been received.

Hoa Quoc: PowerPC IBIS model is available now and he can post them to vhdl.org.

AR Derrick/Randy: send instructions for posting IBIS models to vhdl.org.

Bob Ward: TI is getting closer, IBIS is a "good thing".  Full buy-in should come soon.

Syed: In process of convincing National Semiconductor upper management, but may have to wait for 2.0 for differential support before they can implement.

Barry: Just a matter of time for Digital.  The IBIS cookbook is needed.

Will: The Intel DX4(TM) processor now has a public IBIS model; also several Intel960(TM) family IBIS models are publicly available.  Will is working to get Pentium(TM) family of processors available, but it may take a while.  Will and Derrick will be putting the new Intel models on vhdl.org.  Another set of chipset IBIS models should be coming soon.

### 7.  IBIS 2.0 Ratification

### -  DAC Conference 6/6 - 6/10

After discussing the potential schedule of the conference, we decided to

have Jon and Derrick decide on the two days and post them soon so travel

plans can be made.

AR Derrick and Jon: Decide and post.

### -  Steering/Editing Committee (5/20? Who? Where?)

Bob Ross, Stephen, Kellee, Jon, Will, Syed volunteered to participate

May 20, in Portland, OR.  It is open to anyone that enjoys editing stuff

and making it more logical and comprehensible, and wants to visit

Portland in May.  The output of this committee will be posted to the

reflector for general comment and review before the DAC session, so

nothing will be final until everyone has had a chance to make their

opinions known.

AR Derrick: Make arrangements for May 20 meeting.

### 8.  IBIS Cookbook

Intel's internal Cookbook is nearly complete.  We have an Intel tech writer (Robin R.) available for 1 week to create a generic base document from which the forum can build.  Should be avail in 4 wks.  Kellee will review.

### 9.  Spice-to-IBIS Converter

Steve Lipa:  In holding mode with SPICE-to-IBIS converter because SPICE wasn't working on Sun and RS6000 workstations until yesterday.  They want to hold off until it is tested on all major platforms.  All development is done on DEC 2100's and 3100's.  Steve is writing Man pages and has added all min/max tables except ramp table.  (Steve wants feedback before doing ramp table.)

AR Steve: Post the Man page drafts for forum review. (Done)

Steve has Berkeley SPICE version 3 and version 4, and has not tried any proprietary versions (Hspice, etc.).  Kellee proposed sending one of his IBIS models to the reflector and have the forum try it for sanity check.  He will "put" the executable at \incoming on vhdl.org.

Steve needs a non-Berkeley spice simulator for IBIS model.  Kellee offered to provide one.

AR Steve: Put your IBIS output file to the reflector.  Add test vectors.

Others can test it and provide feedback.

Bob Ross asked if NCSU had made any progress on the Lex/Yacc IBIS parser.  Steve said that Michael has it pretty much working.  It will be posted publicly soon.

### 10. Stnd. driver, quality of support, validation

Agreement from last meeting:  Companies will self-certify (see 2/18/94 minutes).

Old AR Will: See if Intel/Don Telian can release waveforms for the

simulations in the Overview doc.  The data should end up in

Postscript format so the curves can be universally accessed.

Old AR Everyone: If, and when you self-certify (run the test suite), send email

to Derrick_Duehren@ccm.jf.intel.com so Derrick can add the

information to the Participants Roster document.  Include what

IBIS version level you support.

### 11. BIRD 8, Spec. of V/I data monotonicity

The existing BIRD has a few limitations.  Many feel it needs to be more restrictive.  Kellee is waiting to hear from others, but feels that monotonicity should be in one axis only (voltage).  Jon said that the designs that were non-monotonic in current were abandoned because they rang too much.  Many felt that we could restrict both axes to be monotonic, because we currently don't have any time dependency support (hysteresis or fold-back), and therefore we couldn't model it anyway.  Kumar and Maah feel we should make the one-axis restriction conditional.

Stephen drew a picture of non-monotonic I with monotonic V (similar to below) that looked real.  Bob Ward knows of one but is not sure if he can make it available.

|                              *

|               *          *

I  |           *      *   *

|       *

|   *

|______________________________

V

The Forum decided to leave monotonicity on voltage axis and call it good.

AR Kellee: Update BIRD 8 accordingly.

### 12. BIRD 9, Other model types

Bob Ross discussed bird 9.1.  He added series and AC terminators.  Discussion indicated some confusion about whether the new model types were inside or outside IC package (tied in with bird 2.1 questions).

Kellee: "Are we degenerating from original IBIS proposal to stay away from trying to model resisters and capacitors?"

Bob: We need to constrain the IBIS model.

Kellee: Suggests a small modification to current model; add series resisters.  "Maybe we need building blocks that we can add together?"

Bob might create a BIRD to handle the series resistor as a separate model.

No conclusions.  Continue discussion on reflector.

"IBIS is a bird that thrives in muddy waters", J. Powell.

### 13. BIRD 2.1 VIH, VIL Thresholds for Inputs

We briefly discussed BIRD 2.1 and noted some conflicts with the recently passed BIRD 7.2.

AR Jon: Reconcile BIRD 2.1 with BIRD 7.2 and come out with BIRD 2.2 or a new

BIRD.

### 14. Egg 1, mutual pin coupling (ready to hatch?)

Not discussed.

### 15. Simulation temperatures (new BIRD?)

Lynn not present

### 16. Ramp measurement

Not discussed.

### 17. Egg 2 ramp table?

Will suggests this Egg.  No time to discuss.

### 18. Canright paper

Not discussed.

### 19. Formal BNF notation (BIRD?)

Steve Lipa: The BNF parser from Michael Steer is working, but Steve has been involved on other things.  NCSU uses it as their parser.

### 20. High freq. and EMI

Not discussed.

### 21. Phased turn-on/off of multiple devices

Not discussed.

### 22. Wrap-up, Next Meeting Plans

AR Derrick: Schedule phone bridge connections for future meetings.  Next is 4/1/93.