Date:    May 4, 1994

From:    Will Hobbs (503) 696-4369, fax (503) 696-4210

Will_Hobbs@ccm2.jf.intel.com

XTG Modeling Manager, Intel Corp., Chairperson, IBIS Open Forum

Intel Corporation

5200 NE Elam Young Pkwy, Hillsboro, Oregon 97124 USA

and

Derrick Duehren (503) 696-4299, fax (503) 696-4904

Derrick_Duehren@ccm.jf.intel.com

Intel Program Manager

Subject: Minutes from IBIS Open Forum 4/29/94

To:

Anacad                        Steffen Rochel

Ansoft                        Henri Maramis

Atmel Corporation             Dan Terry

Cadence Design                Sandeep Khanna, Chris Reed,

Kumar*

Contec                        Maah Sango, Clark Cochran

Digital Equipment Corp.       Barry Katz*

High Design Technology        Michael Smith

HyperLynx                     Steve Kaufer, Kellee Crisafulli*

IBM                           Jay Diepenbrock

IBM-Motorola alliance         Lynn Warriner, Hoa Quoc*, John Burnett

Integrity Engineering         Greg Doyle, Wayne Olhoft

Intel Corporation             Stephen Peters*, Don Telian, Will Hobbs*

Arpad Muranyi*, D. Duehren*, R. Rosenbaum*

Interconnectix, Inc.          Bob Ross*

Intergraph                    Ian Dodd, David Wiens

IntuSoft                      Charles Hymowitz

Logic Modeling Corp.          Randy Harr

Mentor Graphics               Greg Seltzer, Ravender Goyal

Meta-Software                 Mei Wong, Mei-Ling Wei

MicroSim                      Arthur Wong, Jerry Brown, Graham Bell

National Semiconductor        Syed Huq*

North Carolina State U.       Paul Franzon, Michael Steer, Steve Lipa

Performance Signal Integrity  Vivek Raghawan, Eric Bracken*

Quad Design                   Jon Powell*

Quantic Labs                  Mike Ventham, Zhen Mu

Racal-Redac                   John Berrie

Siemens Nixdorf               Werner Rissiek, Olaf Rethmeier

Texas Instruments             Bob Ward*

Thomson-CSF/SCTF              Jean Lebrun

Zeelan Technology             Hiro Moriyasu, George Opsahl*

CC:

Intel Corporation             Randy Wilhelm, Jerry Budelman,

Intel IBIS team

In the list above, attendees at the meeting are indicated by *

Upcoming Meetings: The room and bridge numbers for future IBIS teleconferences are listed below:

Date        Bridge Number   Reservation #

5/13/94    (415) 904-8944   721619

5/20/94    Face-to-face, editing committee

5/27/94    (415) 904-8800   744468

All meetings are 8:00 AM to 10:00 AM PST (15:00 to 17:00 UTC).  We try to have agendas out 7 days before each open forum and meeting minutes out within 7 days after.  When you call into the meeting, ask for the IBIS Open Forum and give the bridge operator the reservation number.

If you know of someone new who wants to join the e-mail reflector (ibis@vhdl.org) send e-mail to ibis-request@vhdl.org.

NOTE: "AR" = Action Required.

------------------------------------------------------------------------

## 4/15/94 Meeting Agenda

## ----------------------

Check-in

Intros of new IBIS participants                       Hobbs

Review of previous meeting's minutes                  Hobbs

Miscellany/Announcements                              Hobbs

Opens for new issues                                  All

Opens for new issues                                  All

New models available                                  All

IBIS 2.0 Ratification/DAC/Other 2.0 Issues            Hobbs

-  Birds of a Feather session (6/8)

-  Rev 2.0 Ratification Summit (6/9)

-  Levels of support (see 12/3/93 discussion)

IBIS Cookbook                                         Hobbs, Rosenbaum

BIRD 8, Spec. of V/I data monotonicity                Crisafulli

BIRD 9.2,  Terminator Specification                   Ross

BIRD 10, Coupling effects in package models           Bracken

BIRD 11, Sign of current checking                     Hobbs, All

BIRD 12, Non-Linear Ramps                             Peters

BIRD 2, VIH, VIL Thresholds for Inputs                Powell

Egg 3, Simulation temperatures                        Ward

The rest of the nest (Eggs 4, 5...)                   All

Wrap-up, Next Meeting Plans                           Hobbs

------------------------------------------------------------------------

### 1.  Intros of new IBIS participants

New participants: None.

### 2.  Review of Pervious Meeting's Minutes

There were no corrections made to last month's minutes.

The following correction was made to last month's minutes: In item 12, "Bob W." should be "Bob R."

### 3.  Miscellany/Announcements

Barry believes ad-hoc nature of ibis forum causes problems for DEC.  He prefers $500 dues, rather than a source license (easier for corporations to justify).  Will gave a short history of the Golden Parser licensing.  We will discuss this issue at the ratification meeting.

IBIS is mentioned briefly in Steve Ohr's column in Computer Design, April 1994, pg. 112.

The vhdl.org \birds directory is up-to-date as of 5/3/94.

### 4.  Opens for new issues

BIRDs 13 and 14 added to the agenda.

### 5.  New models available

The IntelDX4(TM) processor and Pentium(TM) processor-to-PCI bridge component IBIS files have been posted to vhdl.org.

### 6.  IBIS 2.0 Ratification/DAC

-  May 20 Editing Committee

AR Derrick: Get room, time, hotel info out to May 20 participants

within next week.  DONE

The agenda for the ratification meeting will be published after

the May 20 meeting.

Old AR Will: Engage Paul Munsey after 5/20 for parsability study

of the proposed version 2.0 to minimize a need for a version 2.1

soon after 2.0.

-  Rev 2.0 Ratification Summit (6/9/94)

Kumar has arranged a room near the conference center for 30 to

40 people.

-  Birds of a Feather session (6/8/94)

No discussion.  Arrangements are made for a Marriott room.

-  Levels of support (see 12/3/93 discussion)

Will be discussed at the ratification meeting.

### 9.  IBIS Cookbook

Robin Rosenbuam (an Intel Technical Writer) led a short discussion of the cookbook.  Kellee and Jon received and reviewed copies, and will be giving Robin their comments.  Will asked for volunteers to create new examples for the book; no one volunteered.  Hoa Quoc requested a review copy.  Robin later tried to send it to an email address Hoa provided, but the address failed.

AR Hoa:  Call Robin at (503) 696-4554 to make arrangements.  Also please provide name, address, phone number, etc. to Derrick for the Roster document.

### 11. BIRD 8, Spec. of V/I data monotonicity

We need resolution to Jon and Arpad's disagreement.  Originally, the forum requested that IBIS require monotonicity of data, which Kellee codified in BIRD 8 and later revised to only require one monotonic axis.

Most of the discussion revolved around what is and is not monotonic.  Non-zero?  Non-slope changing?  No change in sign of slope?  Slope going to zero?  We concluded that zero slope is OK, we can allow "flat spots", and could issue a warning for non compliance.

Jon feels nonmonotonic behavior can only take place in the presence of feedback, which implies delay, which IBIS doesn't describe.   Some simulator vendors currently filter out non-monotonic data.

Will proposed: Require monotonic data on at least one axis.  Simulators can issue a warning message to customers if data on the other axis is monotonic.  Customers can put whatever they want in the model.  It would be very difficult to have the GP do the filtering.  We can put a guideline/note in the spec.: "If you have non-monotonic data, some simulators will convert it to monotonic data."  Simulator vendors are responsible for warning user how it will be handled (e.g., filtered out).

Barry wants an example of a nonmonotonic transistor.  Arpad said the irf50 CMOS transistor is an example.

AR Kellee: Update BIRD 8 per this discussion.

### 12. BIRD 9, Other model types

Vote taken: Approved unanimously with the Model_type name changes from "Discrete" to "Terminator".

AR Bob R.: Update the approved BIRD and post to the reflector.

### 13. BIRD 10, Coupling effects in package models

Digital and TI have this data and will likely make it available under this BIRD.

Vote taken: Approved unanimously.

### 14. BIRD 11, Sign of current checking

Vote taken: Approved unanimously.

### 15. BIRD 12, Non-linear Ramps

Stephen Peters clarified some misunderstandings, proposed a few additions to the BIRD.  We need to be able to optionally override the package parameters.

Kellee suggested allowing R_pkg, etc., to be overridden.  He suggested naming the sub-parameters the same as for the package with _dut <device under test> suffix.  Also add a drawing.

The end point of the waveform must line up with the steady DC state current, according to the discussion.  However, measurement errors will inevitably prevent the steady, DC states to not line up.  This is the model creator's problem.  Bob Ward recommends that we add a steady-state agreement as part of the measurement guideline.

Kellee suggested limiting the table length to 100 entries, others agreed.

We discussed supporting more complex loads, and concluded no one would trust the reference waveforms in the model into anything but fairly simple loads, due to low quality transmission line models in older simulators.  The consensus is to keep only simple r, l, c loads, not topologies such as the PCI speedway.

Jon: We need to figure out a way to sync up the curves -- VT curves in different tables, the starting point or some point has to be able to correlate the waveforms in time.  Jon proposed that we create 2 pictures for forum review, one to sync curves, one to ???

AR Stephen:  Reissue the BIRD per this discussion.  Revise names, add c_dut, l_dut, r_dut, set max of 100 data points in the table, set character limits, and add a schematic showing where these elements go.  DONE

### 16. BIRD 2, VIH, VIL Thresholds for Inputs

Vote taken: Approved unanimously.

### 17. BIRD 13, Simulation temperatures

Bob W. put this BIRD out to get discussion going.  This BIRD could migrate into a simple wording-fix BIRD for the IBIS specification.  It clarifies that temperatures are die temperatures, not ambient.  Also, bjt min and max are backwards.

AR Will:  Send note to Bob W. with suggested wording change to Version 1.1 spec.

Discussion to continue via the reflector.

### 18. The rest of the nest (Eggs 4, 5..., BIRD 14)

No discussion.

### 19. Wrap-up, Next Meeting Plans

Next meeting is 5/13/94.