IBISCHK4 Parser Specification and Behaviors

Revision 0.1

By Michael Mirmak, Intel Corp.

Revised October 2, 2005

# Introduction

The following document summarizes the warnings, errors and basic assumptions of the IBISCHK4 IBIS parser program, based on an analysis of the code.  This document should enable model makers, future specification contributors and model users to better understand how the IBIS specification is actually interpreted through the  parser.

Note that the code used for the analysis is 4.1.1.

Non-IBIS related bugs reported by the parser – for example, an inability to write files to directories or allocate sufficient memory – have been omitted from this summary.

Four general types of parser messages are issued: Warnings, Errors, Line-based Warnings and Line-based Errors.  The summaries below preserve this distinction wherever possible.

# General Limits

The following general assumptions are made by the parser in analyzing IBIS files:

- When analyzing V-t tables for matching to I-V data, no interpolation of the I-V data is assumed.  Mismatches greater than 2% but less than 10% will generate a warning message.  Mismatches greater than 10% will generate an error message.

- The maximum value of C_comp permitted without warning is 1 nF.

- The maximum value of package pin capacitance permitted without warning is 1 nF.

- The maximum value of package pin inductance permitted without warning is 1 uH.

- The maximum value of package pin resistance permitted without warning is 1000 ohms.

- The maximum value of [Ramp] dt permitted without warning is 1 ms.

# Warning and Error Summary

## [Alternate Package Model]

- ERROR (LINE) Alternate Package Model has no package models defined

- ERROR (LINE) Invalid Package Model name 'xxx' is too long.

- ERROR (LINE) Missing Package Model name

- ERROR (LINE) Package Model Name Previously Defined("xxx")

- WARNING (LINE) Alternate Package Model has only one package model defined

## [Component] and Related Checks

- ERROR Component <xxx> pin <xxx> model <xxx> type <xxx> using [External Model] without corresponding [Diff_Pin]

- ERROR Required keyword 'Component' not found

- ERROR (LINE) column header not found or out of order

- ERROR (LINE) Data on this line appears to be for the Component keyword, which does not allow data on subsequent lines

- ERROR (LINE) Data on this line appears to be for the Manufacturer keyword, which does not allow data on subsequent lines

- ERROR (LINE) Data on this line appears to be for the Package Model keyword, which does not allow data on subsequent lines

- ERROR (LINE) Duplicate [Alternate Package Models] keyword

- ERROR (LINE) Duplicate [Series_Switch_Groups] keyword found

- ERROR (LINE) Duplicate column header 'xxx'

- ERROR (LINE) Duplicate Diff_Pin keyword found

- ERROR (LINE) Duplicate Node Declaration keyword found

- ERROR (LINE) Duplicate Package keyword

- ERROR (LINE) Duplicate Pin_Mapping keyword found

- ERROR (LINE) Duplicate Pin keyword found

- ERROR (LINE) Duplicate Series_Pin_Mapping keyword found

- ERROR (LINE) Expecting End Alternate Package Models keyword

- ERROR (LINE) Orphan Component keyword

- ERROR (LINE) Orphand at a line keyword

- ERROR (LINE) Unable to find column headers after xxx keyword

- ERROR (LINE) Unexpected End Alternate Package Models keyword

- ERROR (LINE) Unknown column header 'xxx' or out of order

- ERROR (LINE) Unknown column header after function_table_group

- ERROR (LINE) Unknown data after [Alternate Package Models] keyword

- ERROR (LINE) Unknown data after [End Alternate Package Models] keyword

- ERROR (LINE) Unknown data after Package keyword

- ERROR (LINE) Unknown Line after Component Description

- WARNING (LINE) Component 'xxx' contains a blank character

## [Diff Pin]

- ERROR (LINE) Bad value for xxx column

- ERROR (LINE) No Diff_Pin column entry

- ERROR (LINE) No inv_pin column entry

- ERROR (LINE) No tdelay_typ column entry

- ERROR (LINE) No vdiff column entry

- ERROR (LINE) Too many Diff Pin data columns

- WARNING Component 'xxx' Diff_Pin 'xxx' is not unique

- WARNING Component 'xxx' inv_pin 'xxx' is not unique

- WARNING (LINE) Suspicious value for xxx column

- WARNING (LINE) Typ value is not in between Min and Max

## [Driver Schedule]

- ERROR Model 'xxx': The [Driver_Schedule] has no models defined

- ERROR (LINE) At least two delays must be specified

- ERROR (LINE) Driver Schedule Already Defined for this model

- ERROR (LINE) Driver Schedule Missing model Name

- ERROR (LINE) Invalid Model Name in Driver Schedule ("xxx"): Reserved Word

- ERROR (LINE) Model Name ("xxx") previously used in this Driver Schedule

- ERROR (LINE) Model Name in Driver Schedule is the same as enclosing Model ("xxx")

- ERROR (LINE) Model Name Over xxx Characters Long

- WARNING Model 'xxx': The [Driver_Schedule] has only one model defined

## [External Circuit] and Related Checks

## [Model Selector]

## [Model Spec]

## [Model] and Related Checks

## [Package Model]

## [Package]

## [Pin Mapping]

## [Pin]

## [Ramp]

## [Receiver Threshold]

## [Row]

## [Series MOSFET]

## [Series Pin Mapping]

## [Series Switch]

## [Submodel Spec]

## [Submodel]

## [Test Data]

## [Test Load]

## [TT xxx]

## Basic Checks

## EBD Files

## Header

## I-V Tables

## Package Matrix

## RLC Data

## Series Elements

## Waveform Checking

- WARNING  Model 'xxx' : xxx VI curves cannot drive through Vmeas= yyy V given load Rref= zzz Ohms to Vref= aaa V

- WARNING  Model 'xxx' : xxx AC xxx Endpoints (yyy V, yyy V) not within yyy V (zzz %) of (yyy V, yyy V) on VI curves for aaa Ohms to bbb V

- WARNING  Model 'xxx' : xxx AC xxx Endpoints (yyy V, yyy V) *ARE* WITHIN   yyy V (zzz %) of (yyy V, yyy V) on VI curves for aaa Ohms to bbb V

- WARNING  Model 'xxx' : Currents <> 0.0mA in 'Pullup' for Open_drain/Open_sink device

- WARNING  Model 'xxx' : Currents <> 0.0mA in 'Pulldown' for Open_source device

- WARNING  Model xxx data is non-monotonic