IBIS QUALITY SPECIFICATION

Revision 1.1ad (draft)

17 Mar 2008

Purpose

This document is a specification covering standards and methodology to enhance the quality of electronic component model files produced in conformance with the ANSI/EIA-656-A I/O Buffer Information Specification (IBIS). More information on the IBIS specification can be found on the IBIS web page:

http://www.eigroup.org/ibis/default.htm

The purpose of the IBIS Specification is to provide a standard for model data exchange and thus to enhance the value of modeling and simulation.

The purpose of this IBIS Quality Specification is to provide a standard for validating model data against the IBIS Specification and a means of objective measures of correlating model simulation results with measurements or other model simulations. By providing standards for validating, correlating, and replicating simulation results we seek to enhance the value of modeling and simulation.

Neither standard is a means, by itself, for guaranteeing quality. The quality of models and simulations are largely the result of market forces. Standards serve to enhance the exchange of data.

This IBIS Quality Specification is intended to supplement existing support mechanisms for producers of IBIS files. Email reflectors for IBIS community support are open to the public. The IBIS Open Forum also offers a FREE Model Review Service:

IBIS Model Review: If you are developing an IBIS model and would like to have your model reviewed, you may send your model [to the email address found on the IBIS Support web page]. This is a confidential and FREE service provided by the IBIS Model Review committee comprised of a number of EDA vendors. Feedback about the model will only be passed to the requestor of this service. Model Review Service is intended for use by IC MANUFACTURERS ONLY (large or small).

Details on the email reflectors and model review service are described at the web URL given, above.

Revision History

| 1.0a | 31-mar-2004 | Bob Haller | Initial version |
| --- | --- | --- | --- |
| 1.0b | 04-jan-2005 | Mike LaBonte | Review update |
| 1.0c | 15-feb-2005 | Mike LaBonte | Review update |
| 1.0d | 08-mar-2005 | Mike LaBonte | Review update |
| 1.0e | 05-apr-2005 | Bob Haller | Review update |
| 1.0f | 12-apr-2005 | Mike LaBonte | Review update |
| 1.0g | 02-aug-2005 | Mike LaBonte | Review update |
| 1.1a | 14-aug-2006 | Moshiul Haque | Modified section 1, 2, 3, 4 and 5 with the new IQ numbering scheme Added "Receiver Threshold" as an optional requirement in section 4 |
| 1.1b | 31-oct-2006 | Mike LaBonte | Convert to MSWord format. |
| 1.1c | 27-nov-2006 | Mike LaBonte | Review update |
| 1.1d | 12-dec-2006 | Mike LaBonte | Review update |
| 1.1e | 09-jan-2007 | Mike LaBonte | Formatting. More formatting work needed beginning at 4.3.10. |
| 1.1f | 23-jan-2007 | Mike LaBonte | Change “exemption” to “exception”. |
| 1.1g | 6-feb-2007 | Mike LaBonte | Changes to section 2.1. |
| 1.1h | 13-feb-2007 | Bob Ross/Moshiul Haque | Added detail explanation regarding “X” in section 2.1 |
| 1.1i | 20-feb-2007 | Kim Helliwell/Mike LaBonte | Changes to 4.1.3 and 4.1.4. Restore IBISCHK NOTES from 5-jul-2005 version. |
| 1.1j | 20-feb-2007 | Mike LaBonte | Changes from 20-feb-2007 meeting. |
| 1.1k | 27-feb-2007 | Mike LaBonte | Extended Purpose to discuss IBIS support. |
| 1.1l | 26-mar-2007 | Mike LaBonte | Replaced 4.1.4 and 4.3 with material from David Banas. Change 3.1.3 to IQ1. |
| 1.1m | 30-mar-2007 | Mike LaBonte | Change limits in 3.1.2 and add ibischk notes. |
| 1.1n | 16-apr-2007 | Kim Helliwell, Mike LaBonte | Changes to 3.1.2, 3.1.3, 3.1.4 (deleted), and 3.2.1. |
| 1.1o | 24-apr-2007 | Kim Helliwell, Mike LaBonte | Suggest Terminator model in 3.2.1. Merge 3.2.5 into 3.2.4. Deleted 3.1.3, 3.2.2, 3.2.3, 3.2.5. |
| 1.1p | 3-May-2007 | Mike LaBonte | Updated 3.2.5 and 3.3.2. Deleted 3.3.1. |
| 1.1q | 21-May-2007 | Roy Leventhal, Mike LaBonte | Fixed RLC requirement in 3.2.5. New text for 3.3.2. Deleted 3.3.3. Changed 3.3.4 to require a comment for exceptions. |
| 1.1r | 18-Jun-2007 | Moshiul Haque, Mike LaBonte | 3.3.4 becomes LEVEL 2. Deleted 3.4.1. Inserted 3.4.3 and 3.4.4. |
| 1.1s | 19-Jun-2007 | Mike LaBonte | Small changes to 3.4.3 and 3.4.4. |
| 1.1t | 10-Jul-2007 | Mike LaBonte | Deleted 3.4.2. Renumbered so that [Model Selector] checks are in new section 4, all subsequent sections renumbered. |
| 1.1u | 30-Jul-2007 | Bob Ross, Roy Leventhal, Mike LaBonte | Revisions to 5.1.1, 5.1.7, and 5.1.8. Deleted 5.1.2 and 5.1.9 through 5.1.12. |
| 1.1v | 14-Aug-2007 | Mike LaBonte, Roy Leventhal | Revisions to 5.1.7, 5.1.8, 5.2, and 5.2.2. Deleted 5.2.1, 5.2.3, and 5.2.4. |
| 1.1x | 11-Sep-2007 | Mike LaBonte | Updated 5.2.2, 5.2.5, 5.2.6, and 5.2.22. Deleted 5.2.7 and 5.2.8. Further edits to 5.2.2 from Roy Leventhal. |
| 1.1y | 11-Dec-2007 | Roy Leventhal, Mike LaBonte | 5.2.2, 5.2.18. 5.2.3, 5.2.4, 5.2.7, 5.2.8, 5.3.4 updated, 5.2.17, 5.3.3 marked for deletion. Fixed swapped M & S in 5.3.17. |
| 1.1z | 15-Jan-2008 | Mike LaBonte | 5.2.16, 5.2.18 updated. Notes removed from 5.2.21 & 5.2.13. |
| 1.1aa | 22-Jan-2008 | Mike LaBonte | Change all “datasheet” to “data sheet”. Change some check titles to “present and matches data sheet”. Change 5.1.4 from “correct” to “reasonable”. Merged “present” checks and “correct” checks for [Receiver Thresholds]. Minor editorial changes. |
| 1.1ab | 29-Jan-2008 | Mike LaBonte | Wording changes in 5.2.18, 5.2.20, 5.2.24. |
| 1.1ac | 20-Feb-2008 | Mike LaBonte | Cleanup of 5.3, 5.3.1 marked for deletion, 5.3.2 clarified. |
| 1.1ad | 17-Mar-2008 | Mike LaBonte | 5.2.5 clarified. 5.3.4, 5.3.5, 5.3.6, and 5.3.7 updated. |

Table of Contents

1.	IBIS Quality Summary	9

1.1.	IBIS Quality Level Definitions	9

1.1.1.	IQ0 – Not Checked	9

1.1.2.	IQ1 – Passes IBISCHK	9

1.1.3.	IQ2 – Suitable for Waveform Simulation	10

1.1.4.	IQ3 – Suitable for Timing Analysis	10

1.1.5.	IQ4 – Suitable for Power Analysis	10

1.2.	Special Designators	11

1.2.1.	Designator "M" – Measurement Correlated	11

1.2.2.	Designator "S" – Simulation Correlated	11

1.2.3.	Designator "X" - Exceptions	11

1.3.	IBIS Quality summary as comments in file	11

2.	General Header Section Requirements	13

2.1.	{LEVEL 1} IBIS file passes IBISCHK	13

2.2.	{LEVEL 2}  Do not use [Comment Char]	14

2.3.	{LEVEL 2}  [File Name] is correct	14

2.4.	{LEVEL 2}  [File Rev] is correct	14

2.5.	{LEVEL 2}  [Date] is correct	14

2.6.	{LEVEL 2}  [Source] is complete	14

2.7.	{LEVEL 2}  [Notes] is complete	15

2.8.	{OPTIONAL} [Disclaimer] and [Copyright]	15

3.	Component Section	15

3.1.	Component Package Requirements	15

3.1.1.	{LEVEL 2}  [Package] must have typ/min/max values	15

3.1.2.	{LEVEL 2}  [Package] Parasitics must be reasonable	15

3.1.3.	{LEVEL 1}  [Define Package Model] present if [Package Model] is present	16

3.1.4.	{LEVEL 2}  [Package] min/max parasitics consistent with signal pin RLC	16

3.2.	Component Pin Requirements	16

3.2.1.	{LEVEL 2}  [Pin] section complete	16

3.2.2.	{LEVEL 2}  [Pin] model names not too long	16

3.2.3.	{LEVEL 2}  [Pin] models present in file	17

3.2.4.	{OPTIONAL} [Pin] RLC complete	17

3.2.5.	{LEVEL 3}  [Pin] RLC parasitics are present and reasonable	17

3.3.	Component Diff Pin Requirements	17

3.3.1.	{LEVEL 2}  [Diff Pin] referenced pins exist	17

3.3.2.	{LEVEL 3}  [Diff Pin] Vdiff and Tdelay_* complete and reasonable	17

3.3.3.	{LEVEL 3}  [Diff Pin] Vdiff and Tskew correct	18

3.3.4.	{LEVEL 2}  [Diff Pin] referenced pin models matched	18

3.4.	[Model Selector] Requirements	18

3.4.1.	{LEVEL 2}  [Model Selector] referenced [Model]s exist	18

3.4.2.	{LEVEL 2}  [Model Selector] first [Model] is default	18

4.1.	{LEVEL 2} [Model Selector] entries have reasonable descriptions	18

4.2.	{LEVEL 2} Default [Model Selector] entries are consistent	18

5.	Model Section	19

5.1.	Model General Requirements	19

5.1.1.	{LEVEL 2}  [Model] parameters have correct typ/min/max order	19

5.1.2.	{LEVEL 2}  [Model] Model_type	19

5.1.3.	{LEVEL 2}  [Model] C_comp is reasonable	19

5.1.4.	{LEVEL 2}  [Model] C_comp is reasonable	20

5.1.5.	{LEVEL 3S} [Model] C_comp SPICE correlation	20

5.1.6.	{LEVEL 3M} [Model] C_comp laboratory correlation	20

5.1.7.	{LEVEL 2}  [Temperature Range] is reasonable	20

5.1.8.	{LEVEL 2}  [Voltage Range] or [* Reference] is reasonable	21

5.1.9.	{LEVEL 3}  [Pullup Reference] is reasonable	21

5.1.10.	{LEVEL 3}  [Pulldown Reference] is reasonable	21

5.1.11.	{LEVEL 3}  [POWER Clamp Reference] is reasonable	21

5.1.12.	{LEVEL 3}  [GND Clamp Reference] is reasonable	22

5.1.13.	{LEVEL 3}  [Model] timing test load subparameters complete	22

5.2.	Model Switching Behavior Requirements	22

5.2.1.	{LEVEL 2}  [Model] Vinl and Vinh complete	22

5.2.2.	{LEVEL 3}  [Model] Vinl and Vinh reasonable	22

5.2.3.	{LEVEL 3}  [Model] Vinl and Vinh enclose Vmeas	23

5.2.4.	{LEVEL 3}  [Model] Vmeas matches data sheet	23

5.2.5.	{LEVEL 3}  [Model Spec] Vinl and Vinh reasonable	23

5.2.6.	{LEVEL 3}  [Model Spec] Vinl+/- and Vinh+/- complete and reasonable	23

5.2.7.	{LEVEL 2}  [Model Spec] Vinl+/Vinh+ greater than Vinl-/Vinh-	23

5.2.8.	{LEVEL 3}  [Model Spec] Vinl+/- and Vinh+/- enclose Vmeas	23

5.2.9.	{LEVEL 3}  [Model Spec] Pulse subparameters complete	24

5.2.10.	{LEVEL 3}  [Model Spec] Pulse_high greater than Vinh	24

5.2.11.	{LEVEL 3}  [Model Spec] Pulse_low less than Vinl	24

5.2.12.	{LEVEL 3}  [Model Spec] Pulse_time reasonable	24

5.2.13.	{LEVEL 2}  [Model Spec] S_Overshoot subparameters complete	24

5.2.14.	{LEVEL 2}  [Model Spec] S_Overshoot subparameters match data sheet	24

5.2.15.	{LEVEL 2}  [Model Spec] S_Overshoot subparameters track typ/min/max	24

5.2.16.	{LEVEL 2}  [Model Spec] D_Overshoot subparameters complete	24

5.2.17.	{LEVEL 3}  [Model Spec] D_Overshoot subparams exceed S_Overshoot	25

5.2.18.	{LEVEL 3} [Receiver Thresholds] Vth present and matches data sheet, if needed	25

5.2.19.	{LEVEL 3} [Receiver Thresholds] Vth_min and Vth_max present and match data sheet, if needed	25

5.2.20.	{LEVEL 3} [Receiver Thresholds] Vinh_ac, Vinl_ac present and match data sheet, if needed	25

5.2.21.	{OPTIONAL} [Receiver Thresholds] Vinh_ac, Vinl_ac match with datasheet	26

5.2.22.	{LEVEL 3} [Receiver Thresholds] Vinh_dc, Vinl_dc present and match data sheet, if needed	26

5.2.23.	{OPTIONAL} [Receiver Thresholds] Vinh_dc, Vinl_dc match with datasheet	26

5.2.24.	{LEVEL 3} [Receiver Thresholds] Tslew_ac/Tdiffslew_ac present and match data sheet, if needed	26

5.2.25.	{OPTIONAL} [Receiver Thresholds] Tslew_ac/Tdiffslew_ac match with datasheet	26

5.3.	Model I-V Table Requirements	26

5.3.1	{LEVEL 2}  I-V tables complete	27

5.3.2	{LEVEL 3}  I-V tables have correct typ/min/max order	27

5.3.3	{LEVEL 3}  I-V tables have reasonable numerical range	27

5.3.4.	{LEVEL 3}  [Pullup] voltage sweep range is correct	28

5.3.5.	{LEVEL 3}  [Pulldown] voltage sweep range is correct	28

5.3.6.	{LEVEL 3}  [POWER Clamp] voltage sweep range is correct	28

5.3.7.	{LEVEL 3}  [GND Clamp] voltage sweep range is correct	28

5.3.8.	{LEVEL 3}  I-V tables do not exhibit stair-stepping	28

5.3.9.	{LEVEL 3}  Combined I-V tables are monotonic	28

5.3.11.	{LEVEL 3}  [Pullup] I-V tables pass through zero/zero	29

5.3.12.	{LEVEL 3}  No leakage current in clamp I-V tables	29

5.3.13.	{LEVEL 3}  Clamp I-V behavior not double-counted	29

5.3.14.	{LEVEL 3}  On-die termination modeling documented	29

5.3.15.	{LEVEL 3}  ECL models I-V tables swept from -Vdd  to +2 Vdd.	29

5.3.16.	{LEVEL 3} Point distributions in IV curves should be sufficient	30

5.3.17.	{LEVEL 3} Correlate IV curves to combined curves.	30

5.4.	Model V-T Table Requirements	30

5.4.1.	{LEVEL 2}  V-T table endpoints consistent with I-V tables	30

5.4.2.	{LEVEL 3}  V-T tables look reasonable	30

5.4.3.	{LEVEL 3}  Model simulation successful	30

5.4.4.	{LEVEL 2}  Document known model limitations	30

5.4.5.	{LEVEL 3}  Output and IO buffers have should have 2 sets of V-T tables	30

5.5.	Model Ramp Data Requirements	31

5.5.1.	{LEVEL 2}  Output and IO buffers have a [Ramp] section	31

5.5.2.	{LEVEL 3}  [Ramp] R_load present if value other than 50 ohms	31

5.5.3.	{LEVEL 3}  [Ramp] test fixture has no reactives	31

5.5.4.	{LEVEL 3}  [Ramp] typ/min/max order is correct	31

5.5.5.	{LEVEL 3}  [Ramp] data dv and dt values positive	31

5.5.6.	{LEVEL 3}  [Ramp] dV consistent with supply voltages	32

5.5.7.	{LEVEL 3}  [Ramp] dV consistent with V-T table endpoints	32

5.5.8.	{LEVEL 3}  [Ramp] dt is consistent with 20%-80% crossing time	33

5.5.9.	{LEVEL 3}  [Ramp] dt is consistent with data sheet	33

6.	Possible Errors	33

6.1.	{LEVEL 2} Typ/min/max order of parameters correct	33

6.2.	{LEVEL 3} C_comp checked in both input and output mode	33

6.3.	{LEVEL 2} First/last point of waveforms equal to V_fixture values	34

6.4.	{LEVEL 3} Sufficient points in waveform table	34

6.5.	{LEVEL 3} Minimize waveform lead-in time	34

6.6.	{LEVEL 3} Open_sink/Open_source model with correct Vref, Cref, Rref, Vmeas	34

6.7.	{LEVEL 3} Differential models contain appropriate waveform tables	34

6.8.	{LEVEL 3} Models correspond to data sheet	35

6.9.	{LEVEL 3} Model_type correct for model data	35

6.10.	{LEVEL 3} Open_sink/Open_source model not push-pull	35

6.11.	{LEVEL 3} All pins consistent with data sheet	35

7.	Correlation {level 2}	35

7.1.	SPICE Correlation	35

7.1.1.	Purpose of SPICE Correlation	35

- IBIS Quality Summary

The quality of an IBIS file can be determined by checking its data for correctness, and by correlating the data to a reference. Correctness is defined as conforming to a designated version of the IBIS Specification and the component data sheet. A number of individual checks are performed, and the overall file quality is represented with a designator such as “IQ3S”, which would mean data for basic simulation and timing analysis have been checked, and the IBIS model has been correlated to a reference simulation. Information from the checking process should be embedded in the IBIS file as comments.

  - IBIS Quality Level Definitions

The quality level is defined as a combination of correctness checks and correlation checks. The correctness level is a number, and other special designations such as correlation are shown as appended letters. Some examples:

- IQ0	- No IQ checking at all.

- IQ1	- Passes IBISCHK without errors or unexplained warnings.

- IQ2	- IQ1 + data for basic simulation checked.

- IQ3	- IQ2 + data for timing analysis checked

- IQ4	- IQ3 + data for power analysis checked

- IQ3M	- IQ3 + correlated against hardware measurements

- IQ3MS	- IQ3 + correlated against measurements and simulation

- IQ4X	- IQ4, but exception(s) to check(s) commented in file

The 5 recognized levels of correctness checks and 3 levels of correlation checks are discussed below. Details of the referenced checks and correlation tests are given in sections 2 through 7.

    - IQ0 – Not Checked

An IQ0 file has not been checked, or at least the checking has not been documented. This is a placeholder level useful for showing which files are queued for checking. Tools that create IBIS files should put IQ0 comments in the flies.

    - IQ1 – Passes IBISCHK

An IQ1 file has been checked with IBISCHK parser 3.2.9 or later.

- The version of ibischk used must be documented in the Quality Summary.

- IBISCHK must report 0 Errors

- All IBISCHK warnings must be explained if they cannot be eliminated. Ideally, there should be no warnings, but it is recognized that some warnings cannot be eliminated. It is not necessary to flag exceptions with an IQ1X designation in this case.

    - IQ2 – Suitable for Waveform Simulation

An IQ2 file can be simulated with reasonable assurance that the buffer signal waveforms are correct. It does not necessarily have accurate per-pin or coupled package modeling, may not have information needed to check timing, and may not have information to help measure power currents. IQ2 includes all items in IQ1, plus the following checks:

(replace with table of specific checks)

- All pins must be defined and validated for correct logical/physical/model mapping.

- [Model selector]s must be validated.

- [Diff Pins] must be present if required.

- C_comp values must be checked.

- Component [Package] parasitic values reasonable

- V-T tables must be defined for all output drivers.

- All I-V and V-T tables must be inspected.

- On-die termination must be properly modeled if present.

- Typ/Min/Max values must be present and in correct order for all tables and parameters.

- Ramp Data must be validated in accordance with IBIS spec.

- Ramp data must be validated against appropriate V-T tables if available.

    - IQ3 – Suitable for Timing Analysis

An IQ3 file is suitable for signal timing analysis. Package modeling at the pin level is present and accurate, and special keywords for measuring timing are present and correct. Coupled package modeling is not required. IQ3 includes all items in IQ2, plus the following checks:

(replace with table of specific checks)

- Vinl/Vinh must be defined on receivers

- Standard Load/Vmeas must be defined on drivers.

- Must have at least pin RLC parasitics.

- All package parasitics must be checked.

- All model spec waveforms and standard load parameters must be defined and validated.

- All output models must be simulated into standard load and switch through VMEAS.

    - IQ4 – Suitable for Power Analysis

An IQ4 file is suitable for power analysis. The power and ground currents associated with groups of buffers are accurately modeled. This is distinct from the signal analysis capabilities addressed by IQ2 and IQ3. IQ4 includes all items in IQ3, plus the following checks:

(replace with table of specific checks)

- [Pin Mapping] must be present and correct

- Need BIRD95 to be fully capable

  - Special Designators

    - Designator "M" – Measurement Correlated

Special designator "M" indicates that measurement correlation has been performed. The details of the correlation are described in section 7.

    - Designator "S" – Simulation Correlated

Special designator "S" indicates that simulation correlation has been performed. The details of the correlation are described in section 7.

    - Designator "X" - Exceptions

Special designator "X" refers to exception from correctness or correlation. Exceptions should be used to declare that the file is suitable for the purpose indicated by the IQ level even though one or more checks are not passed by strict standards. The reason for the exception must be documented in the note section. Before using an IBIS file with the X designator in its IQ level, the model user should open the file and look for comments explaining exceptions.

  - IBIS Quality summary as comments in file

A summary of the quality of the IBIS model(s) contained in an IBIS file should be included in comments at the end of the Header information as prescribed in the IBIS Quality Checklist. These comments can appear after the Notes section in the Header.

The IBIS Quality Summary comments must begin with the string '|IQ' (that is, the comment character followed by "IQ" for IBIS Quality. Each buffer model in an IBIS file can be separately characterized by its quality. The summary should contain a list of each buffer in the file, followed by the quality level for that buffer, followed by any comments the model creator deems necessary.

An overall quality level should be indicated for each component in the file. This will be the minimum of the quality levels of the buffers used by the component.

If the IBIS file contains more than one component, the overall quality of the IBIS file itself should be indicated. This will be the minimum of the quality levels of components contained in the file.

If the file contains only one component, the component and file quality are synonymous, so only the component quality level needs to be indicated.

The Quality Summary should also include a list of all unresolved warnings given by the Golden Parser (ibischk3) for the IBIS file, together with an explanation if necessary.

Example:

|IQ

|IQ Buffer pdu04tv: IQ3SX (update expected 4/1/2003)

|IQ Buffer pdc01tv: IQ2

|IQ Buffer pdisv:   IQ2MX

|IQ Buffer pduv:    IQ3SM

|IQ Buffer pciv:    IQ2

|IQ Buffer pdt04tv: IQ2

|IQ Buffer pdiv:    IQ2

|IQ

|IQ Component tpz773pn: Overall Quality level IQ2

|IQ

|IQ Warning (line 433) Power Clamp Typical data is nonmonotonic

|IQ Warning (line 433) Power Clamp Minimum data is nonmonotonic

|IQ Warning (line 433) Power Clamp Maximum data is nonmonotonic

|IQ

- General Header Section Requirements

Requirements for the header section of the IBIS file, from the beginning of the file to the line before the first [Component] keyword.

  - {LEVEL 1} IBIS file passes IBISCHK

IBIS models are expected to pass the checks performed by the IBISCHK program before they are released to the public. Passing IBISCHK insures that the file will attain at least an IQ1 level designation. The IBISCHK program is found at http://www.eigroup.org/ibis/tools.htm.

A best practice is to insert the full output from the IBISCHK program into the IBIS file as comments, with explanation for any warnings annotated within. When doing this it is important to insure that the added comments do not cause new “line too long” IBISCHK warnings.

In general it is best to use the latest available version of the IBISCHK program, as the latest version may include new tests and fixes for bugs found in the older versions. The IBISCHK program used must, at a minimum, be able to accommodate the [IBIS Ver] of the IBIS file at hand. The [IBIS Ver] used in the IBIS file is set by the model maker based on the set of IBIS keywords required to completely and correctly represent the behavior of the part. The [IBIS Ver] value can be set to at least 3.2 because this version is supported by a wide variety of EDA tools. It should not be necessary to avoid IBIS 3.2 features to achieve compatibility with IBIS tools. Also, version 3.2 includes IBIS keywords that are needed to correctly describe many I/O buffers in use today.

Some IBISCHK warnings may be permissible due to special circumstances regarding the model, but they must be identified in the IBIS file itself in the [Notes] section of the file.  The warnings, along with the reason why they should be considered acceptable must be identified. Also the IQ level designator must include “X” for exceptions due to warnings.

The X (exception) designation is used to document all exception cases that might be important to some user.  These would mostly apply to Warning messages where the model provider gives further information.  The X designation may also apply to cases where the extracted or specification information has been changed, and its impact.  Finally it can also be used for any unusual situation in the parser, where the model information is correct, but the parser still issues Warning or Caution messages. The main point is that the model provider has purposely issued the model with some deviation, and the user needs to know about its details to understand the issues that might arise in using the model.

For some minor deviations including where model data is changed to eliminate Warning messages, the X designation might not be needed.  For example, the X would normally not be used for I-V table regions where some non-monotonic data due to measurement noise or spurious data is removed.  The changed data has minimal impact on model simulations and helps increase model portability.

Occasionally a problem exists with IBISCHK rather than with the model.  While the IBISCHK problem may be fixed in the future, the existing model could be tagged with X and contain a description of its issues and how this may impact how the IBIS model is used. In the event an error is generated due to a specific bug in the IBISCHK, intentional deviation in the model may be permitted to suppress the error in the model provided such deviations are properly documented in the notes section with “X” tagged in the quality designation. This suppression of error is a unique case and should have minimal impact on the accuracy of the model.

A Level 1 Model must NOT produce ANY errors when run through IBISCHK.

Summary:

- The goal is zero errors and zero warnings.

- In some cases zero warnings are not possible.

- Add the “X” designator to the IQ level if there are warnings.

- Document known cases of acceptable warnings.

- Document the version of IBISCHK used.

  - {LEVEL 2}  Do not use [Comment Char]

NOTE: Move this to section 6

If you are using the default comment character, then this keyword is not needed. Changing the comment character is not advised.

  - {LEVEL 2}  [File Name] is correct

NOTE: Move this to section 6

The [File Name] keyword must have the exact name of the IBIS file. File names must not contain spaces and should have a reasonable correlation to the names of the components in the file.  As an example "ddr_ii.ibs" is not a desirable file name, since it does not distinguish the model file from memory devices produced by any number of vendors. A preferable name for the file would be "mt57w512h36b.ibs", as an example.

  - {LEVEL 2}  [File Rev] is correct

NOTE: Move this to section 6

The IBIS specification has recommended usage for file revisions that is rarely used correctly.

The following guidelines are recommended:

|                  0.x     silicon and file in development

|                  1.x     pre-silicon file data from silicon model only

|                  2.x     file correlated to actual silicon measurements

|                  3.x     mature product, no more changes likely

The minor revision number 'x' should change each time the file is released.

  - {LEVEL 2}  [Date] is correct

NOTE: Move this to section 6

The [Date] keyword must correctly reflect the date the file was last modified.  It is recommended that the month and year be written out completely to aid scripted searches.

  - {LEVEL 2}  [Source] is complete

NOTE: Move this to section 6

The [Source] keyword describes the information from which the model was created.  Normal sources are SPICE and bench measurements.  Enough detail should be present to enable the semiconductor vendor to trace back to the original source of the model data at a later date, for example:

[Source] Derived from Spice I/O models, rev 543b, dated 11/15/02 using HSpice 2001.4/Win2K

[Source] Derived from proto-a lab measurements, 9/4/01

  - {LEVEL 2}  [Notes] is complete

NOTE: Move this to section 6

Any additional information that may help the user should be listed in the [Notes] section.  At a minimum, this section should identify:

- The version of IBISCHK used to check the completed model

- Any IBISCHK warnings that are to be considered "acceptable"

- A contact point (name, phone # or email) for model support

- The model's edit history, with a list of past File Rev/Dates and the reason for each model change.

  - {OPTIONAL} [Disclaimer] and [Copyright]

NOTE: Delete this section

The [Disclaimer] keyword can contain some form of legal disclaimer, similar to what might appear on a data sheet.

The [Copyright] keyword normally denotes the legal rights granted by the company that created the file.  Alternatively it can denote the company for which the model was created, if done as a "work for hire".

- Component Section

  - Component Package Requirements

Requirements for the [Package] and [Package Model] sections:

    - {LEVEL 2}  [Package] must have typ/min/max values

The IBIS specification requires Typ values in the [Package] keywords, and allows Min and Max values to be NA if not available. To achieve IQ level 2 an IBIS file must have Typ, Min and Max values in the [Package] keywords. A reflection analysis based only on Typ package parasitics is likely to be optimistic. Min and Max values are required to insure that peak distortion levels are predicted.

    - {LEVEL 2}  [Package] Parasitics must be reasonable

Reasonable values are: L < 100nH, C < 100pF, R < 10 ohm. Min must be less than typ and typ less than max.

The IBISCHK program detects typographical errors such as omitting the scaling factor following a number value, by checking against higher limits: L < 1000nH, C < 1000pF, R < 50 ohm. An R_pkg value of 1000 would easily fail this test, for example. This IQ check, with its lower limits, will detect other errors such as extra digits or measurement error.

The min and max values of these parasitics should represent the range spanned by the actual pin parasitics for signal pins. The typical value should fall between the min and max values; typically it is the average of the signal pin parasitics, but it need not be. The power and ground pins should NOT be included in the determination of the [Package] parasitics.

    - {LEVEL 1}  [Define Package Model] present if [Package Model] is present

The [Define Package Model] keyword must be present if [Package Model] is present, even if contained in a separate .pkg file.

ibischk v4.0.2 detects this:

ERROR - Unable to find Package Model '860_FCBGA' for Component 'I5000'

NOTE: Delete this section

    - {LEVEL 2}  [Package] min/max parasitics consistent with signal pin RLC

NOTE: Delete this section

It is not necessary to check parasitics for each pin.

  - Component Pin Requirements

Requirements for the [Pin] section:

    - {LEVEL 2}  [Pin] section complete

All pins must be defined for a component. In addition to signal pins:

- No Connects must be represented with NC.

- Power pins must be indicated POWER.

- Gnd pins must be indicated GND.

- Special Pins (e.g. analog) are to be represented in the pin map, even if they are marked NC. Explanatory comments are recommended for these. A common practice is to represent analog pins with Terminator models, so that waveforms of crosstalk received at the pins can be viewed in EDA tools.

For IBIS buffer [Model] libraries, it is recommended that one pin be used for every model and that the pin name be the same as the model name.

    - {LEVEL 2}  [Pin] model names not too long

Model names in the [Pin] section should not exceed 20 characters for IBIS version 3.x, 30 characters for 4.x

ibischk v4.0.2 detects this:

ERROR (line   58) - Model name string 'BCM1250_SSTL2_7xxxxxxxxxxxxxxxx' is too long, truncating to 20 characters.

NOTE: Delete this section

    - {LEVEL 2}  [Pin] models present in file

Models referenced in the [Pin] section must exist in the same IBIS file or refer to a Model Selector. Bear in mind that model names in an IBIS file are case-sensitive.

ibischk v4.0.2 detects this:

ERROR - Component 'BCM1250_Rev_AB': Model 'BCM1250_SSTL2_7xxxxx' for Pin 'A4' not defined.

NOTE: Delete this section

    - {OPTIONAL} [Pin] RLC complete

RLC is optional on pins. If not defined either leave blank or use NA NA NA.

NOTE: Delete this section

    - {LEVEL 3}  [Pin] RLC parasitics are present and reasonable

For a LEVEL 2 model, pin parasitics are optional, but they are mandatory for a LEVEL 3 model (that is, a model suitable for timing). To pass this check the RLC values must be present for all signal pins in the [Pin] section, or [Package Model] must be present. Pin parasitics should either be measured or extracted using a 2D or 3D solver. Reasonable signal pin parasitics will result in impedance and delay characteristics that fall in the ranges:

TD = SQRT(LC)   < 300ps

Z0 = SQRT(L/C)  < 100ohm

Note that IQ check 3.1.2. also requires that each [Pin] RLC value falls within the min/max range as given by the [Package] keyword. The [Package] keyword can be adjusted to accommodate.

  - Component Diff Pin Requirements

Requirements for the optional [Diff Pin] section:

    - {LEVEL 2}  [Diff Pin] referenced pins exist

If [Diff Pin] is defined, referenced physical pins must exist in the [Pin] section.

ibischk v4.0.2 detects this:

ERROR - Component 'BCM1250_Rev_AB': Diff_Pin 'V411' not previously declared in Pin section.

NOTE: Delete this section

    - {LEVEL 3}  [Diff Pin] Vdiff and Tdelay_* complete and reasonable

For input and I/O pins Vdiff must be defined, non-zero and positive. For output and I/O pins Tdelay_typ, Tdelay_min, and Tdelay_max data can be zero, but must be defined. Both Vdiff and Tdelay_* are measured relative to the die pads and must not include additional package delays and offsets. Output pins should have NA for Vdiff. For input pins Tdelay_typ, Tdelay_min, and Tdelay_max must be NA.

    - {LEVEL 3}  [Diff Pin] Vdiff and Tskew correct

Vdiff and Tskew must correspond to data sheet.

NOTE: Delete this section

    - {LEVEL 2}  [Diff Pin] referenced pin models matched

It is expected that both pins of a differential pair will use the same [Model]. If the buffer models referenced by the two physical pins of a [Diff Pin] entry are not the same [Model] name, a comment or [Notes] entry must be present to explain why.

  - [Model Selector] Requirements

Requirements for the optional [Model Selector] section:

    - {LEVEL 2}  [Model Selector] referenced [Model]s exist

All [Model]s referenced by a [Model Selector] must be present in the same IBIS file.

NOTE: Delete this section

    - {LEVEL 2}  [Model Selector] first [Model] is default

The default [Model] reference should be listed fist in each [Model Selector] section.

NOTE: Delete this section

- Model Selector Section

  - {LEVEL 2} [Model Selector] entries have reasonable descriptions

Each line the [Model Selector] keyword must have two fields. The first field lists the referenced [Model] name and second field contains a short description of the model shown in first field. The purpose of the description is to aid the user of the EDA tool in making intelligent buffer model selections. It can be used by the EDA tool in a user interface dialog box as the basis of an interactive buffer selection mechanism. An example of usage of model selector with an appropriate description might be a programmable buffer in DDR3, where the description may include the impedance of the driver and the applicable maximum frequency of the model (800 Mbps, 34 Ohm Data I/O with no ODT). It is recommended to have a specific description in the [Notes] section about the existence of [Model Selector] in an IBIS file with a short explanation of why it was used, possibly with references to the data sheet for further explanation.

  - {LEVEL 2} Default [Model Selector] entries are consistent

The first entry under each [Model Selector] keyword is the default entry. They will be the models used by EDA tools if the user makes no specific choice. The set of default entries should be consistent, describing a state that may likely exist on the part, considering dependencies between them. For example, if the [Model Selector] entries describe controlled output impedances for a part on which all buffers are set to the same impedance, then the default entries for all [Model Selector]s would have the same impedance. We must not have one [Model Selector] defaulting to 35 ohms and another to 50 ohms in that case. Furthermore, the most frequently used setting of a [Model Selector] is the preferred default, if that can be determined.

- Model Section

  - Model General Requirements

    - {LEVEL 2}  [Model] parameters have correct typ/min/max order

For [Model] parameters, Min corresponds to the conditions for weak/slow buffers, Max corresponds to conditions for strong/fast buffers. These conditions are controlled by buffer selections for process, temperature and voltage conditions.

Normally all keywords and subparameters scoped by the [Model] keyword with typ/min/max data has three columns corresponding to typical, weak/slow, and strong/fast, in order. The major exception is C_comp (including C_comp_*), which uses the numerically lowest value for Min, and numerically highest for Max. The highest [Temperature] value may fall into the Min column or the Max column, depending on technology. For CMOS technology the highest [Temperature] usually results in slow/weak operation, and would appear in the Min column.

    - {LEVEL 2}  [Model] Model_type

The Model_type subparameter is case sensitive.  The value must be one of the listed types from the IBIS specification.

Examples of the commonly used model types below illustrate the required IV tables.

MODEL_TYPE PULLUP  PULLDOWN  POWER_CLAMP  GND_CLAMP COMBINED   COMBINED

PULLUP+    PULLDOWN+

POWERCLAMP GND_CLAMP

I/O                             XXX          XXX      XXX        XXX

Output                          XXX          XXX      XXX        XXX

3-state                         XXX          XXX      XXX        XXX

I/O_Open_Drain                  XXX          XXX                 XXX

I/O_Open_Sink                   XXX          XXX                 XXX

I/O_Open_Source                 XXX          XXX      XXX

Open_Drain                      XXX          XXX                 XXX

Open_Sink                       XXX          XXX                 XXX

Open_Source                     XXX          XXX      XXX

ibischk v4.0.2 detects this:

ERROR (line  958) - Invalid Model_type ("Serius")

NOTE: Delete this section

    - {LEVEL 2}  [Model] C_comp is reasonable

The 4.1.3 check is superseded by 4.1.4, which requires correctness and gives guidance on when to be suspicious.

NOTE: Delete this section

    - {LEVEL 2}  [Model] C_comp is reasonable

When present in the model as an alternative to the overall C_comp value, C_comp_pullup, C_comp_pulldown, C_comp_power_clamp, and C_comp_ground_clamp, or any combination thereof chosen to represent the die capacitance of the buffer, must sum to the original C_comp value.  In other words, specifying the die capacitance in this more specific way should not change its overall value.

Note that a model may contain a combination of C_comp_* parameters that appear inconsistent with the buffer type. For instance, an open-drain model might include the C_comp_pullup parameter. This is because a pullup structure, for instance, may exist in the silicon and simply not be used for a particular I/O type. However, its parasitic capacitance will still be present at the node.

All general notes regarding choice of capacitance to report, given that die capacitance is frequency/voltage dependent, that apply to the C_comp parameter apply here as well.

As is the case with C_comp, the C_comp_* parameters must be positive. Also, cases in which the total of the C_comp_* values is greater than 20pF should be explained in the notes section, as is suggested for C_comp.

    - {LEVEL 3S} [Model] C_comp SPICE correlation

C_comp can be extracted through SPICE characterization of cell.  A number of papers describing techniques to extract C_comp, on the IBIS web site.

NOTE: Delete this section

    - {LEVEL 3M} [Model] C_comp laboratory correlation

Bench correlation of C_comp can be accomplished using techniques described in the Accuracy handbook, found on the IBIS web site.

NOTE: Delete this section

    - {LEVEL 2}  [Temperature Range] is reasonable

To pass this check the [Temperature Range] keyword must be present. The keyword needs some explanation because “minimum (min)” corresponds to a slow, weak driver and “maximum (max)” corresponds to a fast, strong driver. Slow and fast, in relation to temperature, depends on the process technology being described.

For CMOS versus Bipolar, switching speed and strength

Normally, CMOS has the relationship:

Temp(Min) > Temp(Max)

While Bipolar has:

Temp(Min) < Temp(Max)

The [Temperature Range] specified should normally match the temperatures at which the model was extracted. This is the chip die temperature, NOT the ambient temperature. The temperature being specified is usually higher than ambient temperature because the IC and parts around it in the measurement setup (or simulation) dissipate power. If the [Temperature Range] might not accurately represent chip die temperature, this should be documented in a comment.

The reasons for differences between [Temperature Range] keyword and any model extraction temperature ranges should be documented as an exception, or comment.

The [Temperature Range] should not exceed the safe operating temperature range as given on the data sheet.

    - {LEVEL 2}  [Voltage Range] or [* Reference] is reasonable

[Voltage Range] is the operating supply voltage for a [Model].  It is required unless ALL FOUR of the other voltage reference keywords are supplied. When [Voltage Range] is used alone, the other keywords default to:

- [Pullup Reference] = [Voltage Range] value

- [Pulldown Reference] = 0V

- [POWER Clamp Reference] = [Voltage Range] value

- [GND Clamp Reference] = 0V

Regardless of whether [Voltage Range] or [* Reference] is used, the values must be reasonable and must represent the actual conditions of IBIS model extraction. The typ, min and max values of the chosen voltages should normally follow the relationship:

min < typ < max

The min and max voltages must fall within the maximum operating conditions specified by the data sheet, but are not required to span the full range. Model users will be looking for min and max voltages that will reflect the range of supply voltages for their design, which will usually not stray more than 10% from the nominal voltage. If a buffer can be operated at more than one nominal voltage, a separate [Model] should be created for each nominal voltage, with reasonable min and max values for each typ voltage. In this case [Model Selector] would be used to allow the user to choose the nominal voltage used for the application.

There should be consistency among models of the same nominal voltage. For example, all 2.5V buffer models used for one [Component] would be expected to have the same min values and the same max values. Departures from this must be documented.

    - {LEVEL 3}  [Pullup Reference] is reasonable

The reference voltage for the [Pullup] table.  This is the offset from GND.  For CMOS, value > 0 is normal.

NOTE: Delete this section

    - {LEVEL 3}  [Pulldown Reference] is reasonable

The reference voltage for the [Pulldown] table.  This is the offset from GND.  For CMOS, value = 0 is normal.

NOTE: Delete this section

    - {LEVEL 3}  [POWER Clamp Reference] is reasonable

The reference voltage for the [POWER Clamp] table.  This is the offset from GND.  For CMOS, value > 0 is normal.  For dual-supply I/O, may be different from [Pullup Reference].

NOTE: Delete this section

    - {LEVEL 3}  [GND Clamp Reference] is reasonable

The reference voltage for the [GND Clamp] table.  This is the offset from GND.  For CMOS, value = 0 is normal.

NOTE: Delete this section

    - {LEVEL 3}  [Model] timing test load subparameters complete

IBISCHK should (but does not) issue a warning message when no timing test load is defined, since this is critical for system-level timing.

Valid combinations of Vref, Rref, Cref, and Vmeas should be as follows:

- Vmeas, Cref

- Vmeas, Vref, Rref

- Vmeas, Vref, Rref, Cref

All other 13 (2^4 - 3) combinations should generate warnings as follows:

- Missing Vmeas -> Warning

- Cref with no Vmeas -> Warning

- Rref with no Vref -> Warning

- Vref with no Rref -> Warning

Ibischk detects this

NOTE: Delete this section

  - Model Switching Behavior Requirements

A number of parameters in the [Model] and [Model Spec] sections specify the switching characteristics of input and I/O buffers, in response to waveforms.

    - {LEVEL 2}  [Model] Vinl and Vinh complete

All input and I/O buffers have Vinl and Vinh subparameters in the [Model] section.

ibischk v4.0.2 detects this:

WARNING - Model 'BCM1250_SSTL1_0': Model_type 'I/O_open_sink' must have Vinl set

NOTE: Delete this section

    - {LEVEL 3}  [Model] Vinl and Vinh reasonable

The Vinl and Vinh parameters of the [Model] keyword represent the range of input threshold voltages for a population of buffers. The input threshold voltage is that voltage at which a very slowly changing input signal is able to switch the sensed input logic level from low to high or high to low.  The Vinl and Vinh parameters would correspond to DC values for Vinl and Vinh in a data sheet. The [Model] Vinl and Vinh values must match corresponding values in [Model Spec], when [Model Spec] is present. The [Model] Vinl and Vinh values are normally worst case and may correspond to typ, min, or max values in the [Model Spec] keyword, as appropriate.

For I/O buffers, Vinl and Vinh values should be below and above, respectively, Vmeas. Exceptions to this should be explained in a comment. It's uncommon.

    - {LEVEL 3}  [Model] Vinl and Vinh enclose Vmeas

For I/O buffers Vinl and Vinh values should be below and above, respectively, Vmeas. Exceptions to this should be explained in a comment. It's uncommon.

NOTE: Delete this section

    - {LEVEL 3}  [Model] Vmeas matches data sheet

Vmeas in the [Model] section matches the value given in the data sheet for the part technology.

NOTE: Delete this section

    - {LEVEL 3}  [Model Spec] Vinl and Vinh reasonable

Because the input switching uncertainty region defined by the Vinl and Vinh sub-parameters of the [Model] section (See section 5.2.2.) can be affected by power supply fluctuation for many I/O standards, a range may be specified for these parameters in the [Model Spec] section. The “min” and “max” values given must be correct for the “min” and “max” values given for the supply voltage in the [Voltage Range] keyword. Vinl and Vinh are needed only for Input and I/O types of models.

    - {LEVEL 3}  [Model Spec] Vinl+/- and Vinh+/- complete and reasonable

For input buffers with different voltage thresholds for rising and falling edges, Vinh+, Vinh-, Vinl+, and Vinl- are given in the [Model Spec] section. This would be required for inputs that exhibit hysteresis, such as Schmitt trigger devices. For I/O buffers, Vinl+ and Vinh- values should be below and above, respectively, Vmeas. Exceptions to this should be explained in a comment.

    - {LEVEL 2}  [Model Spec] Vinl+/Vinh+ greater than Vinl-/Vinh-

Vinh+ is greater than Vinh-, and Vinl+ is greater than Vinl-.

NOTE: ibischk does compare Vinh+ to Vinh- and Vinl+ to Vinl-:

WARNING - Vinh+(Typ) is not greater than or equal to Vinh-(Typ) for   Model Spec defined on line 986

NOTE: Delete this section

    - {LEVEL 3}  [Model Spec] Vinl+/- and Vinh+/- enclose Vmeas

For I/O buffers, Vinl+ and Vinh- values should be below and above, respectively, Vmeas. Exceptions to this should be explained in a comment.

NOTE: Delete this section

    - {LEVEL 3}  [Model Spec] Pulse subparameters complete

If input voltage levels can rise above Vinh or fall below Vinl for short periods of time without being sensed as an input logic level change, Pulse_high, Pulse_low, Pulse_time are given in the [Model Spec] section.

    - {LEVEL 3}  [Model Spec] Pulse_high greater than Vinh

Pulse_high is greater than Vinh.

ibischk v4.0.2 detects this:

WARNING - Pulse_high(Typ) is not greater than Vinh-(Typ) for Model Spec defined on line 986

    - {LEVEL 3}  [Model Spec] Pulse_low less than Vinl

Pulse_low is less than Vinl.

ibischk v4.0.2 detects this:

WARNING - Pulse_low(Typ) is not less than Vinl+(Typ) for Model Spec defined on line 986

    - {LEVEL 3}  [Model Spec] Pulse_time reasonable

Pulse_time is less than the minimum rise time and fall time normally expected at the input.

    - {LEVEL 2}  [Model Spec] S_Overshoot subparameters complete

All input and I/O buffers have S_overshoot_high and S_overshoot_low in the [Model Spec] section.

    - {LEVEL 2}  [Model Spec] S_Overshoot subparameters match data sheet

S_overshoot_high and S_overshoot_low in the [Model Spec] section correspond to the maximum and minimum DC limits for the voltage at the buffer input, as specified in the data sheet for the device technology.

    - {LEVEL 2}  [Model Spec] S_Overshoot subparameters track typ/min/max

When overshoot voltage limits are different in min and max corners, S_overshoot_high and S_overshoot_low should track these differences. For example, S_overshoot_high may increase with the higher supply voltage assumed for max mode.

    - {LEVEL 2}  [Model Spec] D_Overshoot subparameters complete

If greater levels of overshoot can be tolerated for short periods of time, these must be given as D_overshoot_high, D_overshoot_low, and D_overshoot_time subparameters of the [Model Spec] keyword. For some technologies the data sheet may specify different parameters to address this concept. In this case the method by which D_overshoot_high, D_overshoot_low, and D_overshoot_have been determined.

    - {LEVEL 3}  [Model Spec] D_Overshoot subparams exceed S_Overshoot

D_overshoot_high must be greater than S_overshoot_high, and D_overshoot_low must be less than S_overshoot_low.

NOTE: Delete this section

    - {LEVEL 3}	[Receiver Thresholds] Vth present and matches data sheet, if needed

If [Receiver Thresholds] are needed to represent input behavior, the Vth subparameter must be present if the signal is single-ended. Vth is the nominal input threshold voltage at voltage temperature and process conditions that define 'typ'. Vth must match the input buffer timing measurement threshold in the data sheet.

An example of a technology where [Receiver Thresholds] can be used is the DDR Memory Interface. In DDR, the input threshold voltage is nominally 0.50*VDDQ. For DDR2, VDDQ is allowed to change from 1.7 to 1.9V, nominally 1.8V. In this case Vth would be specified as 0.9V. IBIS tools will adjust the threshold voltage actually used as some system voltage fluctuates.

    - {LEVEL 3} [Receiver Thresholds] Vth_min and Vth_max present and match data sheet, if needed

Vth_min is the lowest actual input threshold voltage at typical supply voltage, process, and temperature conditions. Likewise, Vth_max is the highest actual input threshold voltage at typical supply voltage, process, and temperature conditions. These are often specified as tolerance values in data sheets, representing an uncertainty as to where Vth actually lies. Threshold changes due to min and max power supply variation are in addition to the Vth_min and Vth_max values. Vth_min and Vth_max must be present if the data sheet specifies a tolerance for Vth under typical conditions.

For example, the input threshold voltage for DDR technology is allowed to range from 0.49*VDDQ to 0.51*VDDQ, nominally 0.50*VDDQ.  For DDR2, VDDQ is allowed to change from 1.7 to 1.9V, nominally 1.8V.  With the above definition of Vth, Vth_min and Vth_max, the values are calculated as follows: Vth=0.9V, Vth_min=0.49*1.8=0.882V and Vth_max=0.51*1.8=0.918V. As explained above, the variation in Vth only includes the effect of change of process and temperature at nominal voltage, which in this example is 1.8V.

    - {LEVEL 3} [Receiver Thresholds] Vinh_ac, Vinl_ac present and match data sheet, if needed

Vinh_ac, Vinl_ac are the voltages above/below which the input signal must cross before the receiver can be guaranteed to change state. Vinh_ac, Vinl_ac overrides the Vinh and Vinl defined earlier in the [Model] or [Model Spec] section.

The values given for Vinl_ac and Vinh_ac must match those in the data sheet. Note, however, that these parameters are required to be specified as offsets to Vth in the IBIS model, while they may be given as absolute voltages in the data sheet. Therefore, some conversion may be necessary. For instance, taking the SSTL18 standard as an example, the data sheet might call out 0.9V for Vth and 1.150V for Vinh_ac, which would require that Vinh_ac be given the value +250 mV in the IBIS file.

    - {OPTIONAL} [Receiver Thresholds] Vinh_ac, Vinl_ac match with datasheet

NOTE: Delete this section. Text merged into 5.2.20.

    - {LEVEL 3} [Receiver Thresholds] Vinh_dc, Vinl_dc present and match data sheet, if needed

Vinh_dc and Vinl_dc are DC input voltage thresholds which determine the boundary conditions under which the receiver will NOT change state.

The values given for Vinl_dc and Vinh_dc must match those in the data sheet. Note, however, that these parameters are required to be specified as offsets to Vth in the IBIS model, while they may be given as absolute voltages in the data sheet. Therefore, some conversion may be necessary. For instance, taking the SSTL18 standard as an example, the data sheet might call out 0.9V for Vth and 1.0V for Vinh_dc, which would require that Vinh_dc be given the value +100 mV in the IBIS file.

    - {OPTIONAL} [Receiver Thresholds] Vinh_dc, Vinl_dc match with datasheet

NOTE: Delete this section. Text merged into 5.2.22.

    - {LEVEL 3} [Receiver Thresholds] Tslew_ac/Tdiffslew_ac present and match data sheet, if needed

If the data sheet specifies a maximum time that an incoming signal may take to transition between Vinl_ac and Vinh_ac, then the [Receiver Thresholds] Tslew_ac parameter should be set to that value. For differential receivers the Tdiffslew_ac parameter should be set to the maximum allowable transition time between -Vdiff_ac and +Vdiff_ac.

    - {OPTIONAL} [Receiver Thresholds] Tslew_ac/Tdiffslew_ac match with datasheet

NOTE: Delete this section.

  - Model I-V Table Requirements

A summary of the quality of the IBIS model(s) I-V tables is contained in this section. This should be included in |IQ section as prescribed in the IBIS Quality Checklist. These comments can appear after the Notes section in the Header.

The term "table" as used in this document refers to rows and columns of numbers appearing in the text of the IBIS file. The term "curve" refers to the visual plotting of table data. Some checks are more easily performed visually.

[Pullup] and [Power Clamp] tables in IBIS files are Vcc-relative. This means that the voltage values in the first column are actually inverted offsets from Vcc. For example, the value at 0V in a [Pullup] table is actually measured at Vcc and the value at Vcc in the table is measured at 0V. Bear this in mind when checking Vcc-relative tables. The table, below, illustrates this with 3 examples.

Table 1: IBIS table vs. measured voltages

| IBIS (Vcc-relative) | Measured (Ground-relative) |
| --- | --- |
| -Vcc | 2*Vcc |
| 0 | Vcc |
| Vcc/2 | Vcc/2 |
| Vcc | 0 |

Curve viewing tools may offer the ability to translate Vcc-relative tables so that the curves viewed are ground relative.

    - {LEVEL 2}  I-V tables complete

The model verifier should check that all of the appropriate tables are present and correct, for each [Model]:

[Pullup], [Pulldown], [Gnd Clamp], [Power Clamp]

ibischk v4.0.2 detects this:

WARNING - [Model] BCM1250_LDT_OD has no description of the buffer's low

state DC drive characteristics (no [Pulldown] table).  This

warning can be silenced by changing the Model_type or by

adding a [Pulldown] table.

NOTE: Delete this section.

    - {LEVEL 3}  I-V tables have correct typ/min/max order

Inspect every I-V table.  Check for proper order of the I-V tables. Columns Values in table must be:

VOLTAGE, Current Typ, Current Min, Current Max

In most cases theMax current should be greater than the typical current, which should be greater than the Min current, in the active region (e.g. where device is not clamping). The most common exception is for compensate devices, where the typical, minimum, and maximum curves may nearly overlay each other. Also, [Power Clamp] curves may exhibit crossovers due to differences in the voltages at which clamping begins.

This check is easily accomplished by viewing the curves and checking visually. Checking combined curves is preferred, but checking of individual curves is acceptable.

    - {LEVEL 3}  I-V tables have reasonable numerical range

Check the numerical range of I-V values. Voltage Units are implied Volts. Current units are implied Amps.

Recommendation on how to handle excessive currents. Currents in excess of 1 amp should be truncated. At least one additional point should be included that prevents the slope from being 0.

NOTE: Delete this section

    - {LEVEL 3}  [Pullup] voltage sweep range is correct

The voltage column in the [Pullup] table should extend from -vcc to +2*vcc, For the purpose of this check, vcc is defined by the [Pullup Reference] keyword for this check, or [Voltage Range] if [Pullup Reference] is not present.

    - {LEVEL 3}  [Pulldown] voltage sweep range is correct

The voltage column in the [Pulldown] table should extend from -vcc to +2*vcc. For the purpose of this check, vcc is defined by the [Pullup Reference] keyword for this check, or [Voltage Range] if [Pullup Reference] is not present.

    - {LEVEL 3}  [POWER Clamp] voltage sweep range is correct

The voltage column in the [POWER Clamp] should extend at least from -vcc to 0 (it is permitted and recommended to extend from –vcc to +2*vcc, particularly where on-die termination is used). For the purpose of this check, vcc is defined by the [POWER Clamp Reference] keyword for this check, or [Voltage Range] if [POWER Clamp Reference] is not present.

    - {LEVEL 3}  [GND Clamp] voltage sweep range is correct

The voltage column in the [GND Clamp] should extend at least from -vcc to +vcc (it is permitted and recommended to extend from –vcc to +2*vcc, particularly where on-die termination is used). For the purpose of this check, vcc is defined by the [POWER Clamp Reference] keyword for this check, or [Voltage Range] if [POWER Clamp Reference] is not present.

    - {LEVEL 3}  I-V tables do not exhibit stair-stepping

There should be no Stair Stepping of any I-V tables. This is caused by insufficient significant digits in the table current columns. It is also a common problem from the NCSU s2ibis conversion process.

This check is easily accomplished by viewing the curves and checking visually.

    - {LEVEL 3}  Combined I-V tables are monotonic

Check that the combined tables are monotonically increasing, ie. There are no slope reversals in the current values.

This check is easily accomplished by viewing the curves and checking visually.  This must be checked by visual inspection until a Parser Bug is fixed in IBISCHK 4.x.  This is important if there are non monotonic warnings because the Golden IBIS parser does NOT check combined tables.

The Golden IBIS parser also stops reporting errors on a table after the first non-monotonic points are found.

ibischk v4.0.2 detects this, but [IBIS ver] must be 4:

TBD: insert message here

    - {LEVEL 3}  [Pulldown] I-V tables pass through zero/zero

Typ, Min, and Max Pulldown table currents should all pass through zero at zero volts from ground (cmos). These three pull down tables should pass through zero/zero except in special cases (i.e. Differential, PECL, LVDS, or SERDES driver).

    - {LEVEL 3}  [Pullup] I-V tables pass through zero/zero

For Un-terminated Models (i.e. CMOS push-pull), Typ, Min, and Max Pullup table currents should pass through zero current at zero volts for Vcc relative tables.

If these curves are viewed graphically, ground relative, they should pass through their respective supply rail (eg. Typ should pass through 3.3 volts, Max should pass through 3.6 volts, min should pass through 3.0).

Note that an IBIS I-V table is offset such that supply voltage on the Typ table falls at 0 volts in the table. For example, 3.3 volts usually falls at 0.0 in the table.

Terminated drivers and some technologies may be exceptions

i.e. Bipolar, ECL, and CMOS LVDS.

    - {LEVEL 3}  No leakage current in clamp I-V tables

Review each clamp table.  The expected currents should be less than 1 uA in the normal operating ranges (typically from 0 to 2 Vcc range in the table).  If a table is truncated, use the extrapolated values based on the last two points prior to extrapolation.  Or use a viewer which can combine the two clamp tables into one.  Exceptions can exist for older TTL technologies where several milliamps may be observed and for some ECL and other technologies with which can have internal termination networks.  Exceptions should be understood and documented.

    - {LEVEL 3}  Clamp I-V behavior not double-counted

Verify that if [GND Clamp] exists there is no clamp current behavior in the negative portion of the [Pulldown] table.  Verify that if [Power Clamp] exists there is no clamp current behavior in the negative portion of the [Pullup] table.

    - {LEVEL 3}  On-die termination modeling documented

Any IBIS models with on-die termination should be labeled as such using comment lines.  On die termination should be modeled in [Power Clamp] and/or [GND Clamp] tables.  Document the method used to embed the termination into the clamps.

NOTE: NCSU s2ibis2 does not correctly model on-die termination. It places the full termination characteristic in both [Power Clamp] and [GND Clamp] tables, effectively double-counting the termination.

    - {LEVEL 3}  ECL models I-V tables swept from -Vdd  to +2 Vdd.

I-V tables in ECL models should be swept from -Vdd  to +2 Vdd, even though the operating range is narrower.

    - {LEVEL 3} Point distributions in IV curves should be sufficient

We recommend a minimum of 10 data points at points of inflection to prevent interpolation issues in simulations.

    - {LEVEL 3} Correlate IV curves to combined curves.

I-V sweep in Simulator (level 3S)

I-V sweep on Bench with table tracer (level 3M)

  - Model V-T Table Requirements

    - {LEVEL 2}  V-T table endpoints consistent with I-V tables

The voltage associated with the intersection of the V_fixture, R_fixture, and R_dut (if present) load line with the respective combined high/low DC I-V characteristic should be within 2% of the V-T table DC endpoints based on the V-T table DC range.

The combined High State DC I-V characteristic is defined as the sum of the [Power Clamp], [GND Clamp], and [Pullup] I-V tables (ground relative). Similarly, the combined Low State DC I-V characteristic is defined as the sum of the [Power Clamp], [GND clamp], and [Pulldown] I-V tables (ground relative).

ibischk v4.0.2 detects this, but [IBIS ver] must be 4:

TBD: insert message here

    - {LEVEL 3}  V-T tables look reasonable

V-T tables should be well behaved, with continuous second derivative. V-T point density should be greater in areas with non-zero second derivative. Excess V-T points in beginning and end of V-T tables should be removed.

Relative time position between all tables should be maintained. Final DC value must be achieved, ie. the ending slope should be very small.

This check is easily accomplished by viewing the curves and checking visually.

    - {LEVEL 3}  Model simulation successful

Standard simulations run on models and documented.  Documentation should include test circuit and simulator run on.

    - {LEVEL 2}  Document known model limitations

Document known model limitations as comments. Examples include PVT, unknown/placeholder threshold levels, LVDS restrictions, etc.

    - {LEVEL 3}  Output and IO buffers have should have 2 sets of V-T tables

Outputs and IO buffers should have four V-T tables:

50 Ohm Pullup (Voltage = [Pullup Reference])

- Rising

- Falling

50 Ohm Pulldown (Voltage = [Pulldown Reference])

- Rising

- Falling

LVDS and other terminated technologies may be modeled using 2 sets of V-T tables. by including Common mode voltage close to the region of operation. for example:

50 Ohm to Vcm (Common Mode Voltage)

- Rising

- Falling

  - Model Ramp Data Requirements

The [Ramp] section is required even if [Rising Waveform] and [Falling Waveform] are present in a [Model]. [Ramp] information is used in some tools for non-simulation purposes, for example quickly finding the fastest pin on a net.

    - {LEVEL 2}  Output and IO buffers have a [Ramp] section

All buffers capable of output have a [Ramp] section.  Input-only buffers, terminators, Series and Series_switch models do not have a [Ramp] section.

ibischk v4.0.2 detects this:

ERROR - Model 'BCM1250_SSTL2_7': Ramp Not Defined

    - {LEVEL 3}  [Ramp] R_load present if value other than 50 ohms

If the [Ramp] data was measured using a load resistor other than 50 ohms, the [Ramp] section has an R_load subparameter giving the load resistor value used.

    - {LEVEL 3}  [Ramp] test fixture has no reactives

The fixture used for generating [Ramp] measurement waveforms has only the buffer, a load resistor, and a terminating voltage. No capacitors or inductors, for example.

    - {LEVEL 3}  [Ramp] typ/min/max order is correct

The typ, min, and max [Ramp] values are taken from typ, min, and max buffer measurements. They are not necessarily sorted by dV, dt, or dV/dt. Although the progression from min to max usually has dV increasing  and dt decreasing, the correct order is actually determined by the  test conditions used, the same conditions used to derive typ/min/max I-V tables.

    - {LEVEL 3}  [Ramp] data dv and dt values positive

The dV and dt values are greater than zero. Note that s2ibis2 will incorrectly generate negative [Ramp] values in the case where the [Ramp] simulation duration is much longer than the [Ramp] dt, leaving no waveform points that fall between the 20% and 80% voltage levels.

The default test fixture for determining [Ramp] data has a 50 ohm resistor, tied to power for falling data and tied to ground for rising data. The initial and final values of the waveforms are used to define 20% and 80% voltage points, where [Ramp] measurements are made. These test fixtures may be similar to test fixtures used to produce [Rising Waveform] and [Falling Waveform] data, in which case the data can be checked for consistency. For example, a Rising Waveform with R_fixture = 50 and V_fixture = 0.0 should have a 20% to 80% dt that is similar to the [Ramp] dt_r value.

ibischk v4.0.2 detects this:

ERROR (line 1243) - dV must be greater than 0 in a Ramp specification

    - {LEVEL 3}  [Ramp] dV consistent with supply voltages

The [Ramp] dV values must not exceed 60% (80% - 20%) of:

[Pullup Reference] - [Pulldown Reference]

or

[Voltage Range]

    - {LEVEL 3}  [Ramp] dV consistent with V-T table endpoints

Each [Ramp] dV value must match within a factor of two 60% (80% - 20%) of the difference between the initial and final voltages of the corresponding V-T table with test fixture matching the Ramp test fixture, if matching V-T tables are present.  The [Ramp] dV values must not be less than half, nor greater than double, the calculated 60% of beginning-to-end voltage differences in the corresponding V-T tables.

For Output, I/O, or 3-state models the V-T table that must be consistent with the dV values of [Ramp] dV/dt_r will be a [Rising Waveform] with R_fixture equal to the [Ramp] R_load subparameter (or 50 ohms if R_load is absent) and V_fixture equal to [Pulldown Reference] or 0V.  The V-T table that must be consistent with the dV values of [Ramp] dV/dt_f will be a [Falling Waveform] with the same R_fixture, and V_fixture equal to [Pullup Reference] or [Voltage Range].

For Output_ECL, I/O_ECL, 3-state_ECL models the V-T table that must be consistent with the dV values of [Ramp] dV/dt_r will be a [Rising Waveform] with R_fixture equal to the [Ramp] R_load subparameter (or 50 ohms if R_load is absent) and V_fixture equal to ([Pullup Reference] or [Voltage Range]) minus 2V.  The V-T table that must be consistent with the dV values of [Ramp] dV/dt_f will be a [Falling Waveform] with the same R_fixture and V_fixture.

For Open_sink and Open_drain models the V-T table that must be consistent with the dV values of [Ramp] dV/dt_r will be a [Rising Waveform] with R_fixture equal to the [Ramp] R_load subparameter (or 50 ohms if R_load is absent) and V_fixture equal to [Pullup Reference] or [Voltage Range]. The V-T table that must be consistent with the dV values of [Ramp] dV/dt_f will be a [Falling Waveform] with the same R_fixture and V_fixture.

For Open_source models the V-T table that must be consistent with the dV values of [Ramp] dV/dt_r will be a [Rising Waveform] with R_fixture equal to the [Ramp] R_load subparameter (or 50 ohms if R_load is absent) and V_fixture equal to [Pulldown Reference] or 0V. The V-T table that must be consistent with the dV values of [Ramp] dV/dt_f will be a [Falling Waveform] with the same R_fixture and V_fixture.

When comparing V_Fixture against [Pullup Reference] or [Voltage Range] it is expected that the typ, min, and max values of [Voltage Range] will correspond to the V_fixture, V_fixture_min, and V_fixture_max values, respectively.

This check is not to be performed using V-T tables with V_fixture or R_fixture not matching [Ramp] fixture values. Reasonably small values of C_fixture, L_fixture, R_dut, L_dut, and C_dut parameters in [Rising Waveform] and [Falling Wavform] can be overlooked in the V-T table selection process, although these may degrade the correlation of [Ramp] to V-T table endpoints.

    - {LEVEL 3}  [Ramp] dt is consistent with 20%-80% crossing time

Each dt value matches within a factor of two the difference between the times of crossing the 20% voltage point and the 80% voltage point on the corresponding [Rising Waveform] or [Falling Waveform] with test fixture most similar to the Ramp test fixture.

    - {LEVEL 3}  [Ramp] dt is consistent with data sheet

Each dt value matches within a factor of two the corresponding rise or fall time specified in the data sheet for a part pin using that buffer model.

- Possible Errors

Here is a list of possible problems/errors that may be common. This list may be helpful in detecting correctness and completeness problems covered by the requirements of the previous sections.

  - {LEVEL 2} Typ/min/max order of parameters correct

The parameters of different keywords for min typ max must be in the correct numerical order e.g. rpkg,cpkg,lpkg,ccomp, ( exception temp, which states only the conditions ) Ibis checks for numerical order, ( smallest value is in min column ) if the parameters are not in numerical order, please document why (doesn't apply to vI-Vt-tables )

  - {LEVEL 3} C_comp checked in both input and output mode

The values for c-comp must be checked for plausibility, usually each input/output/IO has different values. Sometime a compromise must be reached because c_comp input and C_comp output for optimal correlation.

Model maker is encouraged to document in comments the basis for compromise i.e  C_comp = |C_comp_off + |C_comp_on /2

|C_comp_off 4.0p 3.9p  4.1p

|C_comp_on  1.0p 900.f 1.1p

|

C_comp 2.5p 2.4p 2.6p

  - {LEVEL 2} First/last point of waveforms equal to V_fixture values

If the V_fixture values equal the power supply reference voltages for the [Pullup] or [Pulldown] tables, then the starting or ending points of the V-T tables are expected to equal these V_fixture values.

For example for a 3.3 V device with the [Voltage Range] set to 3.3 V, V_fixture = 3.3 V, and R_fixture = 50 ohms, the [Rising Waveform] table should end at 3.3 V, and the [Falling Waveform] table should begin at 3.3 V.

Exceptions to this check occur in cases where internal terminators or bias conditions exist such that the combined V-I tables have current flows at the V_fixture voltages.

  - {LEVEL 3} Sufficient points in waveform table

The timestep in  v-t-table should be about 1/10 of ramp-time ( the smaller of both rising and falling ; timestep: dv / 10 ), there should be at least 10 points in the transition regions ( start of transition and end of transition ) and the points don't have to be evenly spaced. ( many points in the transition region and fewer elsewhere )

This doesn't apply to waveforms from measurement.

  - {LEVEL 3} Minimize waveform lead-in time

The v-t-tables must be referenced to a common input-stimulus-time See ibis 4 specification at the keywords [rising falling waveform].

Be aware that lead-in-time is not the buffer delay.  Excessive lead-in-time must be removed, but it must be the same amount of time for all waveforms Hint: The sum of risetime and falltime should be less than the expected minimum cycle time ( maximum cycle period ).

If there are big lead-in-times, please explain why.

  - {LEVEL 3} Open_sink/Open_source model with correct Vref, Cref, Rref, Vmeas

The appropriate values of these 4 parameters are added referring to the min typ max conditions ( now possible in ibis4 ) The parameters for Vref,Rref are a further indication, that the model is an open-sink ( open-drain ) Be sure that Rref,Vref is appropriate to the model-type.

e.g. open-sink:  Vref 3.3V ; Rref 500Ohm

  - {LEVEL 3} Differential models contain appropriate waveform tables

Differential models like lvds, pecl, pcml must contain at least one rising and one falling v-t-table, and both the same termination voltage (same v-fixture ) There is a discussion, whether a waveform is always needed, but if there is a waveform supplied, the designer can decide, whether to use it or not. e.g. for LVDS ramp are outside of operating range.

ECL is an exception if the ramp is defined under appropriate conditions

  - {LEVEL 3} Models correspond to data sheet

In the ibis-model file there are some models missing, which should be there corresponding to the data sheet.

e.g. different driver strength, inputs with internal pullup either put them in a model selector and if there doesn't exist all models, please note in ibisfile with  NC | No-model.

  - {LEVEL 3} Model_type correct for model data

Does the used model type belong to the parameters" e.g. Output with threshold parameters => suspected IO IO without threshold parameters => suspected output.

  - {LEVEL 3} Open_sink/Open_source model not push-pull

An open sink model must be modeled with a resistive load to vcc/vtt for both rising and falling. It is assumed for ramp that the 50 ohms is present and for both edges it goes to VCC. Be sure fixture voltage goes to reasonable voltage.

Failure to do so will result in no rising wavforms, and unswitching waveforms.

  - {LEVEL 3} All pins consistent with data sheet

All pins must be listed. Pin names must match data sheet and/or appropriate design files. (Be careful of consistency of pins names  A01 vs A1 and how tools will interpret this data).

- Correlation {level 2}

Please refer to "I/O Buffer Accuracy Handbook":

http://www.eda.org/pub/ibis/accuracy/

The I/O Buffer Accuracy Handbook defines a quantitative method for correlating test hardware with simulation predictions and documenting the results of the correlation.  The method is general and applicable to the two most common formats for I/O buffer model data: SPICE and IBIS.

Its foundation is a set of "golden waveforms" derived from SPICE simulations of the I/O buffer under various test conditions defined by the I/O Buffer Accuracy handbook.  If  the simulations reflect test conditions and the modeling engineer has some knowledge of semiconductor processing conditions, it is possible to correlate the golden waveforms with lab data, both graphically and quantitatively.

  - SPICE Correlation

    - Purpose of SPICE Correlation

IBIS quality correlation level "S" specifies that the model developer must simulate identical test loads in SPICE and in a IBIS . The intention of SPICE correlation is to assess the degree to which the IBIS model data and the behavioral model internal to the behavioral simulator agree with the I/O circuit model extracted from a schematic or physical layout and simulated in SPICE. By careful attention to detail and understanding the behavior of the I/O circuit, it is possible to achieve extremely close correlation between SPICE and IBIS simulation results. Be aware that not all behavioral simulators are created equal; discrepancies may be an artifact of the behavioral simulator rather than the IBIS model extraction process.

    - SPICE Correlation overview

The best recipe for success in SPICE correlation is a healthy sense of curiosity.  How does the circuit work"  What circuit elements are necessary and sufficient to model the behavior of the circuit"  What test loads might stress the accuracy of the behavioral model" How will the circuit be used"  Take the time to answer these questions before beginning the exercise.

Neither the SPICE nor the IBIS simulations should include the package model at least in the first attempt at correlation.

This simplifies the correlation process and makes it easier to identify discrepancies.  IBIS deals primarily with I/O circuit behavior; its package model features are relatively simple and should produce the same effects in behavioral simulation as in SPICE.

    - Correlation process

Correlation process is to make a quantitative comparison between behavioral simulation results and lab data.  Section X.X.X  defines the correlation process for IV curves and transient waveforms. It also describes how to compare data from capacitance and edge rate measurements. There are in general two different kinds of correlation:

lab measurement vs. SPICE simulation and IBIS simulation vs. SPICE simulation. One of the three correlation levels defined in this section applies only to lab measurement vs. SPICE simulation correlation. The other two correlation levels apply to both types of correlation.

In the case of the IBIS model, correlation is a two-step procedure. In the first step, the semiconductor vendor correlates lab data against SPICE-based golden waveforms that are embedded in the IBIS data sheet in the form of voltage-time tables.  In the second step, the user correlates behavioral simulation results against the same golden waveforms using his or her simulator of choice.  This approach effectively decouples the behavioral simulator from the hardware and splits the correlation problem into independent components. However, we still recommend that the semiconductor vendor run behavioral simulations using the IBIS data sheet for one behavioral simulator.

This will enable the modeling engineer to fine-tune the model data and ferret out possible discrepancies that would go otherwise unnoticed.

    - Correlation Levels

Model users have accuracy needs that vary with the demands imposed by their designs.  For this reason, the I/O Buffer Accuracy Handbook defines several "correlation levels."  A correlation level is a means for categorizing model data by the amount of effort the modeling engineer invests in verifying their accuracy.  Each individual correlation level is defined on the basis of how much the modeling engineer knows about the semiconductor processing conditions of the sample component(s).

For the purposes of the handbook, a metric is simply a numerical method for quantifying how well two sets of data points agree with each other.

Level Component Sample            Envelope Metric         Overlay Metric

1     Random                      YES                     NO

2     Known typical               YES                     YES

3     Known typical fast, slow    YES                     YES

Correlation Level 1 applies in the case that the modeling engineer knows nothing about the processing conditions of the DUT.  In other words, the DUT is a random sample.  The Curve Overlay Metric only applies in cases where the two curves should theoretically lie on top of one another.

Therefore, the Curve Envelope Metric is the only valid metric in this case.  The Envelope Metric is not useful in all waveforms or IV curves.

Correlation Level 1 provides the least accuracy information. It is relevant to correlation of golden waveforms to lab measurements only.

Correlation Level 2 applies in the case that the modeling engineer has a sample component that is known to come from a lot with typical semiconductor device parameters.  The Envelope Metric applies in all Correlation Levels, but the Overlay Metric provides more accuracy information. Correlation Level 2 is relevant to correlation of golden waveforms to lab measurements and behavioral simulations.

Correlation Level 3 applies in the case that the modeling engineer has three sample components: one from a lot with known typical semiconductor device parameters, one from a fast lot, and one from a slow lot.  As in the previous two Correlation Levels, the Envelope Metric applies. Correlation Level 3 insures the highest degree of accuracy as well as confidence that the semiconductor vendor can indeed control the process in a manner consistent with the model data.

This allows the model user to have confidence in the timing and noise margins of the system he or she is designing. Correlation Level C is relevant to correlation of golden waveforms to lab measurements and behavioral simulations.

    - Figures of Merit (FOM)

      - The Curve Overlay Metric (FOM1)

The Curve Overlay Metric applies to cases in which the measured and simulated data should theoretically lie directly on top of each other.

For example, a SPICE simulation of a 50 Ohm load and a IBIS simulation of the same load should theoretically yield identical results.

Another example is the measurement of a known-typical sample component and a structural simulation of the same network under identical process-voltage-temperature conditions.

The Curve Overlay Metric  measures how well the two curves or waveforms match each other by summing the absolute value of the x-axis (or y-axis) differences between the two data points, weighing the sum against the range of data points along that axis, and dividing by the number of data points.

FOM1 = 100 * [1- (SUM |X(golden) - X(dut)| / X*N)]

SUM from i = 1->N

      - The Curve Envelope Metric (FOM2)

The Curve Envelope Metric applies to cases in which the measured data are, in theory, bounded by two curves (or waveforms) that represent process-voltage-temperature extremes.  In general, this metric is useful when the processing conditions of the sample component are unknown.  The Curve Overlay Metric returns a yes/no value depending on whether or not every one of the data points falls within the envelope boundaries defined by the min and max curves. The plot below demonstrates a lab pull-down curve (solid line) that is slightly stronger than the typical curve (middle dashed line) and lies well within the (outer dashed lines).

The Curve Envelope Metric presents a difficulty in the case of unterminated transmission line loads.  Because these waveforms overshoot normal logic levels and ring back, the min and max waveforms intersect each other and do not define an envelope.  Therefore, the Curve Envelope Metric may not be applied to the Open Transmission Line load or the Transmission Line and Receiver load.

      - Figure of Merit Determination

The user must determine what values for the three figures of merit are acceptable for a given application, but the model developer should have some general goals for these values because he or she cannot know the application before hand.

- Shoot for FOM1 (Curve Overlay) greater than or equal to 98%.

- Shoot for FOM2 (Curve Envelope) less than or equal to 10%.

- Include plot of overlay metric.

- Include plot of envelope metric.

- Include reference to ack (tools) .

- Include reference to trailer (example).

- Model Limitations and Model Maker Notes:

  - Document Known Limitations

There are certain additional pieces of information that model makers should include as comments. This includes any known limitations the model For example, this may be a specific version of spice that supports an advanced feature utilized in the IBIS model (i.e. On Die Termination).

  - Document Model Maker Notes

There are certain additional pieces of information that model makers may include as comments for the rising and falling waveforms. These are not essential (and not always available), but are nice to have. These items can be helpful to a user in assessing how the rise and fall waveforms were created. These apply only if the waveforms were generated by simulation.

    - Number of Internal Stages

The number of internal pre-stages used in the simulation for rising and falling waveforms may be provided in a comment, if available. This information provides a clue as to the shape of the original simulated waveforms. Ordinarily, at least 3 stages are expected.

    - Stimulus Used

The rise and fall times and the period of the input waveform are also helpful in interpreting the output rise and fall waveforms.