Targets for Analog Simulation of Digital Circuitries

w		Correct logical function under normal operating conditions

- Signal integrity

- Crosstalk

- Propagation delay

w	Avoidance of incorrect function and component destruction under ‘non normal’

operating conditions

- Switching and On/Off conditions of multiple power supplies

- Static and dynamic current / voltage over-/undershoots at inputs and outputs

- Maximum operating frequencies under load conditions

w	Increase of long time reliability

- Junction Temperature

- Break Down Voltage

- Max. Currents

Practical experience: important parameters

w	Monotony of rising and falling edges at inputs			1)

w	Min./max. rise and fall times at inputs				1)

w	Ground Bounce effects						1)

w	Bus Hold circuitry at inputs						1)

w	Switching and On/Offf conditions of multiple power supplies	1)

w	Input capacitance							2)

w	Clamping diodes							2)

w	Load normalization of Data Sheet delay times			3)

1)	Not covered by IBIS

2)	Covered by IBIS

3) Interface problem between different libraries

- Analog Simulation of digital circuitries is becoming more and more important.

- Many of the model parameters are not part of Data Sheet.

REQUIRED IBIS ENHANCEMENTS

Gerald Bannert, Siemens

Presentation Contents

w	Targets for Analog Simulation of Digital

Circuitries

w	Practical Experience: Important Parameters

w	Examples:

-	Pulse Immunity and Monotony of Rising

and Falling Edges

-	Switching of Power Supplies / Hot Insertion

-	Ground- and VCC-Bounce affect Propa-

gation Delay as well as Rise / Fall Times

-	Summary

Summary

w	Most critical parameters are difficult to be defined,

measured and modelled

w	Modelling parameters should take into account

future practical needs

w	Vendor interface is very important:

- Valid IBIS revisions

- Optional parameters

- Interpretation of IBIS parameters

- Feed Back of simulation results to I/O-(re)designs

- Model and / or  silicon changes

- Model validity and guarantee

Silicon and models must go together