# IBIS CONNECTOR SPEC: RELIMINARY PROPOSAL FOR A

# PIN NAMING LANGUAGE

|  The Auto_Map section contains a simple program function that provides the algorithm for

| generating the signal and pin names. The function must be written in a simplified form of the

| ‘C’ programming language.

|

| The function may include:

|	the built in variables ROW and COLUMN

|              the result variables SIGNAL_NAME and PIN_NAME

| 	Integer variables, simply created on usage.

| 	Integer constants e.g 55

|	 String constants enclosed by double quotes e.g. “example”

| 	Character constants enclosed in forward single quotes e.g. ‘a’

|	The mathematical operators + - * / %

| 	Boolean operators > < >= <= == AND OR

|	Statements terminated by a semicolon

|	The formatting function stringf(<template string>, <list of variables>)

| 	the conditional statements if (<expression>) { <statement> } else { <statement>}

|

|	The formating template string may include the following substitution sequences:

|	%d	decimal number

| 	%c	character

|	%s	string

|	%%        a percent character

|

| Example one:

|

row_letter = 'a' + ROW - 1;

SIGNAL_NAME = stringf("Signal_%c%d", row_letter, COLUMN);

PIN_NAME = stringf (“%c%d”, row_letter, COLUMN);

|

| This function results in pin and signal names as follows:

|

| Pin Name    Signal Name

|

|         a1          Signal_a1

|         a2          Signal_a2

|         ....

|         b1              Signal_b1

|         ....

|

| Example one can be condensed to:

|

SIGNAL_NAME = stringf("Signal_%c%d”, ROW + 'a' - 1, COLUMN);

PIN_NAME = stringf (“%c%d”, ROW + 'a' - 1, COLUMN);

|

|

| Example Two (with conditional)

|

if ((ROW < 5) OR (ROW > 7))

{

SIGNAL_NAME = stringf("Signal_%c%d", ROW + 'a' - 1, COLUMN);

PIN_NAME = stringf (“%c%d”, ROW + 'a' - 1, COLUMN);

}

else

{

- SIGNAL_NAME = "NA";

- PIN_NAME = "NA";

}

|