SERIES SWITCH MODELING TESTS

Bob Ross, June 12, 1997

Interconnectix

- SERIES CROSSBAR & MOSFET SIMPLIFIED MODEL

- CURRENT Vs SOURCE VOLTAGE FOR TWO Vds VALUES

(Level 3 MOSFET Model)

(Instead of Resistance vs Voltage)

Linear Equation:   ids = Itable vds/Vds

Did not have good success with two tables and quadratic fit

- GOOD CORRELATION WITH 8 pF LOAD

- REFLECTED WAVE SAMPLES (5 V and 3.3V)

- 4.3 V Vcc (Used for 5 V and 3.3 V Interfaces)

Table Relative to Power Clamp Ref.

A 4.3 V Model and [Model Selector] should yield better results

- ACTUAL SPICE MODEL OF 74CBT3383 (Level 13)

With Ground Clamps at BOTH terminals

With 4 pF Caps for Input Capacitance at BOTH terminals

3.3 V & 5 V Inputs - Spice Model has Glitch.

(A Non Linear Cap. ??)

A Power Clamp Effect, but no Power Clamp Exists

Still a reasonable APPROXIMATION